3GPP TSG RAN WG4 Meeting #98-bis-e	R4-2106637

Electronic Meeting, April 12 – April 20, 2021

Agenda item:			5.5.2.2.2.3

Source:	vivo

Title:	Link level simulation results for UE RX-TX time difference

Document for:	Information

- Introduction

In the RAN4#98-e meeting, there were agreements to provide simulation results based on parameter combinations in [2]. The agreements were captured in the WF [1].

- Companies are encouraged to bring PRS accuracy simulation results in RAN4 #99e for the identified set of parameters - PRS BW defined in number of PRBs - PRS SCS - dl-PRS-ResourceRepetitionFactor - dl-PRS-NumSymbols - dl-PRS-CombSizeN - Other parameters are FFS - Note: the full set of parameters and values were included in R4-2104092, which will be also used as the template used to collect the simulation results also.

In this contribution, we provide simulation results for UE Rx-Tx time difference measurement accuracy based on simulation assumptions in [2] and [3].

# Discussion

Simulation results for UE Rx-Tx time difference measurement accuracy are provided in Tables 1, 2, 3 for 15kHz SCS, 30kHz SCS and 120kHz SCS respectively. Detailed simulation assumptions are provided [3]. The accuracy results are based on measurements of 4 samples. Es/Iot for the 3 cells (cell 1, cell 2, cell 3) is (-3dB, -13dB, -13dB).

Table 1. simulation results of UE Rx-Tx time difference accuracy (Tc) for 15KHz SCS

| BW (RB) | Comb | Syms | AWGN |  |  | TDL-A |  |  | TDL-B |  |  | TDL-C |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | cell1 | cell2 | cell3 | cell1 | cell2 | cell3 | cell1 | cell2 | cell3 | cell1 | cell2 | cell3 |
| 52 | 2 | 2 | 62.37 | 73.91 | 74.02 | 63.41 | 79.14 | 77.01 | 66.09 | 102.09 | 103.40 | 73.78 | 198.18 | 212.49 |
|  | 2 | 4 | 60.97 | 67.46 | 69.64 | 61.81 | 69.38 | 71.42 | 62.77 | 85.81 | 85.02 | 65.92 | 173.22 | 169.99 |
|  | 2 | 6 | 60.32 | 65.96 | 69.16 | 61.06 | 68.81 | 69.89 | 61.65 | 74.05 | 80.82 | 64.44 | 127.09 | 113.30 |
|  | 2 | 8 | 60.23 | 65.01 | 65.41 | 60.51 | 65.52 | 67.41 | 61.32 | 69.92 | 75.19 | 61.90 | 82.04 | 104.81 |
|  | 2 | 12 | 60.02 | 67.46 | 61.89 | 60.04 | 67.71 | 64.56 | 60.64 | 69.87 | 65.79 | 61.13 | 89.76 | 83.98 |
|  | 4 | 4 | 62.27 | 73.53 | 74.52 | 63.97 | 80.27 | 78.82 | 65.40 | 99.65 | 116.59 | 74.03 | 203.11 | 198.14 |
|  | 4 | 12 | 60.11 | 65.17 | 64.92 | 60.86 | 67.81 | 66.86 | 61.42 | 69.32 | 71.80 | 62.21 | 99.63 | 100.14 |
|  | 6 | 6 | 62.60 | 70.49 | 74.91 | 63.81 | 79.88 | 81.86 | 66.61 | 97.70 | 98.99 | 83.84 | 199.60 | 196.47 |
|  | 6 | 12 | 60.73 | 64.70 | 64.96 | 61.62 | 69.74 | 68.60 | 62.56 | 75.51 | 79.44 | 64.63 | 136.83 | 117.73 |
|  | 12 | 12 | 61.60 | 67.04 | 68.22 | 63.39 | 75.28 | 75.99 | 64.67 | 98.45 | 90.97 | 71.59 | 182.44 | 163.85 |
| 104 | 2 | 2 | 30.51 | 34.27 | 34.23 | 31.26 | 38.45 | 38.74 | 31.81 | 47.35 | 50.12 | 32.59 | 145.48 | 150.48 |
|  | 2 | 4 | 30.04 | 32.62 | 32.94 | 30.49 | 34.94 | 34.83 | 31.42 | 41.64 | 43.66 | 31.16 | 45.98 | 153.37 |
|  | 2 | 6 | 29.79 | 31.60 | 32.59 | 30.16 | 32.97 | 33.20 | 30.41 | 38.11 | 39.01 | 30.53 | 35.56 | 119.39 |
|  | 2 | 8 | 29.78 | 31.51 | 31.56 | 29.94 | 32.21 | 32.82 | 30.06 | 34.16 | 35.54 | 30.28 | 34.11 | 39.53 |
|  | 2 | 12 | 29.42 | 30.55 | 30.46 | 29.72 | 32.32 | 31.38 | 29.91 | 34.28 | 33.35 | 29.86 | 33.09 | 32.05 |
|  | 4 | 4 | 30.48 | 35.56 | 33.87 | 31.20 | 39.36 | 37.65 | 33.38 | 51.78 | 47.04 | 32.35 | 117.07 | 154.27 |
|  | 4 | 12 | 29.69 | 31.17 | 31.44 | 30.12 | 32.25 | 32.33 | 30.41 | 35.54 | 35.57 | 30.31 | 33.91 | 34.95 |
|  | 6 | 6 | 30.38 | 34.84 | 34.44 | 31.31 | 38.16 | 38.55 | 32.12 | 46.88 | 47.57 | 32.88 | 149.63 | 154.74 |
|  | 6 | 12 | 29.80 | 31.37 | 31.43 | 30.33 | 33.26 | 33.46 | 30.78 | 36.32 | 38.74 | 30.82 | 38.81 | 37.25 |
|  | 12 | 12 | 30.26 | 32.38 | 32.40 | 31.01 | 36.67 | 35.78 | 31.58 | 41.66 | 41.69 | 31.89 | 117.63 | 152.11 |
| 268 | 2 | 2 | 14.77 | 15.56 | 15.60 | 14.89 | 15.92 | 15.93 | 15.32 | 24.80 | 25.60 | 15.06 | 16.95 | 78.79 |
|  | 2 | 4 | 14.66 | 15.25 | 15.76 | 14.77 | 15.39 | 15.54 | 15.03 | 18.33 | 19.60 | 14.85 | 15.88 | 15.95 |
|  | 2 | 6 | 14.61 | 15.04 | 15.10 | 14.71 | 15.28 | 15.24 | 14.89 | 17.25 | 16.60 | 14.76 | 15.53 | 15.64 |
|  | 2 | 8 | 14.58 | 15.00 | 14.88 | 14.65 | 15.03 | 15.11 | 14.82 | 15.88 | 17.01 | 14.70 | 15.35 | 15.49 |
|  | 2 | 12 | 14.57 | 14.99 | 14.80 | 14.60 | 15.00 | 14.88 | 14.74 | 15.76 | 15.58 | 14.64 | 15.20 | 15.09 |
|  | 4 | 4 | 14.76 | 15.71 | 15.93 | 14.91 | 15.89 | 15.93 | 15.29 | 23.93 | 22.17 | 15.04 | 17.26 | 17.27 |
|  | 4 | 12 | 14.59 | 14.86 | 14.95 | 14.66 | 15.08 | 15.10 | 14.92 | 16.12 | 16.22 | 14.73 | 15.37 | 15.39 |
|  | 6 | 6 | 14.74 | 15.63 | 15.85 | 14.88 | 15.82 | 15.86 | 15.37 | 20.37 | 22.04 | 15.04 | 16.62 | 17.53 |
|  | 6 | 12 | 14.62 | 15.13 | 15.05 | 14.71 | 15.30 | 15.32 | 15.04 | 16.51 | 17.33 | 14.80 | 15.57 | 15.60 |
|  | 12 | 12 | 14.70 | 15.27 | 15.27 | 14.86 | 15.71 | 15.57 | 15.29 | 23.96 | 21.56 | 15.01 | 16.29 | 16.41 |

Table 2. simulation results of UE Rx-Tx time difference accuracy (Tc) for 30KHz SCS

| BW (RB) | Comb | Syms | AWGN |  |  | TDL-A |  |  | TDL-B |  |  | TDL-C |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | cell1 | cell2 | cell3 | cell1 | cell2 | cell3 | cell1 | cell2 | cell3 | cell1 | cell2 | cell3 |
| 48 | 2 | 2 | 31.41 | 37.36 | 36.32 | 32.77 | 48.78 | 43.00 | 39.16 | 60.24 | 70.86 | 39.80 | 170.07 | 166.20 |
|  | 2 | 4 | 30.63 | 35.68 | 35.65 | 31.31 | 40.28 | 37.92 | 33.15 | 49.96 | 50.15 | 32.45 | 152.18 | 163.55 |
|  | 2 | 6 | 30.29 | 33.80 | 35.08 | 30.93 | 36.94 | 37.29 | 31.86 | 46.37 | 43.29 | 31.99 | 48.52 | 151.30 |
|  | 2 | 8 | 30.16 | 32.51 | 32.33 | 30.51 | 34.18 | 35.09 | 30.88 | 42.78 | 41.95 | 31.13 | 45.87 | 62.52 |
|  | 2 | 12 | 30.05 | 32.94 | 31.97 | 30.16 | 34.37 | 33.23 | 30.32 | 41.49 | 39.07 | 30.51 | 40.68 | 40.72 |
|  | 4 | 4 | 31.31 | 36.27 | 37.76 | 32.45 | 47.32 | 44.82 | 37.99 | 74.42 | 71.80 | 37.97 | 161.99 | 169.50 |
|  | 4 | 12 | 30.10 | 32.26 | 32.35 | 30.67 | 35.01 | 35.27 | 31.09 | 42.58 | 41.75 | 31.92 | 109.52 | 108.92 |
|  | 6 | 6 | 31.28 | 35.60 | 36.43 | 32.91 | 48.21 | 44.27 | 38.98 | 57.78 | 61.54 | 36.66 | 158.37 | 167.24 |
|  | 6 | 12 | 30.35 | 32.50 | 32.79 | 31.13 | 37.92 | 36.51 | 32.02 | 43.73 | 47.77 | 33.33 | 146.41 | 155.61 |
|  | 12 | 12 | 31.01 | 33.82 | 34.49 | 32.34 | 41.91 | 41.29 | 37.57 | 54.22 | 57.23 | 35.83 | 160.63 | 157.48 |
| 132 | 2 | 2 | 14.92 | 16.35 | 16.15 | 15.17 | 16.57 | 16.56 | 16.18 | 36.59 | 25.00 | 15.46 | 99.03 | 141.42 |
|  | 2 | 4 | 14.79 | 15.55 | 15.58 | 14.92 | 15.95 | 16.01 | 15.36 | 23.80 | 23.99 | 15.09 | 17.25 | 18.57 |
|  | 2 | 6 | 14.71 | 15.38 | 15.37 | 14.81 | 15.65 | 15.80 | 15.12 | 19.98 | 20.59 | 14.98 | 16.37 | 17.71 |
|  | 2 | 8 | 14.72 | 15.31 | 15.67 | 14.76 | 15.45 | 15.66 | 15.07 | 18.69 | 19.50 | 14.88 | 16.10 | 16.19 |
|  | 2 | 12 | 14.60 | 15.31 | 15.15 | 14.70 | 15.36 | 15.21 | 14.91 | 18.21 | 16.20 | 14.75 | 15.58 | 15.54 |
|  | 4 | 4 | 14.92 | 16.85 | 16.05 | 15.11 | 16.69 | 16.61 | 15.94 | 31.90 | 33.68 | 15.39 | 100.92 | 140.98 |
|  | 4 | 12 | 14.68 | 15.15 | 15.13 | 14.79 | 15.48 | 15.52 | 15.13 | 19.06 | 17.05 | 14.88 | 15.88 | 16.21 |
|  | 6 | 6 | 14.90 | 16.09 | 16.88 | 15.16 | 16.65 | 16.77 | 16.28 | 29.86 | 30.49 | 15.50 | 139.43 | 141.29 |
|  | 6 | 12 | 14.72 | 15.29 | 15.30 | 14.90 | 15.67 | 15.61 | 15.35 | 20.43 | 20.98 | 14.99 | 16.55 | 16.89 |
|  | 12 | 12 | 14.92 | 15.71 | 15.61 | 15.07 | 16.22 | 16.23 | 15.88 | 27.57 | 24.24 | 15.33 | 48.39 | 18.18 |
| 272 | 2 | 2 | 7.36 | 7.77 | 7.78 | 7.54 | 8.28 | 8.24 | 7.79 | 38.50 | 39.57 | 7.60 | 70.87 | 70.80 |
|  | 2 | 4 | 7.32 | 7.62 | 7.70 | 7.42 | 7.97 | 7.98 | 7.58 | 29.67 | 28.12 | 7.46 | 8.12 | 8.68 |
|  | 2 | 6 | 7.30 | 7.50 | 7.77 | 7.39 | 7.76 | 7.88 | 7.47 | 24.73 | 28.77 | 7.42 | 7.96 | 8.04 |
|  | 2 | 8 | 7.29 | 7.48 | 7.54 | 7.36 | 7.69 | 7.71 | 7.44 | 11.26 | 24.28 | 7.38 | 7.72 | 7.88 |
|  | 2 | 12 | 7.29 | 7.54 | 7.38 | 7.33 | 7.60 | 7.53 | 7.40 | 11.10 | 7.80 | 7.34 | 7.67 | 7.62 |
|  | 4 | 4 | 7.36 | 7.83 | 7.73 | 7.52 | 8.44 | 8.23 | 7.87 | 41.59 | 44.19 | 7.62 | 90.70 | 91.55 |
|  | 4 | 12 | 7.28 | 7.42 | 7.45 | 7.37 | 7.70 | 7.65 | 7.45 | 11.69 | 11.62 | 7.41 | 7.76 | 7.79 |
|  | 6 | 6 | 7.37 | 7.86 | 8.01 | 7.53 | 8.31 | 8.35 | 7.96 | 37.84 | 37.65 | 7.60 | 71.18 | 9.33 |
|  | 6 | 12 | 7.31 | 7.49 | 7.48 | 7.42 | 7.86 | 7.80 | 7.51 | 27.88 | 17.48 | 7.45 | 7.93 | 8.01 |
|  | 12 | 12 | 7.35 | 7.60 | 7.60 | 7.51 | 8.18 | 8.13 | 7.74 | 34.65 | 33.62 | 7.55 | 8.57 | 8.51 |

Table 3. simulation results of UE Rx-Tx time difference accuracy (Tc) for 120KHz SCS

| BW (RB) | Comb | Syms | AWGN |  |  | TDL-C |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | cell1 | cell2 | cell3 | cell1 | cell2 | cell3 |
| 32 | 2 | 2 | 15.90 | 19.09 | 19.64 | 17.90 | 29.72 | 29.01 |
|  | 2 | 4 | 15.51 | 17.64 | 18.32 | 16.13 | 21.80 | 21.90 |
|  | 2 | 6 | 15.30 | 17.29 | 17.32 | 16.00 | 20.28 | 19.57 |
|  | 2 | 8 | 15.20 | 16.89 | 16.44 | 15.66 | 18.39 | 18.92 |
|  | 2 | 12 | 15.05 | 16.33 | 15.87 | 15.42 | 18.11 | 17.59 |
|  | 4 | 4 | 15.99 | 20.98 | 19.34 | 17.24 | 32.15 | 26.83 |
|  | 4 | 12 | 15.20 | 16.44 | 16.46 | 15.80 | 19.37 | 19.19 |
|  | 6 | 6 | 15.94 | 19.01 | 18.86 | 17.16 | 28.09 | 32.71 |
|  | 6 | 12 | 15.35 | 17.24 | 16.96 | 16.10 | 20.51 | 21.43 |
|  | 12 | 12 | 15.86 | 17.72 | 18.30 | 16.95 | 26.21 | 27.15 |
| 64 | 2 | 2 | 7.77 | 8.98 | 9.06 | 8.10 | 39.25 | 41.96 |
|  | 2 | 4 | 7.60 | 8.36 | 8.98 | 7.83 | 9.49 | 9.42 |
|  | 2 | 6 | 7.53 | 8.29 | 8.26 | 7.72 | 8.80 | 8.94 |
|  | 2 | 8 | 7.46 | 7.98 | 8.11 | 7.65 | 8.48 | 8.87 |
|  | 2 | 12 | 7.49 | 8.39 | 7.77 | 7.57 | 8.34 | 8.12 |
|  | 4 | 4 | 7.80 | 9.12 | 9.67 | 8.07 | 48.04 | 36.56 |
|  | 4 | 12 | 7.47 | 7.99 | 7.95 | 7.67 | 8.47 | 8.64 |
|  | 6 | 6 | 7.76 | 8.92 | 8.80 | 8.13 | 40.18 | 18.16 |
|  | 6 | 12 | 7.56 | 8.05 | 8.25 | 7.75 | 9.08 | 9.02 |
|  | 12 | 12 | 7.73 | 8.36 | 8.64 | 8.00 | 10.00 | 9.94 |
| 128 | 2 | 2 | 3.79 | 4.33 | 4.36 | 3.95 | 4.80 | 4.87 |
|  | 2 | 4 | 3.74 | 4.08 | 4.14 | 3.83 | 4.44 | 4.41 |
|  | 2 | 6 | 3.71 | 3.97 | 4.14 | 3.78 | 4.23 | 4.20 |
|  | 2 | 8 | 3.69 | 3.91 | 3.94 | 3.75 | 4.06 | 4.08 |
|  | 2 | 12 | 3.67 | 3.82 | 3.87 | 3.73 | 4.00 | 3.97 |
|  | 4 | 4 | 3.80 | 4.43 | 4.53 | 3.94 | 5.03 | 5.34 |
|  | 4 | 12 | 3.70 | 3.86 | 3.89 | 3.77 | 4.08 | 4.18 |
|  | 6 | 6 | 3.79 | 4.19 | 4.16 | 3.94 | 4.73 | 4.95 |
|  | 6 | 12 | 3.73 | 3.94 | 3.97 | 3.81 | 4.22 | 4.26 |
|  | 12 | 12 | 3.77 | 4.03 | 4.06 | 3.91 | 4.54 | 4.62 |

Following observations can be made based on simulation results above.

Observation 1. UE Rx-Tx time difference measurement accuracy is sensitive to PRS BW.

Observation 2. Repitions of PRS resource, e.g., by increasing PRS symbol number, will improve measurement accuracy.

# Summary

In this contribution we provided simulation results for UE Rx-Tx time difference measurement accuracy. Following observations are made.

Observation 1. UE Rx-Tx time difference measurement accuracy are very sensitive to PRS BW.

Observation 2. Repitions of PRS resource, e.g., by increasing PRS symbol number, will improve measurement accuracy.

- References

- R4-2103586	WF on NR positioning UE performance requirements, Intel Corporation

- R4-2104092	Summary of link level simulation result of RSTD, PRS RSRP and UE DToA, Intel Corporation

- R4-2009127	Link-level simulation assumptions for UE Rx-Tx time difference measurements,	Qualcomm Incorporated