3GPP TSG-RAN WG4 Meeting # 98bis-e                                                         R4-2106652

Electronic Meeting, 12– 20 April, 2021

Source:	Huawei, HiSilicon

Title:	TP for TR 38.717-03-01 CA_n1A-n3A-n7A

Agenda item:	7.9.2

Document for:	Approval

Background

This contribution provides text proposal on the NR CA band combination CA_n1A-n3A-n7A as defined in [1].

Text Proposal

---Start of changes---

6.X	CA_n1-n3-n7

6.X.1	Operating bands for CA

Table 6.X.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n1-n3-n7 | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |

6.X.2	Channel bandwidths per operating band for CA

Table 6.X.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA config | UL config | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A-n7A | - | n1 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  | n3 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |

6.X.3	Co-existence studies

Co-existence studies can be omitted because fallback combinations cover these requirements and analysis.

6.X.4	∆TIB,c and ∆RIB,c values

For three simultaneous DLs and one UL of Band n1, n3 and n7, the TIB,c and RIB,c values are shown in table 6.X.4-1 and table 6.X.4-2, respectively.

Table 6.X.4-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n1-n3-n7 | n1 | 0.6 |
|  | n3 | 0.6 |
|  | n7 | 0.6 |

Table 6.X.4-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n1-n3-n7 | n1 | 0 |
|  | n3 | 0 |
|  | n7 | 0 |

6.X.5	REFSENS requirements

MSD studies can be omitted because the corresponding analysis and requirements are covered by fallback combinations.

---End of changes---

Reference

[1]	RP-210593, “Revised WID: Rel-17 NR inter-band CA for 3 bands DL with 1 band UL”, CATT