3GPP TSG-RAN WG4 Meeting # 98bis-e                                                         R4-2106653

Electronic Meeting, 12– 20 April, 2021

Source:	Huawei, HiSilicon

Title:	TP for TR 38.717-04-01 CA_n1A-n8A-n78A-n79A CA_n1A-n8A-n78(2A)-n79A

Agenda item:	7.10.2

Document for:	Approval

Background

This contribution provides text proposal on the NR CA band combination CA_n1A-n8A-n78A-n79A CA_n1A-n8A-n78(2A)-n79A  as defined in [1].

Text Proposal

---Start of changes---

5.x	CA_n1-n8-n78-n79

5.x.1	Operating bands for CA

Table 5.x.1-1: 4DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n1-n8-n78-n79 | n1 | 1710 MHz | - | 1785 MHz | 1805 MHz | - | 1880 MHz | FDD |
|  | n8 | 880 MHz | - | 915 MHz | 925 MHz | - | 960 MHz | FDD |
|  | n78 | 3300 MHz | - | 3800 MHz | 3300 MHz | - | 3800 MHz | TDD |
|  | n79 | 4400 MHz | - | 5000 MHz | 4400 MHz | - | 5000 MHz | TDD |

5.x.2	Channel bandwidths per operating band for CA

Table 5.x.2-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| NR CA config | UL config | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n8A-n78A-n79A | - | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n1A-n8A-n78(2A)-n79A | - | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

5.x.3	∆TIB,c and ∆RIB,c values

For four DLs of Band n1, n8, n78 and n79, the TIB,c and RIB,c values can be specified as below..

Table 5.x.3-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n1-n8-n78-n79 | n1 | 0.3 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |
|  | n79 | 0 |

Table 5.x.3-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n1-n8-n78-n79 | n1 | 0.2 |
|  | n8 | 0 |
|  | n78 | 0.5 |
|  | n79 | 0 |

5.x.4	REFSENS requirements

There are no additional MSD requirements for this band combination.

---End of changes---

Reference

[1]	RP-210760, “Revised WID: Rel-17 NR inter-band Carrier Aggregation for 4 bands DL with 1 band UL”, Ericsson