3GPP TSG-RAN WG4 Meeting #98-bis-e									   																				R4-2106668

Electronic Meeting, April 12th – 20th, 2021

Agenda item:			5.3.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	IAB RF conformance testing coverage considerations

Document for:	Approval

# Introduction

In this contribution we discuss the test burden of IAB-Nodes and how to avoid excessive testing while still maintaining test coverage in similar level as it is for gNBs.

# Discussion

A way forward agreed in RAN4#98-e outlining the high level principles of how test coverage can be maintained by means of reducing overlapping testing for IAB-MT and IAB-DU, in cases when they are RF implementation is either shared or uses identical hardware implementation [1]. Some key agreements from the way forward are reproduced below

- Test reduction cannot be applied when IAB-DU and IAB-MT are of different class.

- A new declaration or multiple new declarations are needed.

- Only when the side conditions are fulfilled, it is possible to declare shared or same RF HW. The reduction is allowed only when declaration for shared or same RF HW is done.

- When test reduction is applied, combined test coverage of IAB-MT and IAB-DU needs to be at least as large as it is for IAB-DU when test reduction is not applied.

- Test reduction cannot be applied for requirements which are defined for only IAB-DU or only IAB-MT.

- Test reduction cannot be applied when IAB-DU and IAB-MT are of different class.

- A new declaration or multiple new declarations are needed.

- Only when the side conditions are fulfilled, it is possible to declare shared or same RF HW. The reduction is allowed only when declaration for shared or same RF HW is done.

- When test reduction is applied, combined test coverage of IAB-MT and IAB-DU needs to be at least as large as it is for IAB-DU when test reduction is not applied.

- Test reduction cannot be applied for requirements which are defined for only IAB-DU or only IAB-MT.

Based on the way forward many of the principles are clearly laid out, but three major open issues exists: What are the side conditions when IAB-MT and IAB-DU are similar enough to allow test reduction, what is the exact test scope when reduction is applied, and how all this is captured into technical specifications. In the following we discuss these three aspects one at a time.

Side conditions for allowing declaration of shared/same RF HW

It has been previously agreed that declaration sets will be independent for IAB-DU and IAB-MT, which allows declaring different performance parameters, such as maximum output powers, for IAB-MT and IAB-DU. Therefore, it is reasonable to expect that not all declarations needs to be fully identical between IAB-MT and IAB-DU, as also the use case are different given that IAB-DU operates also directly with UEs, and not only in backhaul link. For example, demodulation related declarations can easily differ due to this. However, this should not be a motivation to reduce the test coverage.

Observation 1: As the use case for IAB-MT and IAB-DU is different, it is unnecessary to require all declarations to be identical for test reduction to apply

Rather than analysing each declaration individually, it is more important to understand the end goal: avoiding overlapping testing. In our view, the testing is overlapping in case the key test parameters are the same. This means that the final configuration of the test parameters should have

- the same wanted signal power, both for Rx (sensitivity) and Tx

- the same total transmission bandwidth

- the same beam directions

for both IAB-MT and IAB-DU.

In practice this means whether the test repetition optimisation applies is evaluated case by case for each test during the testing process. Consequently, there needs to be no agreement on specific side conditions which allow the declaration of shared/same RF implementation.

Proposal 1: Test reduction can be applied when the test parameters result in same wanted signal power, same total transmissions bandwidth and same beam directions, and when manufacturer has declared that the RF implementation is the same or shared between IAB-DU and IAB-MT.

Detailed test scope

Our proposal for the test scope is provided in table 1 to table 4. The tables contain the baseline of what is specified for gNBs which is applied for IAB-DU, and changes to that baseline is shown with revision marks.

Proposal 2: Adopt the test scope as captured in Table 1 to Table 4 when IAB-DU and IAB-MT use identical or shared RF HW.

Table 1: Radiated Tx requirements

| Tx requirement |  | Classification | Number of | Number of conformance directions for IAB-MT |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | conformance directions for IAB-DU |  | RF channels for IAB-DU | RF channels for IAB-MT | Comments |
| Radiated transmit power |  | Directional | 3 | 2 | B, T | M | Conformance directions for IAB-DU and IAB-MT shall be non-overlapping. Coverage of total number of directions and channels is equal to gNB test coverage. |
| OTA BS output power |  | TRP | See annex I |  | B, T | M |  |
| OTA output power dynamics |  | Directional | 1 | 1 | M | 1 | No reduction due to different requirment |
| OTA transmitter OFF power |  | Co-location | See clause 4.12 | - | M | - | FR1: It is sufficient to test only IAB-DU as the requirement is the same |
|  |  | Directional | 1 | - | M | - | FR2: It is sufficient to test only IAB-DU as the requirement is the same |
| OTA transient period |  | Co-location | See clause 4.12 | - | M | - | FR1: It is sufficient to test only IAB-DU as the requirement is the same |
|  |  | Directional | 1 | - | M | - | FR2: It is sufficient to test only IAB-DU as the requirement is the same |
| OTA modulation quality |  | Directional | 5 | - | B, T | - | Sufficient to test only IAB-DU as both IAB-MT and IAB-DU output CP-OFDM signals |
| OTA frequency error |  | Directional | 1 | 1 | B | T | Split testing between IAB-DU and IAB-MT |
| OTA time alignment error |  | Directional | 1 | N/A | M | N/A | Test reduction not possible as requirement is not defined for IAB-MT |
| OTA occupied bandwidth |  | Directional | 1 | - | M | - | Sufficient to test only IAB-DU as the requirement is the same as for IAB-MT |
| OTA ACLR |  | TRP | See annex I | - | B, T | - | Sufficient to test only IAB-DU as the requirement is the same or more stringent that for IAB-MT. |
| OTA operating band unwanted emission |  | TRP | See annex I | See annex I | B, T | M |  |
| OTA transmitter spurious emission | General requirement | TRP | See annex I | - | B below Tx signal frequency, T above Tx signal frequency | - | It is sufficient to test only IAB-DU as the emission performance of CP-OFDM signal does not depend on whether the underlying data content is UL or DL. |
|  | Additional spurious emissions | TRP | See annex I | - | B below Tx signal frequency, T above Tx signal frequency | - | It is sufficient to test only IAB-DU as the emission performance of CP-OFDM signal does not depend on whether the underlying data content is UL or DL. |
|  | Co-location with other base stations | Co-location | See clause 4.12 | - | M | - | Requirement is applicable only for FR1. It is sufficient to test only IAB-DU as the emission performance of CP-OFDM signal does not depend on whether the underlying data content is UL or DL. |
| OTA transmitter intermodulation |  | Co-location | See clause 4.12 | - | M | - | Requirement is applicable only for FR1. It is sufficient to test only IAB-DU as the emission performance of CP-OFDM signal does not depend on whether the underlying data content is UL or DL. |

Table 2: Radiated Rx requirements

| Rx requirement |  | Classification | Applicability levels |  | Number of conformance directions for IAB-DU | Number of conformance directions for IAB-MT | RF channels for IAB-DU | RF channels for IAB-MT | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | FR1 | FR2 |  |  |  |  |  |
| OTA sensitivity |  | Directional | Minimum EIS | N/A | 3 | 2 | M | M | Conformance directions for IAB-DU and IAB-MT shall be non-overlapping. |
| OTA reference sensitivity |  | Directional | OTA REFSENS | OTA REFSENS | 5 | 5 | B, T | M |  |
| OTA Dynamic range |  | Directional | OTA REFSENS | N/A | 1 | N/A | M | N/A | Test reduction not possible as requirement is not defined for IAB-MT |
| OTA adjacent channel selectivity |  | Directional | minSENS | OTA REFSENS |  | 1 |  | M | It is sufficient to test only IAB-MT as CP-OFDM interfering signal has higher PAR and therefore the test case is more challenging |
| OTA in-band blocking |  | Directional | OTA REFSENS and minSENS | OTA REFSENS |  | 5 |  | M | It is sufficient to test only IAB-MT as CP-OFDM interfering signal has higher PAR and therefore the test case is more challenging |
| OTA out-of-band blocking | General requirement | Directional | minSENS | OTA REFSENS | M | - | M | - | It is sufficient to test only IAB-DU as the requirement is the same |
|  | Co-location with other base stations | Co-location | minSENS | N/A | M | - | M | - | It is sufficient to test only IAB-DU as the requirement is the same |
| OTA receiver spurious emissions |  | TRP | See clause 7.7 | See clause 7.7 | See annex I | - | B below Rx signal frequency, T above Rx signal frequency | - | It is sufficient to test only IAB-DU as the requirement is the same |
| OTA receiver intermodulation |  | Directional | OTA REFSENS and minSENS | OTA REFSENS |  | 1 |  | M | Requirement is specified only in FR1 It is sufficient to test only IAB-MT as CP-OFDM interfering signal has higher PAR and therefore the test case is more challenging |
| OTA in-channel selectivity |  | Directional | minSENS | OTA REFSENS | 1 | - | M | - | Requirement is only defined for IAB-DU |

Table 3: Conducted Tx requirements

| Tx requirement |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | RF channels for IAB-DU | RF channels for IAB-MT | Comments |
| Maximum output power |  | B, T | M |  |
| Output power dynamics |  | M | M |  |
| Transmitter OFF power |  | M | - | Sufficient to test only IAB-DU as requirement is the same |
|  |  | M | - | Sufficient to test only IAB-DU as requirement is the same |
| Transient period |  | M | - | Sufficient to test only IAB-DU as requirement is the same |
|  |  | M | - | Sufficient to test only IAB-DU as requirement is the same |
| Modulation quality |  | B, T | M, | It would be sufficient to test only IAB-DU as the requirement is the same and both output CP-OFDM signals, but given so many channels are specified for IAB-DU, One channel is moved to IAB-MT to keep equivalent test scope and to test MT function also. |
| Frequency error |  | B, T | M |  |
| Time alignment error |  | M | N/A | Test reduction not possible as requirement is not defined for IAB-MT |
| Occupied bandwidth |  | M |  | Sufficient to test only IAB-DU as requirement is the same |
| ACLR |  | B, T | M |  |
| Operating band unwanted emission |  | B, T | M |  |
| Transmitter spurious emission | General requirement | B below Tx signal frequency, T above Tx signal frequency | - | It is sufficient to test only IAB-DU as the emission performance of CP-OFDM signal does not depend on whether the underlying data content is UL or DL. |
|  | Additional spurious emissions | B below Tx signal frequency, T above Tx signal frequency | - | It is sufficient to test only IAB-DU as the emission performance of CP-OFDM signal does not depend on whether the underlying data content is UL or DL. |
|  | Co-location with other base stations | B below Tx signal frequency, T above Tx signal frequency | - | It is sufficient to test only IAB-DU as the emission performance of CP-OFDM signal does not depend on whether the underlying data content is UL or DL. |
| OTA transmitter intermodulation |  | M |  | It is sufficient to test only IAB-DU as the emission performance of CP-OFDM signal does not depend on whether the underlying data content is UL or DL. |

Table 4: Conducted Rx requirements

| Rx requirement |  | RF channels for IAB-DU | RF channels for IAB-MT | Comments |
| --- | --- | --- | --- | --- |
| Reference sensitivity |  | B, T | M |  |
| Dynamic range |  | M | N/A | Test reduction not possible as requirement is not defined for IAB-MT |
| Adjacent channel selectivity |  |  | M | It is sufficient to test only IAB-MT as CP-OFDM interfering signal has higher PAR and therefore the test case is more challenging |
| In-band blocking |  |  | M | It is sufficient to test only IAB-MT as CP-OFDM interfering signal has higher PAR and therefore the test case is more challenging |
| Out-of-band blocking | General requirement | M |  | It is sufficient to test only IAB-DU as the requirement is the same |
|  | Co-location with other base stations | M |  | It is sufficient to test only IAB-DU as the requirement is the same |
| Receiver spurious emissions |  | M |  | It is sufficient to test only IAB-DU as the requirement is the same |
| Receiver intermodulation |  |  | M | It is sufficient to test only IAB-MT as CP-OFDM interfering signal has higher PAR and therefore the test case is more challenging |
| In-channel selectivity |  | M | N/A | Test reduction not possible as requirement is not defined for IAB-MT |

How to capture reduced test scope to TS

Some options how the test reduction is captured to TS were provided as part of the WF in [1]. In our view none of the options by itself is sufficient to provide the full picture and the achieve clarity and easy readability of the specification. The test reduction would require to be reflected in the specification in three different ways.

Firstly, one new declaration would need to be created to enable manufacturer to declare that RF implementation is either shared for IAB-MT and IAB-DU or the implementations are the same for both, even though the components are not shared.

Secondly, as the test reduction is mainly for tested RF channels or covering either IAB-MT or IAB-DU with the other one, the clearest way to capture the reduction scope is in the initial conditions of each tests. Initial conditions clause can include a separate description what is the test scope in case test reduction is applied.

Thirdly, with only the initial conditions the impact of test reduction would be scattered around the specification. Therefore, it is beneficial to provide a summary of the scope within clause 4 of the specification, in similar style that e.g. requirement set applicability is summarized.

Proposal 3: One new declaration is sufficient to state RF implementation is shared or the same for IAB-DU and IAB-MT

Proposal 4: Capture the reduced test scope as a section within initial conditions of each tests, and additional provide overview of the test scope in summary tables within clause 4 of the conformance specifications.

An example how the initial conditions section could look like is provided below:

#### Initial conditions

Test environment:

-	Normal, see annex B.2,

In case test reduction is applicable, RF channels to be tested for:

- IAB-DU: B, T

- IAB-MT: M.

Otherwise, RF channels to be tested for single carrier: B, M and T; see clause 4.9.1

In case test reduction is applicable, RF Bandwidth positions to be tested for multi-carrier and/or CA for:

- IAB-DU: BRFBW, TRFBW for single-band connector(s), see clause 4.9.1.

- IAB-MT: MRFBW for single-band connector(s), see clause 4.9.1.

- IAB-DU and IAB-MT: BRFBW_T'RFBW and B'RFBW_TRFBW for multi-band connector(s), see clause 4.9.1.

Otherwise, IAB-DU and IAB-MT RF Bandwidth positions to be tested for multi-carrier and/or CA:

-	BRFBW, MRFBW and TRFBW for single-band connector(s), see clause 4.9.1.

-	BRFBW_T'RFBW and B'RFBW_TRFBW for multi-band connector(s), see clause 4.9.1.

# Conclusion

In this contribution the test burden and test coverage analysis was provided to enable controlled reduction of test cases for IAB-Nodes. The following observation and proposals were made.

Proposal 1: Test reduction can be applied when the test parameters result in same wanted signal power, same total transmissions bandwidth and same beam directions, and when manufacturer has declared that the RF implementation is the same or shared between IAB-DU and IAB-MT.

Proposal 2: Adopt the test scope as captured in Table 1 to Table 4 when IAB-DU and IAB-MT use identical or shared RF HW.

Proposal 3: One new declaration is sufficient to state RF implementation is shared or the same for IAB-DU and IAB-MT

Proposal 4: Capture the reduced test scope as a section within initial conditions of each tests, and additional provide overview of the test scope in summary tables within clause 4 of the conformance specifications.

# References

[1] R4-2003857, WF on IAB-MT test coverage and IAB node test burden reduction, Nokia, Nokia Shanghai Bell