3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2106731

Electronic Meeting, 12 April – 20 April 2021

Source:	Ericsson, Bell Mobility

Title:	TP for TR 38.717-03-01 to include CA_n13-n66-n77

Agenda Item:	7.9.2

Document for:	Approval

# Introduction

This contribution provides a text proposal to introduce CA_n13A-n66A-n77A in 38.717-03-01.

# Reference

- R4-2100492 TR 38.717-03-01, Rel-17 NR inter-band Carrier Aggregation for 3 bands DL with 1 band UL, v0.3.0

# Text Proposal

# ---Start of changes---

## 6.X	CA_n13-n66-n77

### 6.X.1	Operating bands for CA

Table 6.X.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n13-n66-n77 | n13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.X.2	Channel bandwidths per operating band for CA

Table 6.X.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA Configuration | UL Config | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n13A-n66A-n77A | - | n13 | 15 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | n66 |  | 15 |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 30 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 60 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | n77 |  | 15 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 30 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 | 100 |  |
|  |  |  |  |  |  |  |  | 60 |  |  |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 | 100 |  |

### 6.X.3	Co-existence studies

For 3DL/1UL NR CA, only single uplink operation needs to be considered. For single uplink operation of this combination, only harmonic issue and harmonic mixing issue need to be considered.

Table 6.x.3-1 summarizes frequency ranges where harmonics occur due to 3DL bands CA with 1 UL. 2nd harmonic from n66 UL might fall into n77 DL.

Table 6.x.3-1: Harmonic Interference for 3DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n13 | 777 | 787 | 746 | 756 | 1554 | 1574 | 2331 | 2361 | 3108 | 3148 |
| n66 | 1710 | 1780 | 2110 | 2200 | 3420 | 3560 | 5130 | 5340 | 6840 | 7120 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 |

Table 6.x.3-2 gives harmonic mixing issue for the 3DL bands CA with 1 UL. 2nd harmonic mixing from n66 DL might fall into n77 UL.

Table 6.x.3-2 Harmonic mixing for 3DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n13 | 777 | 787 | 746 | 756 | 1492 | 1512 | 2238 | 2268 | 2984 | 3024 |
| n66 | 1710 | 1780 | 2110 | 2200 | 4220 | 4400 | 6330 | 6600 | 8440 | 8800 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 |

    - 6.X.4	∆TIB and ∆RIB values

For three simultaneous DLs and one UL of Band n13, n66 and n77, the TIB,c and RIB,c  values are shown in table 6.1.x.4-1 and  table 6.1.x.4-2, respectively. Values are derived from DC_13-66_n77.

Table 6.x.4-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n13-n66-n77 | n13 | 0.5 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |

Table 6.x.4-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n13-n66-n77 | n13 | 0.3 |
|  | n66 | 0.3 |
|  | n77 | 0.5 |

### 6.X.5	MSD

The 2nd harmonic and 2nd harmonic mixing issues between n66 and n77 will be addressed in lower order combinations.
---End of changes---