3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2106732

Electronic Meeting, 12 April – 20 April 2021

Source:	Ericsson, Bell Mobility

Title:	TP for TR 38.717-03-02: CA_n13-n66-n77

Agenda item:			7.11.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.717-03-02 to include CA_n13A-n66A-n77A as defined in WID [1].

# 2. Text Proposal

# ---Start of changes---

### 5.1.x	CA_n13A-n66A-n77A

#### 5.1.x.1	Operating bands for CA

Table 5.1.x.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n13-n66-n77 | n13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

#### 5.1.x.2	Channel bandwidths per operating band for CA

Table 5.1.x.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA Configuration | UL Config | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n13A-n66A-n77A | CA_n13A-n66A CA_n13A-n77A CA_n66A-n77A | n13 | 15 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | n66 |  |  | 15 |  |  | 5 |  |  | 10 |  |  | 15 |  |  | 20 |  |  | 25 |  | 30 |  |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | 30 |  |  |  |  |  | 10 |  |  | 15 |  |  | 20 |  |  | 25 |  | 30 |  |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | 60 |  |  |  |  |  | 10 |  |  | 15 |  |  | 20 |  |  | 25 |  | 30 |  |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | n77 |  |  | 15 |  |  |  |  |  | 10 |  |  | 15 |  |  | 20 |  |  | 25 |  |  | 30 |  |  | 40 |  | 50 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 30 |  |  |  |  |  | 10 |  |  | 15 |  |  | 20 |  |  | 25 |  |  | 30 |  |  | 40 |  | 50 |  |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 60 |  |  |  |  |  | 10 |  |  | 15 |  |  | 20 |  |  | 25 |  |  | 30 |  |  | 40 |  | 50 |  |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  |

#### 5.1.x.3	UE co-existence studies

IMD3, IMD4 and IMD5 generated by UL n13-n66 might affect DL n77.

IMD3 generated by UL n13-n77 might affect DL n66.

IMD3 generated by UL n66-n77 might affect DL n13.

#### 5.1.x.4	REFSENS requirements

MSD values n13 and n66 are derived from DC_13A-66A_n77A, and MSD values n77 are derived from DC_18A_n3A-n77A.

Below are the updates needed in Table 7.3A.5-2 of TS 38.101-1.

Table 5.1.x.4-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n13A-n66A-n77A | n13 | 782 | 5 | 25 | 751 | N/A | FDD | N/A |
|  | n66 | 1736 | 5 | 25 | 2136 | 17.1 | FDD | IMD3 |
|  | n77 | 3700 | 10 | 50 | 3700 | N/A | TDD | N/A |
|  | n13 | 781 | 5 | 25 | 750 | 15.2 | FDD | IMD3 |
|  | n66 | 1710 | 5 | 25 | 2110 | N/A | FDD | N/A |
|  | n77 | 4170 | 10 | 50 | 4170 | N/A | TDD | N/A |
|  | n13 | 782 | 5 | 25 | 751 | N/A | FDD | N/A |
|  | n66 | 1770 | 5 | 25 | 2170 | N/A | FDD | N/A |
|  | n771,2 | 3334 | 10 | 50 | 3334 | 16.3 | TDD | IMD3 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

---End of changes---

# Reference

[1] 		RP-202201, “Revised WID on Rel-17 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with 2 bands UL”, ZTE