3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2106773

Electronic Meeting, 12 April – 20 April 2021

Source:	Ericsson, T-Mobile US

Title:	TP for TR 38.717-02-01 to include BCS’s and UL configurations for n41-n66

Agenda item:			7.2.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.717-02-01 to include BCS’s and UL configurations for n41-n66 as defined in WID [1].

# 2. Text Proposal

---Start of changes---

## 6.x	CA_n41-n66

### 6.x.1	Common for 1 band UL and 2 bands DL CA

#### 6.x.1.1		Operating bands for CA

No updates compared to Rel-16 TR 38.716-02-00 v16.0.0

#### 6.x.1.2		Channel bandwidths per operating band for CA

Table 6.x.1.2-1: Supported bandwidths per CA band combination of band n41+n66

| NR CA configuration | NR Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Max Aggr BW [MHz] | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n41(2A)-n66A | CA_n41A-n66A | n41 | See CA_n41(2A) Bandwidth Combination Set 1 in 38.101-1 Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 230 | 1 |
|  |  | n66 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |  |

#### 6.x.1.3		UE co-existence studies

No updates compared to Rel-16 TR 38.716-02-00 v16.0.0

#### 6.x.1.4		∆TIB and ∆RIB values

No updates compared to Rel-16 TR 38.716-02-00 v16.0.0

#### 6.x.1.5		REFSENS requirements

Updates compared to clause 6.35.1.5 of TR 38.716-02-00 v16.0.0 in that MSD for channel BW 25 MHz and 30 MHz are added, see below:

A reference sensitivity exception (MSD) already exists in 38.101-3 due to cross band isolation for EN-DC of band n41 and 66 in NR FR1. The same should apply for CA_n41-n66 as given in Table 6.35.1.5-1 with uplink configuration specified in Table 6.35.1.5-2.

Table 6.35.1.5-1: MSD for the CA configuration for asynchronous operation and cross band isolation for CA

| NR CA Configuration CA_n41-n66 | NR UL band | NR DL band | Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
|  | n411 | n66 | 3.5 | 3.5 | 3.5 | 3.5 | 3.5 | 3.5 | 3.5 |  |  |  |  |  |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.35.1.5-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for CA

| Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR UL band | NR DL band | SCS of UL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n41 | n66 | 30 | 128 | 128 | 128 | 128 | 128 | 128 | 128 |  |  |  |  |  |

### 6.x.2	Specific for 2 bands UL CA

#### 6.x.2.1		UE co-existence studies

No updates compared to Rel-16 TR 38.716-02-00 v16.0.0

#### 6.x.2.2		REFSENS requirements

No updates compared to how it is already written in clause 6.35.2.2 of Rel-16 TR 38.716-02-00 v16.0.0, see below:

Based on the co-existence studies for CA_n41-n66 MSD does not need to be defined.

---End of changes---

# Reference

[1] 		RP-210495, Rel-17 NR Inter-band Carrier Aggregation/Dual Connectivity  for 2 bands DL with x bands UL (x=1,2), ZTE