3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2106775

Electronic Meeting, 12 April – 20 April 2021

Source:	Ericsson, T-Mobile US

Title:	TP for TR 38.717-02-01 to include BCS’s and UL configurations for n66-n71

Agenda item:			7.2.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.717-02-01 to include BCS’s and UL configurations for n66-n71 as defined in WID [1].

# 2. Text Proposal

---Start of changes---

## 6.x	CA_n66-n71

### 6.x.1	Common for 1 band UL and 2 bands DL CA

#### 6.x.1.1		Operating bands for CA

No updates compared to Rel-16 TR 38.716-02-00 v16.0.0

#### 6.x.1.2		Channel bandwidths per operating band for CA

Table 6.x.1.2-1: Supported bandwidths per CA band combination of band n66+n71

| NR CA configuration | NR Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Max Aggr BW [MHz] | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n66A-n71(2A) | CA_n66A-n71A | n66 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 70 | 0 |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |  |
|  |  | n71 | See CA_n71(2A) Bandwidth Combination Set 0 in 38.101-1 Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 6.x.1.3		UE co-existence studies

No updates compared to Rel-16 TR 38.716-02-00 v16.0.0

#### 6.x.1.4		∆TIB and ∆RIB values

No updates compared to Rel-16 TR 38.716-02-00 v16.0.0

#### 6.x.1.5		REFSENS requirements

No updates compared to how it is already written in clause 6.4.1.5 of Rel-16 TR 38.716-02-00 v16.0.0, see below:

There are no specific REFSENS requirements for 1 band UL.

### 6.x.2		Specific for 2 bands UL CA

#### 6.x.2.1		UE co-existence studies

No updates compared to Rel-16 TR 38.716-02-00 v16.0.0

#### 6.x.2.2		REFSENS requirements

No updates compared to how it is already written in clause 6.4.2.2 of Rel-16 TR 38.716-02-00 v16.0.0, see below:

MSD is defined for n66 as follows.

Table 6.4.2.2-1: 2DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n66A-n71A | n66 | 1750 | 5 | 25 | 2150 | 5 | FDD | IMD4 |
|  | n71 | 675 | 5 | 25 | 629 | N/A | FDD | N/A |

---End of changes---

# Reference

[1] 		RP-210495, Rel-17 NR Inter-band Carrier Aggregation/Dual Connectivity  for 2 bands DL with x bands UL (x=1,2), ZTE