3GPP TSG-RAN WG4 Meeting # 98-bis-e 	R4-2106783

# Electronic Meeting, Apr. 12-20, 2021

Title: 	Simulation results on NR-U PDSCH performance requirements

Source: 	Huawei, HiSilicon

Agenda item:	5.1.4.2

Document for:	Discussion

Background

At the last meeting, RAN 4 agreed to submit the simulation results for NR-U PDSCH performance requirements.  In this contribution, we provide our initial simulation results.

Discussions

The simulation assumptions derived from WF [1] are listed in Table 2-1~2-3

Table 2-1: Detailed simulation assumptions 1/3

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Active DL BWP index |  |  | 1 |
| Slot Pattern |  |  | [TBD, according to decision in the WF] |
| LBT failure probability |  |  | [0.50, 0.25] |
| Common serving cell parameters | Physical Cell ID |  | 0 |
|  | SSB position in burst |  | The first SSB |
|  | SSB Q factor |  | 8 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | According to DL Transmission Model |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| RB Allocation |  |  | Full BW |
| MIMO Layers |  |  | 2 |
| Number of HARQ Processes |  |  | 8 |
| TX EVM |  |  | 6% |

Table 2-1: Detailed simulation assumptions 2/3

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Slot Pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) |
| 1-1 | NR-U DL Transmission Model | [20, 40, 60, 80]/30 | 16 QAM, 0.48 | According to NR-U DL Transmission Model | TDLA30-10 | 2x2, ULA Low | 70 |
| 2-1 | NR-U DL Transmission Model | [20, 40, 60, 80]/30 | 16 QAM, 0.48 | According to NR-U DL Transmission Model | TDLA30-10 | 2x2, ULA Low | 70 |

Table 2-3: Detailed simulation assumptions 2/3 (DL Transmission model)

Simulation results are shown in Figure 2-1 and Table 2-4. We set duration of the downlink portion of the COT to {2, 3, 5, 6} slots and PDSCH allocation in the last slot of the downlink portion of the COT to {6, 9, 12, 14} slots. For TDD pattern, we use 7D11S2U, the reason why we use it is included in our discussion paper [2].

Figure 2-1: Simulation results for NR-U PDSCH requirements

Table 2-4: Summary of simulation results for target SNR

| SNR(dB)@70% of norminal maximum throughput | 2RX |  | 4RX |  |
| --- | --- | --- | --- | --- |
|  | Prob(LBT failure)=0.5 | Prob(LBT failure)=0.25 | Prob(LBT failure)=0.5 | Prob(LBT failure)=0.25 |
| 20MHz | 11.34 | 11.20 | 5.66 | 5.62 |
| 40MHz | 11.54 | 11.55 | 5.69 | 5.66 |
| 60MHz | 11.63 | 11.64 | 5.75 | 5.71 |
| 80MHz | 11.90 | 12.05 | 5.91 | 6.02 |

Conclusion

In this contribution, we provide our simulation results for NR-U PDSCH performance requirements.

Reference

[1]   R4-2103987 Way Forward on NR-U UE demodulation requirements, Qualcomm

[2]   R4-2106784 Discussions on NR-U UE performance requirements, Huawei, HiSilicon