3GPP TSG-RAN WG4 Meeting # 98-bis-e 	R4-2106796

# Electronic Meeting, Apr. 12-20, 2021

Title: 	Simulation results and discussions on NR-U PRACH requirements

Source: 	Huawei, HiSilicon

Agenda item:	5.1.4.4.4

Document for:	Discussion

Background

As per [1], RAN 4 agreed to introduce the performance requirements for NR-U PRACH, the simulation assumptions are basically determined but there are still some issues left. In this paper we will provide our simulation results and discussions about this issue.

Simulation results

Simulation results are shown in Figure 2-1 to 2-3 and summarized in Table 2-1 and simulation assumptions derived from [1] are shown in Appendix.

Figure 2-1: Simulation results for Format A2

Figure 2-2: Simulation results for Format B4

Figure 2-3: Simulation results for Format C2

Table 2-1: Summary of PRACH simulation results

| Format | AWGN |  | TDLA-30-10 |  |
| --- | --- | --- | --- | --- |
|  | LRA=1151 | LRA=571 | LRA=1151 | LRA=571 |
| A2 | -23.36 | -20.34 | -18.24 | -14.69 |
| B4 | -27.02 | -24.0 | -21.18 | -18.38 |
| C2 | -23.7 | -20.6 | -18.35 | -14.83 |

Discussions

One remain issue on PRACH requirements is declaration field. We list it as follows:

- Create new declaration field for extended PRACH sequences in 38.141-1 which includes format, SCS, and LRA. - Option 1: [D.111]; PRACH format with LRA = 1151 and LRA = 571 and SCS; Declaration of the supported PRACH format(s) as specified in TS 38.211 [17], i.e., format: A2, B4, C2. Declaration of the supported SCS(s) per supported PRACH format as specified in TS 38.211 [17], i.e., 15 kHz, 30 kHz or both.. - Option 2: [D.111]; PRACH format with LRA = 1151 and LRA = 571 and SCS; Declaration of the supported PRACH format(s) with LRA=1151 and LRA=571 as specified in TS 38.211 [17], i.e., format: A1, A2, A3, B4, C0, C2. Declaration of the supported SCS(s) per supported PRACH format, as specified in TS 38.211 [17], i.e., 15 kHz, 30 kHz or both, as well as sequence length other than LRA =139, i.e. LRA =1151 for 15 kHz SCS, and LRA =571 for 30 kHz SCS.

We prefer option 1, option 1 is a concise summary of all declaration fields. i.e.  Format and SCS. For option 2, since there are no requirements for A1, A3, C0, so they needn’t be included in declaration fields. Meanwhile, BS doesn’t need to declare the supported sequence length since it has been determined by declared SCS. i.e.1171 for 15kHz SCS and 571 for 30kHz.

Proposal 1:  Create new declaration field for extended PRACH sequences in 38.141-1 which includes format, SCS, and LRA.:

| [D.111] | PRACH format with LRA = 1151 and LRA = 571 and SCS | Declaration of the supported PRACH format(s) as specified in TS 38.211 [17], i.e., format: A2, B4, C2. Declaration of the supported SCS(s) per supported PRACH format as specified in TS 38.211 [17], i.e., 15 kHz, 30 kHz or both. |
| --- | --- | --- |

Conclusion

In this paper, we provide the simulation results and discussions on remain issues for NR-U PRACH. The proposals are:

Proposal 1:  Create new declaration field for extended PRACH sequences in 38.141-1 which includes format, SCS, and LRA.:

| [D.111] | PRACH format with LRA = 1151 and LRA = 571 and SCS | Declaration of the supported PRACH format(s) as specified in TS 38.211 [17], i.e., format: A2, B4, C2. Declaration of the supported SCS(s) per supported PRACH format as specified in TS 38.211 [17], i.e., 15 kHz, 30 kHz or both. |
| --- | --- | --- |

Reference

[1]   R4-2103808 Way forward on NR-U PRACH demodulation requirements. Nokia, Nokia Shanghai Bell

Appendix

Table 5-1: Simulation assumptions for wideband PRACH

| Parameters | Values |
| --- | --- |
| Format | A2, B4,C2 |
| SCS | 15kHz and 30kHz |
| Bandwidth | 20MHz |
| Ncs | 164 for LRA=1151 and 190 for LRA=571 |
| Antenna configuration | 1x2 |
| Propagation conditions | AWGN, TDLA-30-10 |
| v | 0 |
| Logical sequence index | 0 |
| Time estimation error tolerance | For AWGN: 0.52 us for 15 kHz; 0.26 us for 30kHz. For TDLA: 0.67 us for 15 kHz; 0.41 us for 30kHz. |
| Test metric | 0.1% of false alarm probability. 1% of PRACH detection error and time estimation error probability |