3GPP TSG-RAN WG4 Meeting # 98-bis-e 	R4-2106799

# Electronic Meeting, Apr. 12-20, 2021

Title: 	Simulation results on PSBCH requirements

Source: 	Huawei, HiSilicon

Agenda item:	5.2.1.2.3

Document for:	Discussion

Background

At last meeting, simulation assumptions for PSBCH were discussed. In this paper, we provide our simulation results based on simulation assumptions from [1].

Simulation results

Simulation results for PSBCH are shown in Figure 2-1 and Table 2-1. Corresponding simulation assumptions are shown in Table 5-1.

Figure 2-1: Simulation results for PSBCH

Table 2-1 Summary of PSBCH simulation results

| Propagation conditions | SNR@1% BLER |  |
| --- | --- | --- |
|  | Ideal results | Impairment results |
| TDLA-30-180 | -2.49 | -1 |

Conclusion

In this contribution, we provide our simulation results for PSBCH test.

Reference

[1]   R4-2103991 Updated simulation assumptions for V2X single link test cases

Appendix

Table 5-1 Simulation assumption for PSBCH demodulation performance

| Parameters | Unit | Value |
| --- | --- | --- |
| Test |  | BCH_Test1 |
| SLID |  | 0 |
| Synchronization source |  | SLSS |
| Propagation condition |  | TDLA30-180 |
| Channel bandwidth | MHz | 20 |
| Number SSB per SL period |  | 1 |
| Allocated resource blocks |  | 11 |
| Subcarrier spacing | kHz | 30 |
| Timing offset (Note2) |  | 0 |
| Frequency offset (Note 3) | Hz | 0 |
| The number of symbols (Note 1) |  | 9 |
| Modulation order |  | QPSK |
| Transport Block Size (without CRC) | Bits | 32 |
| Transport block CRC | Bits | 24 |
| Binary Channel Bits | Bits | 1782 |
| Antenna configuration |  | 1x2 Low |
| Performance metric |  | SNR@1% BLER of PSBCH |
| Note 1: The first OFDM symbol is used for AGC, not included in PSBCH decoding. OFDM symbols used for PSS/SSS transmission and guard symbol are not included either. Note 2: Time offset of Sidelink UE receive signal with respect to GNSS reference timing. Note 3: Frequency offset of Sidelink UE with respect to GNSS reference frequency. |  |  |