3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2106802

# Electronic Meeting, Apr.12-20, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0173 | rev |  | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for 38.101-4: Introduction of power imbalance requirements for NR V2X. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Perf |  |  |  |  |  | Date: |  |  | 2021-3-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RAN 4 has agreed to introduce NR V2X power imbalance test |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the requirments for power imbalance with two links. Added the RMC for power imbalance with two links. Added the related resource pool. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The performance requirements for NR V2X will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.6, A.6.2.2,A,7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521 -4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR … CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

11.1.6	Power imbalance performance with two links

11.1.6.1	2RX requirements

11.1.6.1.1	Minimum requirements

The purpose of this test is to check the demodulation performance when receiving PSSCH transmissions from two Sidelink UEs with power imbalance in one slot.

The minimum requirements are specified in Table 11.1.6.1.1-2 with the test parameters specified in Table 11.1.6.1.1-1. The Sidelink UE 1 and 2 are synchronized to GNSS or GNSS-equivalent synchronization reference.

Table 11.1.6.1.1-1:  Test Parameters

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Communication resource pool configuration |  |  | Configuration #1-V2X |
| Active cell(s) |  |  | None |
| Active Sidelink UE(s) |  |  | Sidelink UE 1, Sidelink UE 2 |
| Sidelink UE 1 | Sidelink Transmissions |  | PSCCH + PSSCH |
|  | RB allocation |  | PRB (0~9) |
|  | Time offset (Note 1) | s | 0 |
|  | Frequency offset (Note 2) | Hz | 0 |
|  | Antenna configuration |  | 1x2 Low |
|  | PSFCH periodicity | Slots | 4 |
|  | MinTimeGapPSFCH | Slots | 3 |
| Sidelink UE 2 | Sidelink Transmissions |  | PSCCH + PSSCH |
|  | RB allocation |  | PRB (30~39) |
|  | Time offset (Note 1) | s | 0 |
|  | Frequency offset (Note 2) | Hz | 0 |
|  | Antenna configuration |  | 1x2 Low |
|  | PSFCH periodicity | Slots | 4 |
|  | MinTimeGapPSFCH | Slots | 3 |
| Note 1: Time offset of received signal by Sidelink UE with respect to GNSS reference timing. Note 2: Frequency offset of Sidelink UE with respect to GNSS reference frequency. |  |  |  |

Table 11.1.6.1.1-2: Minimum performance

| Test number | Bandwidth (MHz)/ Subcarrier spacing(kHz) | Sidelink UE | PSSCH Reference channel | Modulation format and code rate | Propagation condition | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | PSSCH BLER (%) | SNR (dB) of PSSCH |
| 1 | 20/30 | 1 | R.PSSCH.2-1.4 | QPSK, 0.30 | AWGN | (Note 1) | 30.34 |
|  |  | 2 | R.PSSCH.2-1.4 | QPSK, 0.30 | AWGN | 10 | TBD |
| Note 1: There is no BLER requirement for Sidelink UE 1. |  |  |  |  |  |  |  |

<End of change>

<Start of change>

### A.6.2.2	Reference measurement channels for SCS 30 kHz FR1

Table A.6.2.2-1: PSSCH Reference Channel for V2X

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Reference channel |  |  | R.PSSCH.2-1.4 |
| Channel bandwidth |  | MHz | 20 |
| Subcarrier spacing |  | kHz | 30 |
| Allocated resource blocks |  | RB | 10 |
| CP-OFDM symbols for slot with PSFCH(Note 1) |  |  | 9 |
| CP-OFDM symbols for slot without PSFCH |  |  | 12 |
| DMRS symbols for slot with PSFCH |  |  | 2 |
| DMRS symbols for slot without PSFCH |  |  | 3 |
| Modulation order |  |  | QPSK |
| MCS index |  |  | 4 |
| Number of MIMO layers |  |  | 1 |
| NREDMRS(Note 2) |  |  | 15 |
| 2nd stage SCI format 2-A configuration | Payload | Bits | 35 |
|  | α |  | 1 |
|  | βoffset |  | 3.5 |
| Transport Block Size for slots with PSFCH |  | Bits | 208 |
| Transport Block Size for slots without PSFCH |  | Bits | 432 |
| Transport block CRC |  | Bits | 24 |
| Maximum number of HARQ transmissions |  |  | 1 |
| Binary Channel Bits for slots with PSFCH |  |  | 744 |
| Binary Channel Bits for slots without PSFCH |  | Bits | 1464 |
| Note 1: OFDM symbols is for PSCCH/PSSCH transmission not including first symbol (AGC), PSFCH symbols, and guard symbols Note 2: The parameters for derivation of DMRS overhead for TBS calculation. |  |  |  |

<End of change>

<Start of change>

A.7	SL reference resource pool configurations

Table A.7-1 V2X sidelink communication resource pool for PSSCH/PSCCH tests and power imbalance test (Configuration #1-V2X)

| Information Element |  |  | Value |
| --- | --- | --- | --- |
| SL-ResourcePool-r16 | sl-PSCCH-Config-r16 | sl-TimeResourcePSCCH-r16 | n2 |
|  |  | sl-FreqResourcePSCCH-r16 | n10 |
|  | sl-SyncAllowed-r16 |  | gnss-Sync-r16 |
|  | sl-SubchannelSize-r16 |  | n10 |
|  | sl-TimeResource-r16 |  | 160 |
|  | sl-StartRB-Subchannel-r16 |  | 0 |
|  | sl-NumSubchannel-r16 |  | 5 |
|  | sl-Additional-MCS-Table-r16 |  | Not presented |
|  | sl-RB-Number-r16 |  | 51 |
|  | sl-X-Overhead-r16 |  | n0 |

<End of change>