3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2106818

# Electronic Meeting, 12th - 20th Apr, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.174 | CR | - | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on IAB-MT conducted performance requirements (General and Demodulation) in TS 38.174 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Perf |  |  |  |  |  | Date: |  |  | 2021-04-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Provide initial draft CR for NR IAB-MT conducted performance requirements (General and Demodulation) as per work split. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For introducing IAB-MT conducted performance requirements (General and Demodulation), update clause 8.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be inconsistence between the specification 38.174 and RAN 4 agreements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.176-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF THE CHANGE 1>

8.2	IAB-MT performance requirements

8.2.1	General

8.2.1.1	Scope and definitions

Conducted performance requirements specify the ability of the IAB-MT type 1-H to correctly demodulate signals in various conditions and configurations. Conducted performance requirements are specified at the TAB connector(s) (for IAB-MT type 1-H).

Conducted performance requirements for the IAB-MT are specified for the fixed reference channels defined in annex A and the propagation conditions in annex X. The requirements only apply to those FRCs that are supported by the IAB-MT.

In tests performed with signal generators, a synchronization signal may be provided between the IAB node and the signal generator, or a common (e.g., GNSS) source may be provided to both IAB node and the signal generator, to enable correct timing of the wanted signal.

The SNR used in this clause is specified based on a single carrier and defined as:

SNR = S / N

Where:

S	is the total signal energy in the slot on a single TAB connector (for IAB-MT type 1-H).

N	is the noise energy in a bandwidth corresponding to the transmission bandwidth over the duration of a slot on a single TAB connector (for IAB-MT type 1-H).

8.2.1.2	Void

8.2.2	Demodulation performance requirements (Conducted requirements)

8.2.2.1	General

(Void)

8.2.2.2	PDSCH demodulation requirements

8.2.2.2.1	Minimum requirements for PDSCH Mapping Type A

The performance requirements are determined by a minimum required throughput for a given SNR. The throughput shall be equal to or larger than the fraction of maximum throughput for the FRCs defined in annex A, for the cases stated in Table 8.2.2.2.1-2 to 8.2.2.2.1-5 at the given SNR with the test parameters stated in Table 8.2.2.2.1-1.

Table: 8.2.2.2.1-1 Test parameters for testing PDSCH

| Parameter |  | Value |
| --- | --- | --- |
| Cyclic prefix |  | Normal |
| Default TDD UL-DL pattern (Note 1) |  | 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | DM-RS position (l0) | 2 |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 1 for Rank 1 and Rank 2 tests 2 for Rank 3 and Rank 4 tests |
|  | DM-RS port(s) | {1000} for Rank 1 tests {1000-1001} for Rank 2 tests {1000-1002} for Rank 3 tests {1000-1003} for Rank 4 tests |
|  | DM-RS sequence generation | NID0=0 |
| Time domain resource assignment | PDSCH mapping type | A |
|  | Start symbol | 2 |
|  | Allocation length | 12 |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
| PT-RS configuration |  | Not configured |
| PRB bundling size |  | 2 |
| VRB-to-PRB mapping type |  | Not interleaved |
| PDSCH & PDSCH DMRS Precoding configuration |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity |
| Note 1: The same requirements are applicable to TDD with different UL-DL patterns. |  |  |

Table 8.2.2.2.1-2: Minimum requirements for PDSCH Type A with Rank 1

| Test number | FRC (Annex A) | Modulation format and code rate | Bandwidth (MHz) / Subcarrier spacing (kHz) | Propagation conditions (Annex G) | Antenna configuration | Fraction of maximum throughput (%) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | G.PDSCH-FR1-Ax-x | 256QAM, 0.82 | 40/30 | TDLA30-10 | 2x2, ULA Low | 70 | 25.3 |
| 1-2 | G.PDSCH -FR1-Ax-x | 256QAM, 0.82 | 40/30 | TDLA30-10 | 2x4, ULA Low | 70 | 21.6 |
| 1-3 | G.PDSCH -FR1-Ax-x | 16QAM, 0.48 | 40/30 | TDLA30-10 | 2x2, ULA Low | 30 | [TBD] |
| 1-4 | G.PDSCH -FR1-Ax-x | 16QAM, 0.48 | 40/30 | TDLA30-10 | 2x4, ULA Low | 30 | [TBD] |

Table 8.2.2.2.1-3: Minimum requirements for PDSCH Type A with Rank 2

| Test number | FRC (Annex A) | Modulation format and code rate | Bandwidth (MHz) / Subcarrier spacing (kHz) | Propagation conditions (Annex G) | Antenna configuration | Fraction of maximum throughput (%) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2-1 | G.PDSCH -FR1-Ax-x | 64QAM, 0.50 | 40/30 | TDLA30-10 | 2x2, ULA Low | 70 | 19.8 |
| 2-2 | G.PDSCH -FR1-Ax-x | 64QAM, 0.50 | 40/30 | TDLA30-10 | 2x4, ULA Low | 70 | 13.6 |

Table 8.2.2.2.1-4: Minimum requirements for PDSCH Type A with Rank 3

| Test number | FRC (Annex A) | Modulation format and code rate | Bandwidth (MHz) / Subcarrier spacing (kHz) | Propagation conditions (Annex G) | Antenna configuration | Fraction of maximum throughput (%) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| [3-1?] | G.PDSCH -FR1-Ax-x | 16QAM, 0.48 | 40/30 | TDLA30-10 | 4x4, ULA Low | 70 | [TBD] |

Table 8.2.2.2.1-5: Minimum requirements for PDSCH Type A with Rank 4

| Test number | FRC (Annex A) | Modulation format and code rate | Bandwidth (MHz) / Subcarrier spacing (kHz) | Propagation conditions (Annex G) | Antenna configuration | Fraction of maximum throughput (%) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4-1 | G.PDSCH -FR1-Ax-x | 16QAM, 0.48 | 40/30 | TDLA30-10 | 4x4, ULA Low | 70 | 15.4 |

8.2.2.3	PDCCH demodulation requirements

8.2.2.3.1	Minimum requirements for PDCCH

The receiver characteristics of the PDCCH are determined by the probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg).

The Pm-dsg shall be equal to or smaller than 1%, for the cases stated in Table 8.2.2.3.1-2 at the given SNR with the test parameters stated in Table 8.2.2.3.1-1.

Table: 8.2.2.3.1-1 Test parameters for testing PDCCH

| Parameter | Value |
| --- | --- |
| Cyclic prefix | Normal |
| Default TDD UL-DL pattern (Note 1) | 7D1S2U, S=6D:4G:4U |
| DM-RS sequence generation | NID=0 |
| Frequency domain resource allocation for CORESET | Start from RB = 0 with contiguous RB allocation |
| CCE to REG mapping type | Interleaved |
| Interleaver size | 3 |
| REG bundle size | 6 for Test 5, 6 2 for others |
| Shift Index | 0 |
| Slots for PDCCH monitoring | Each slot |
| Number of PDCCH candidates for the tested aggregation level | 1 |
| PDCCH Precoding configuration | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination with REG bundling granularity for number of Tx larger than 1 |
| Note 1: The same requirements are applicable to FDD and TDD with different UL-DL patterns. |  |

Table 8.2.2.3.1-2: Minimum requirements for PDCCH

| Test number | Bandwidth (MHz) / Subcarrier spacing (kHz) | CORESET RB | CORESET duration | Aggregation level | FRC (Annex A) | Propagation conditions (Annex G) | Antenna configuration | Pm-dsg (%) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 40/30 | 102 | 1 | 2 | G.PDCCH-FR1-Ax-x | TDLA30-10 | 1x2, ULA Low | 1 | 7.0 |
| 2 | 40/30 | 102 | 1 | 2 | G.PDCCH -FR1-Ax-x | TDLA30-10 | 1x4, ULA Low | 1 | 2.1 |
| 3 | 40/30 | 102 | 1 | 4 | G.PDCCH -FR1-Ax-x | TDLA30-10 | 1x2, ULA Low | 1 | TBD |
| 4 | 40/30 | 102 | 1 | 4 | G.PDCCH -FR1-Ax-x | TDLA30-10 | 1x4, ULA Low | 1 | TBD |
| 5 | 40/30 | 90 | 1 | 8 | G.PDCCH -FR1-Ax-x | TDLA30-10 | 2x2, ULA Low | 1 | TBD |
| 6 | 40/30 | 90 | 1 | 8 | G.PDCCH -FR1-Ax-x | TDLA30-10 | 2x4, ULA Low | 1 | TBD |

8.2.2.4	PBCH demodulation requirements

(Void)

8.2.2.5	Sustained downlink data rate provided by lower layers

(Void)

8.2.2.5A	Sustained downlink data rate provided by lower layers

(Void)

<END OF THE CHANGE 1>