3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2106819

# Electronic Meeting, 12th - 20th Apr, 2021

Title: 	pCR on IAB conducted conformance testing (Manufacturer declarations) to TS 38.176-1

Source: 	Huawei, HiSilicon

Agenda item:	5.3.5.1

Document for:	Approval

- Introduction

During RAN4#98-e meeting, way forward [1] for NR IAB demodulation requirements was approved. This contribution provides the ininial text proposal about IAB conducted conformance testing (Manufacturer declarations) to TS 38.176-1 as per work split.

- Reference

- R4-2103994, WF on Rel-16 NR IAB demodulation requirements, RAN4#98-e, Nokia

Text Proposal

<START OF THE TEXT PROPOSAL 1>

x.y	Manufacturer declarations

x.y.1	Manufacturer declarations for IAB-DU

The following IAB-DU declarations listed in table x.y.1-1, when applicable to the IAB-DU under test, are required to be provided by the manufacturer for the conducted requirements testing of the IAB-DU type 1-H.

For the IAB-DU type 1-H declarations required for the radiated requirements testing, refer to TS 38.xxx-2 [TBD].

Table x.y.1-1 Manufacturer declarations for IAB-DU type 1-H conducted test requirements

| Declaration identifier | Declaration | Description |
| --- | --- | --- |
| D.100 | PUSCH mapping type | Declaration of the supported PUSCH mapping type as specified in TS 38.211 [x], i.e., type A, type B or both. |
| D.101 | PUSCH additional DM-RS positions | Declaration of the supported additional DM-RS position(s), i.e., pos0, pos1 or both. |
| D.102 | PUCCH format | Declaration of the supported PUCCH format(s) as specified in TS 38.211 [x], i.e., format 0, format 1, format 2, format 3, format 4. |
| D.103 | PRACH format and SCS | Declaration of the supported PRACH format(s) as specified in TS 38.211 [x], i.e., format: 0, A1, A2, A3, B4, C0, C2. Declaration of the supported SCS(s) per supported PRACH format with short sequence, as specified in TS 38.211 [x], i.e., 15 kHz, 30 kHz or both. |
| D.104 | Additional DM-RS for PUCCH format 3 | Declaration of the supported additional DM-RS for PUCCH format 3: without additional DM-RS, with additional DM-RS or both. |
| D.105 | Additional DM-RS for PUCCH format 4 | Declaration of the supported additional DM-RS for PUCCH format 4: without additional DM-RS, with additional DM-RS or both. |
| D.106 | PUCCH multi-slot | Declaration of multi-slot PUCCH support. |
| D.107 | UL CA | For the highest supported SCS, declaration of the carrier combination with the largest aggregated bandwidth. If there is more than one combination, the carrier combination with the largest number of carriers shall be declared. |
| D.108 | Modulation order for demodulation | Declaration of the supported modulation order, i.e. QPSK, 16QAM, 64QAM |
| D.109 | DFT-s-OFDM | Declaration of the supported of DFT-s-OFDM, i.e. supported or not supported. |

x.y.2	Manufacturer declarations for IAB-MT

The following IAB-MT declarations listed in table x.y.2-1, when applicable to the IAB-MT under test, are required to be provided by the manufacturer for the conducted requirements testing of the IAB-MT type 1-H.

For the IAB-MT type 1-H declarations required for the radiated requirements testing, refer to TS 38.xxx-2 [TBD].

Table x.y.2-1 Manufacturer declarations for IAB-DU type 1-H conducted test requirements

| Declaration identifier | Declaration | Description |
| --- | --- | --- |
| D.200 | 256QAM for demodulation | Declaration of the supported of 256QAM for demodulation, i.e. supported or not supported. |

<END OF THE TEXT PROPOSAL 1>