3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2106820

# Electronic Meeting, 12th - 20th Apr, 2021

Title: 	pCR on IAB-MT conducted conformance testing (CSI reporting and Interworking) to TS 38.176-1

Source: 	Huawei, HiSilicon

Agenda item:	5.3.5.3

Document for:	Approval

- Introduction

During RAN4#98-e meeting, way forward [1] for NR IAB demodulation requirements was approved. This contribution provides the ininial text proposal about IAB-MT conducted conformance testing (CSI reporting and Interworking) to TS 38.176-1 as per work split.

- Reference

- R4-2103994, WF on Rel-16 NR IAB demodulation requirements, RAN4#98-e, Nokia

Text Proposal

<START OF THE TEXT PROPOSAL 1>

8.2.3	CSI reporting requirements (Conducted requirements)

8.2.3.1	General

(Void)

8.2.3.2	Reporting of Channel Quality Indicator (CQI)

8.2.3.2.1	Minimum requirement for CQI reporting under AWGN conditions

The purpose of the requirements is to verify that the reported CQI values are in accordance with the CQI definition given in TS 38.214 [x]. The reporting accuracy of CQI under AWGN condition is determined by the reporting variance and BLER performance using the transport format indicated by the reported CQI median.

For the parameters specified in Table 6.2.2.2.1.1-1, and using the downlink physical channels specified in Annex X.X, the minimum requirements are specified by the following:

a)	The reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90% of the time.

b)	If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 0.1.

Table 6.2.2.2.1.1-1: CQI reporting definition test

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |  |  |  |
| Subcarrier spacing |  | kHz | 30 |  |  |  |
| Default TDD UL-DL pattern (Note 1) |  |  | 7D1S2U, S=6D:4G:4U |  |  |  |
| SNR |  | dB | 8 | 9 | 14 | 15 |
| Propagation channel |  |  | AWGN |  |  |  |
| Antenna configuration |  |  | 2×2 with static channel specified in Annex X.X |  |  |  |
| ReportConfigType |  |  | Periodic |  |  |  |
| CQI-table |  |  | Table 2 |  |  |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |  |  |
| cqi-FormatIndicator |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  | Wideband |  |  |  |
| Sub-band Size |  | RB | 16 |  |  |  |
| Csi-ReportingBand |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  | slot | 10/9 |  |  |  |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  | Codebook Mode |  | 1 |  |  |  |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |  |  |
|  | CodebookSubsetRestriction |  | 010000 |  |  |  |
|  | RI Restriction |  | N/A |  |  |  |
| Maximum number of HARQ transmission |  |  | 1 |  |  |  |
| Measurement channel |  |  | XXX |  |  |  |
| Note 1: The same requirements are applicable to TDD with different UL-DL patterns. |  |  |  |  |  |  |

8.2.3.3	Reporting of Precoding Matrix Indicator (PMI)

[TBD]

8.2.3.4	Reporting of Rank Indicator (RI)

[TBD]

8.2.3	Demodulation performance requirements for interworking

(Void)

8.2.3	CSI reporting requirements for interworking

(Void)

<END OF THE TEXT PROPOSAL 1>