3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2106822

# Electronic Meeting, 12th - 20th Apr, 2021

Title: 	pCR on IAB-MT radiated conformance testing (General and Demodulation) to TS 38.176-2

Agenda item:	5.3.5.3

Document for:	Approval

- Introduction

During RAN4#98-e meeting, way forward [1] for NR IAB demodulation requirements was approved. This contribution provides the ininial text proposal about IAB-MT radiated conformance testing (General and Demodulation) to TS 38.176-2 as per work split.

- Reference

- R4-2103994, WF on Rel-16 NR IAB demodulation requirements, RAN4#98-e, Nokia

Text Proposal

<START OF THE TEXT PROPOSAL 1>

8.2	IAB-MT performance requirements

8.2.1	General

8.2.1.1	Scope and definitions

Radiated performance requirements specify the ability of the IAB-MT type 1-O and IAB-MT type 2-O to correctly demodulate signals in various conditions and configurations. Radiated performance requirements are specified at the RIB.

Radiated performance requirements for the IAB-MT are specified for the fixed reference channels defined in annex A and the propagation conditions in annex X. The requirements only apply to those FRCs that are supported by the IAB-MT.

The radiated performance requirements for IAB-MT type 1-O and for IAB-MT type 2-O are limited to two OTA demodulations branches as described in clause 8.2.1.2. Conformance requirements can only be tested for 1 or 2 demodulation branches depending on the numbezr of polarizations supported by the IAB-MT, with the required SNR applied separately per polarization.

NOTE 1: IAB-MT can support more than 2 demodulation branches, however OTA conformance testing can only be performed for 1 or 2 demodulation branches.

In tests performed with signal generators, a synchronization signal may be provided between the IAB node and the signal generator, or a common (e.g., GNSS) source may be provided to both IAB node and the signal generator, to enable correct timing of the wanted signal.

The SNR used in this clause is specified based on a single carrier and defined as:

SNR = S / N

Where:

S	is the total signal energy in the slot on a single TAB connector (for IAB-MT type 1-H).

N	is the noise energy in a bandwidth corresponding to the transmission bandwidth over the duration of a slot on a single TAB connector (for IAB-MT type 1-H).

8.2.1.2	Applicability rule for IAB-MT

8.2.1.2.1	General

Unless otherwise stated, for a IAB-MT declared to support more than 2 demodulation branches (for IAB-MT type 1-O and IAB-MT type 2-O), the performance requirement tests for 2 demodulation branches shall apply, and the mapping between connectors and demodulation branches is up to IAB-MT implementation.

The tests requiring more than [20] dB SNR level are set to N/A in the test requirements.

8.2.1.2.2	Applicability of requirements for different subcarrier spacings

Unless otherwise stated, the tests shall apply only for each subcarrier spacing declared to be supported (see D.xxx in table xxx).

8.2.1.2.3	Applicability of requirements for different channel bandwidths

Unless otherwise stated, for each subcarrier spacing declared to be supported, the tests for a specific channel bandwidth shall apply only if the IAB-MT supports it (see D.xxx in table xxx).

Unless otherwise stated, for each subcarrier spacing declared to be supported, the tests shall be done only for the widest supported channel bandwidth. If performance requirement is not specified for this widest supported channel bandwidth, the tests shall be done by using performance requirement for the closest channel bandwidth lower than this widest supported bandwidth; the tested PRBs shall then be centered in this widest supported channel bandwidth.

8.2.1.2.4	Applicability of requirements for TDD with different UL-DL patterns

Unless otherwise stated, for each subcarrier spacing declared to be supported, if IAB-MT supports multiple TDD UL-DL patterns, only one of the supported TDD UL-DL patterns shall be used for all tests.

8.2.2	Demodulation performance requirements (Radiated requirements)

8.2.2.1	General

(Void)

8.2.2.2	PDSCH demodulation requirements

8.2.2.2.1	Minimum requirements for PDSCH Mapping Type A

The performance requirements are determined by a minimum required throughput for a given SNR. The throughput shall be equal to or larger than the fraction of maximum throughput for the FRCs defined in annex A, for the cases stated in Table 8.2.2.2.1-2 to 8.2.2.2.1-3 at the given SNR with the test parameters stated in Table 8.2.2.2.1-1.

Table: 8.2.2.2.1-1 Test parameters for testing PDSCH

| Parameter |  | Value |
| --- | --- | --- |
| Cyclic prefix |  | Normal |
| Default TDD UL-DL pattern (Note 1) |  | 3D1S1U, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | DM-RS position (l0) | 2 |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 1 |
|  | DM-RS port(s) | {1000} for Rank 1 tests {1000-1001} for Rank 2 tests |
|  | DM-RS sequence generation | NID0=0 |
| Time domain resource assignment | PDSCH mapping type | A |
|  | Start symbol | 1 |
|  | Allocation length | 13 |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
| PT-RS configuration | Frequency density (KPT-RS) | 2 |
|  | Time density (LPT-RS) | 1 |
| PRB bundling size |  | 2 |
| VRB-to-PRB mapping type |  | Not interleaved |
| PDSCH & PDSCH DMRS Precoding configuration |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity |
| Note 1: The same requirements are applicable to TDD with different UL-DL patterns. |  |  |

Table 8.2.2.2.1-2: Minimum requirements for PDSCH Type A with Rank 1

| Test number | FRC (Annex A) | Modulation format and code rate | Bandwidth (MHz) / Subcarrier spacing (kHz) | Propagation conditions (Annex G) | Antenna configuration | Fraction of maximum throughput (%) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | G.PDSCH-FR2-Ax-x | 16QAM, 0.48 | 100/120 | TDLA30-75 | 2x2, ULA Low | 30 | TBD |
| 1-2 | G.PDSCH –FR2-Ax-x | 64QAM, 0.46 | 100/120 | TDLA30-75 | 2x2, ULA Low | 70 | TBD |

Table 8.2.2.2.1-3: Minimum requirements for PDSCH Type A with Rank 2

| Test number | FRC (Annex A) | Modulation format and code rate | Bandwidth (MHz) / Subcarrier spacing (kHz) | Propagation conditions (Annex G) | Antenna configuration | Fraction of maximum throughput (%) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2-1 | G.PDSCH -FR1-Ax-x | 16QAM, 0.48 | 100/120 | TDLA30-75 | 2x2, ULA Low | 70 | TBD |
| 2-2 | G.PDSCH -FR1-Ax-x | 16QAM, 0.48 | 50/60 | TDLA30-75 | 2x2, ULA Low | 70 | 14.3 |
| 2-3 | G.PDSCH -FR1-Ax-x | 64QAM, 0.43 | 100/120 | TDLA30-75 | 2x2, ULA Low | 70 | 18.6 |

8.2.2.3	PDCCH demodulation requirements

8.2.2.3.1	Minimum requirements for PDCCH

The receiver characteristics of the PDCCH are determined by the probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg).

The Pm-dsg shall be equal to or smaller than 1%, for the cases stated in Table 8.2.2.3.1-2 at the given SNR with the test parameters stated in Table 8.2.2.3.1-1.

Table: 8.2.2.3.1-1 Test parameters for testing PDCCH

| Parameter | Value |
| --- | --- |
| Cyclic prefix | Normal |
| Default TDD UL-DL pattern (Note 1) | 3D1S1U, S=10D:2G:2U |
| DM-RS sequence generation | NID=0 |
| Frequency domain resource allocation for CORESET | Start from RB = 0 with contiguous RB allocation |
| CCE to REG mapping type | Interleaved |
| Interleaver size | 2 for Test 2 3 for others |
| REG bundle size | 6 for Test 2 2 for others |
| Shift Index | 0 |
| Slots for PDCCH monitoring | Each slot |
| Number of PDCCH candidates for the tested aggregation level | 1 |
| PDCCH Precoding configuration | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination with REG bundling granularity for number of Tx larger than 1 |
| Note 1: The same requirements are applicable to TDD with different UL-DL patterns. |  |

Table 8.2.2.3.1-2: Minimum requirements for PDCCH

| Test number | Bandwidth (MHz) / Subcarrier spacing (kHz) | CORESET RB | CORESET duration | Aggregation level | FRC (Annex A) | Propagation conditions (Annex G) | Antenna configuration | Pm-dsg (%) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 100/120 | 60 | 1 | 2 | G.PDCCH-FR1-Ax-x | TDLA30-75 | 1x2, ULA Low | 1 | 6.4 |
| 2 | 100/120 | 60 | 1 | 4 | G.PDCCH -FR1-Ax-x | TDLA30-75 | 1x2, ULA Low | 1 | TBD |
| 3 | 100/120 | 60 | 1 | 8 | G.PDCCH -FR1-Ax-x | TDLA30-75 | 2x2, ULA Low | 1 | 0.1 |

8.2.2.4	PBCH demodulation requirements

(Void)

8.2.2.5	Sustained downlink data rate provided by lower layers

(Void)

8.2.2.5A	Sustained downlink data rate provided by lower layers

(Void)

<END OF THE TEXT PROPOSAL 1>