3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2106824

# Electronic Meeting, 12th - 20th Apr, 2021

Title: 	Discussion on NR BS demodulation for 47GHz band

Source: 	Huawei, HiSilicon

Agenda item:	7.27.4.3

Document for:	Discussion

Background

At the RAN4#98-e meeting, the Way forward [1] on UE/BS demodulation on NR 47GHz band was approved. In this contribution, we share our views about the demodulation requirements for NR BS for 47GHz band.

Discussion

Requirements applicability

- Requirements applicability - The baseline assumption is the existing BS requirements are applicable up to 52.6GHz - Companies can bring more analysis to further check whether above assumption is valid or not

The first issue is that whether the existing BS performance requirements are applicable for 47GHz band. Here we provide the evaluation results on the impact of the Rx phase noise at 47GHz band as shown in the following Table 2-1 and Figure 2-1, using 64QAM and 567/1024 code rate that is the highest in current TS 38.104.

Table 2-1 Performance between modelling phase noise or not

| Case number | Modulation and code rate | Rx Phase noise model | SNR@70%maximum throughput |
| --- | --- | --- | --- |
| G-FR2-A5-9 in Table 11.2.2.1.2-4 of TS 38.101-4 | 64QAM, 567/1024 | Example 2, BS | 11.97 |
|  |  | N/A | 11.89 |

Figure 2-1 Performance between modelling phase noise or not

There is negligible performance difference with or without the phase noise modelling at 47GHz band for 64QAM and 567/1024 code rate.

The existing BS requirements are applicable for 47GHz band.

Proposals

In this contribution, we discuss on demodulation performance for NR UE demodulation for 47GHz band. Our observations and proposals are:

- There is negligible performance difference with or without the phase noise modelling at 47GHz band for 64QAM and 567/1024 code rate.

- The existing BS requirements are applicable for 47GHz band.

Reference

R4-2103875, Way forward on UE/BS demodulation on NR 47GHz band, RAN4#98-e, Ericsson