3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2106827

# Electronic Meeting, 12th - 20th Apr, 2021

Title: 	Discussion on NR FR2 HST deployment Scenario-B

Source: 	Huawei, HiSilicon

Agenda item:	8.7.2.2

Document for:	Discussion

Background

During RAN4#97-e meeting, Way forward [1] on Deployment Scenario and UE RF Requirement for FR2 HST was approved. In this contribution, we share our views about NR FR2 HST deployment Scenario-B.

Discussion

Here we analysis the link budget with the assumption shows in the following Table 2-1.

Table 2-1 Simulation assumptions for link budget analysis

| Parameter | Value |
| --- | --- |
| Carrier frequency | 30GHz |
| Ds | 700m |
| Dmin | 150m |
| RRH Tx power | 47dBm |
| RRH height | 15m |
| RRH antenna array | [Mg, Ng, M, N, P]=[1, 1, 8, 8, 2] |
| Path Loss | RMa LoS |
| UE antenna height | 5m |
| UE antenna array | [Mg, Ng, M, N, P]=[1, 1, 4, 4, 2] |
| UE noise figure | 10dB |
| ILs | 13 dB |
| SNR | 18.6dB (i.e. FR2 Test 2-6, 64QAM CR=0.43 and Rank2 in TS 38.101-4) |
| Others | In WF [1] |

Bi-directional

For Bi-directional deployment, one or two beam is assumed. The boresight is pointed to the railway in the middle point between 2 RRHs. The best beam in figures can be derived as the beam which has the strongest power among all beams.

The link budget remaining can be about 7.9 dB and 21.1dB for one and two beam case respectively. For one beam case, the power of side-lobes for different beams change rapidly when UE is near to the midpoint of two RRH and the minimum time duration for the best beam with same beam index can be about 200 ms that is a little larger than 160 ms that is the L1-RSRP measurement period for HST FR2 scenario as analysis in [2]. The link budget remaining is also degradation when UE is near to the RRH. For two beam case, the number of UE beam switching can be further reduced and the coverage when UE is under the RRH can be further improved. Therefore, we propose to use 2 beams for Bi-directional deployment for Scenario B.

Use 2 beams for Bi-directional deployment for Scenario B (700m/150m).

Uni-directional

For Uni-directional deployment, one or two beam is assumed. The boresight is pointed to the railway at the distance of Ds. The best beam in figures can be derived as the beam which has the strongest power among all beams.

The link budget remaining can be about 14.1 dB and 16.7 dB for one and two beam case respectively. For one beam case, the main-lobe of beams are used for all time. The coverage and the number of beam switching should be balanced for selecting the number of beams to be used. Considering sufficient link budget remaining for one beam case, we prefer to use one beam for Uni-directional deployment for Scenario B.

Use one beam for Uni-directional deployment for Scenario B.

Coverage

For downlink, as per Section 6.3.1 of TS 38.141-2, there is no upper limit for the rated carrier TRP of BS type 2-O. We don’t think there is any coverage issue under HST FR2 scenario for downlink.

The BS rated carrier TRP for BS type 1-O shall be within limits as specified in table 6.3.1-1. Table 6.3.1-1: BS rated carrier TRP limits for BS type 1-O BS class; Prated,c,TRP. Wide Area BS; (note). Medium Range BS; ≤ + 47 dBm. Local Area BS; ≤ + 33 dBm. NOTE: There is no upper limit for the Prated,c,TRP of the Wide Area Base Station.. There is no upper limit for the rated carrier TRP of BS type 2-O.

For uplink, UE maximum output power limits for PC4 is derived from TS 38.101-2 as following Table 2.3-1 shows. Maximum EIRP can be 43dBm at the candidate operating band, so the gap between RRH Tx power assumed in downlink budget and the maximum PC4 UE Tx power is 4dB. As per analysis in above clauses, the link budget remaining can be 17.1dB and 10.1dB for Bi-directional and Uni-directional deployment respectively for Scenario B for uplink that is sufficient.

Table 2.3-1: UE maximum output power limits for power class 4

| Operating band | Max TRP (dBm) | Max EIRP (dBm) |
| --- | --- | --- |
| n257 | 23 | 43 |
| n258 | 23 | 43 |
| n260 | 23 | 43 |
| n261 | 23 | 43 |

There is no any coverage issue under HST FR2 Scenario B assuming PC4 for both uplink and downlink.

Proposals

In this contribution, we discuss on NR FR2 HST deployment Scenario-B. Our observations and proposals are:

- There is no any coverage issue under HST FR2 Scenario B assuming PC4 for both uplink and downlink.

- Use 2 beams for Bi-directional deployment for Scenario B (700m/150m).

Use one beam for Uni-directional deployment for Scenario B.

Reference

R4-2103240, Way forward on Deployment Scenario and UE RF Requirement for FR2 HST, RAN4#98-e, Samsung

R4-2106825, Discussion on general issues for NR FR2 HST deployment scenario, RAN4#98-bis-e, Huawei, HiSilicon