3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2106830

# Electronic Meeting, 12th - 20th Apr, 2021

Title: 	Discussion on UE demodulation requirements for FR2 HST

Source: 	Huawei, HiSilicon

Agenda item:	8.7.5.2

Document for:	Discussion

Background

During RAN#97-e meeting, Work plan [1] for NR support for high speed train scenario in FR2 was approved. In this contribution, we share our views about the UE demodulation requirements for FR2 HST.

Discussion

Maximum Doppler

As per our contribution [2], 350km/h can be supported by using TRS+SSB. With the assumption of 29.5GHz carrier frequency, the maximum Doppler can be 9596Hz for 350km/h.

Channel model

As per our contribution [3], we propose to define performance requirements for single-tap and DPS for both Scenario A and B.

CBW

For CBW, we prefer to choose the typical one, such as 100 MHz.

DMRS

As per RAN1 feature list, DMRS 1+1+1 is the mandatory without capability signaling feature while DMRS 1+1+1+1 is the optional with capability signalling. So we prefer to use DMRS 1+1+1 for HST FR2 performance requirements definition.

Others

For other parameters, such as MCS, test metric, etc., it is reasonable to reuse the same value from HST FR1 performance requirements.

Define PDSCH performance requirements for HST FR2 with the following parameters:

| Parameter | Value |
| --- | --- |
| Maximum Doppler | 9596Hz |
| Channel model | single-tap, DPS |
| CBW/SCS | 100MHz/120kHz |
| PUSCH mapping | Type A, start symbol 1, duration 13 |
| DMRS | 1+1+1 |
| PTRS | KPTRS=2, LPTRS=1 |
| Antenna configuration | 2x2 |
| MCS | 17 |
| Test metric | 70% of maximum throughput |

Proposals

In this contribution, we discuss on demodulation performance for NR UE HST FR2. Our observations and proposals are:

- Define PDSCH performance requirements for HST FR2 with the following parameters:

| Parameter | Value |
| --- | --- |
| Maximum Doppler | 9596Hz |
| Channel model | single-tap, DPS |
| CBW/SCS | 100MHz/120kHz |
| PUSCH mapping | Type A, start symbol 1, duration 13 |
| DMRS | 1+1+1 |
| PTRS | KPTRS=2, LPTRS=1 |
| Antenna configuration | 2x2 |
| MCS | 17 |
| Test metric | 70% of maximum throughput |

Reference

R4-2014846, Work plan for NR support for high speed train scenario in FR2, RAN4#97-e, Samsung, Nokia, Nokia Shanghai Bell

R4-2106829, Discussion on general issues for NR FR2 HST demodulation requirements, RAN4#98-bis-e, Huawei, HiSilicon

R4-2106828, Discussion on channel modeling for NR FR2 HST, RAN4#98-bis-e, Huawei, HiSilicon