3GPP TSG-RAN WG4 #98bis-e	R4-2106855

Electronic Meeting, April 12 – April 20, 2021

Title: 					Discussions on handover test cases for NR-U

Source: 				Ericsson

Agenda item:	5.1.3.3.3

Document for:	Discussion

# Introduction

At previous RAN4 meeting three types of handover test cases were agreed as follows [1]:

- Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; known target cell

- Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell

- Inter-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell

The test case list was also updated to include follow additional test cases [1]:

| HO (delay and interruptions) | NR-U-> NR-U | intra-frequency, known | 6.1B | Yes | Huawei |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | intra-frequency, unknown |  | Yes |  |  |  |
|  |  | inter-frequency, unkown |  | Yes |  |  |  |
|  |  | Inter-frequency, known |  | FFS |  |  |  |
|  | NR(FR1) -> NR-U | known | 6.1B | Yes |  |  |  |
|  |  | unkown |  | Yes |  |  |  |
|  | NR-U -> NR(FR1) | known |  | Yes |  |  |  |
|  |  | unknown | 6.1.1.2 | Yes |  |  |  |
|  | NR-U - > E-UTRAN (FDD,TDD) |  | 6.1.2.1 | Yes |  |  |  |
|  | E-UTRAN (FDD,TDD) -> NR-U |  | TS 36.133 | Yes | Nokia |  |  |

Table 1 New handover test cases

In this contribution we provide views on testing for the new agreed handover test cases.

# Discussion

Test configurations

The test configurations depend on the type handover the UE is performing, e.g. whether it is from NR-U to NR-U, NR-U to NR, NR-U to E-UTRAN etc. Configurations as follows were used in the test cases endorsed at previous meeting:

Table 2 Different types of test configurations

| Configuration type | Description |
| --- | --- |
| NR-U | With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NR | Same as legacy |
| E-UTRAN | Same as legacy |

- Proposal #1: Following test configurations are proposed for handovers between NR-U and NR, and between NR-U and E-UTRAN:

Table 3 Test configurations for cell reselections between NR-U and NR

| Configuration | Description of a cell with CCA | Description of a cell without CCA |
| --- | --- | --- |
| 1 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Table 4 Test configurations for cell reselections between NR-U to E-UTRAN

| Configuration | Description of a cell with CCA | Description of a cell without CCA |
| --- | --- | --- |
| 1 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 2 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

In addition, the cell which is subject to CCA shall contain LBT configuration that specifies the probability of LBT failure. The agreed test cases from previous meeting include following LBT configurations which also apply to the new test cases:

- DL CCA model

- UL CCA model

- DBT Window Configuration

- DL CCA probability PCCA_DL

- UL CCA probability PCCA_UL

Moreover PDSCH reference measurement channel, RMSI CORESET and dedicated CORESET need to be defined differently.

- Proposal #2: Cell specific test parameters should contain following new or modified parameters to account for the LBT impact:

  - DL CCA model

  - UL CCA model

  - DBT Window Configuration

  - DL CCA probability PCCA_DL

  - UL CCA probability PCCA_UL

  - New RMCs

Test requirements:

The handover delay is verified during the handover test, and it is expressed as RRC procedure delay + Tinterrupt. The RRC procedure delay is assumed to be fixed and specified in TS 38.33, while the Tinterrupt delay is specified as follows [2]:

Tinterrupt = Tsearch + TIU + Tprocessing  + T∆ + Tmargin ms

The interruption delay contains following parameters which are CCA dependent:

- Tsearch is the time required to search the target cell when the target cell is not already known when the handover command is received by the UE.

  - Tsearch = 0 ms if the target cell is known.

  - Tsearch = (1+L1) *Trs if the target cell is an intra-frequency unknown cell

  - Tsearch = (3+L1´) *Trs if the target cell is an inter-frequency unknown cell.

- T∆ is time for fine time tracking and acquiring full timing information of the target cell.

  - T∆ = (1+ L2) *Trs ms

- TIU is the interruption uncertainty due to the random access procedure when sending PRACH to the new cell.

  - TIU = TSSB,RO + L3 * TRO + 10 ms  if not Type 2C UL channel access procedure is used

Where

- L1 and L1´ are the number of SMTC occasions not available at the UE during the intra-frequency and inter-frequency detection period, respectively,

- L2 is the number of SMTC occasions not available at the UE during the time tracking period,

- L3 is the number of consecutive PRACH occasions that are unavailable for PRACH transmission due to UL CCA failure.

The test requirements should verify the total handover delay which needs to be expressed using a formula for NR-U instead of a constant delay as in legacy test due to the CCA uncertainty. The delay will further depend on whether the target cell is known or unknown for which separate tests are defined.

It is important to note that the total handover delay which includes the extension caused by L1, L1’, L2 and L3 and UL CCA failure/detection mechanism is limited by the T304 timer. However, the UE behaviour associated with expiry of T304 timer does not need to be tested since it the timer is not specific to handover.

- Proposal #3: Handover delay verified in test requirements is expressed using a formula containing L1, L1’, L2 and L3 depending on the type of test case, and the total delay is limited by T304 timer.

# Conclusions

In this contribution we have discussed testing of handover requirements for NR-U and have made following proposal:

Proposal #1: Following test configurations are proposed for handovers between NR-U and NR, and between NR-U and E-UTRAN:

Table 3 Test configurations for cell reselections between NR-U and NR

| Configuration | Description of a cell with CCA | Description of a cell without CCA |
| --- | --- | --- |
| 1 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Table 4 Test configurations for cell reselections between NR-U to E-UTRAN

| Configuration | Description of a cell with CCA | Description of a cell without CCA |
| --- | --- | --- |
| 1 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 2 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

- Proposal #2: Cell specific test parameters should contain following new or modified parameters to account for the LBT impact:

  - DL CCA model

  - UL CCA model

  - DBT Window Configuration

  - DL CCA probability PCCA_DL

  - UL CCA probability PCCA_UL

  - New RMCs

- Proposal #3: Handover delay verified in test requirements is expressed using a formula containing L1, L1’, L2 and L3 depending on the type of test case, and the total delay is limited by T304 timer.

# References

- R4-2103523, “Updated test case list for NR-U”, Ericsson