3GPP TSG-RAN WG4 Meeting #98bis-e	R4-2106859

Electronic Meeting, 12 April – 20 April 2021

Agenda Item:	7.27.4.2

Source:	Ericsson

Title:	Applicability of FR2 UE demodulation requirements for NR 47GHz band

Document for:	Discussion

# 1	Introduction

RAN#98-e [1] agreed with the way forward on UE demodulation requirements for 47GHz.

- Whether the existing UE performance requirements applicable for 47GHz band or not - Interested companies are encouraged to provide the evaluation results whether the existing UE demodulation requirements are applicable for 47GHz band or not - For evaluation, use MCS 18 Rank 1 (Test 1-3 in TS38.101-4 Table 7.2.2.2.1-3) and MCS 17 Rank 2 (Test 2-6 in Table 7.2.2.2.1-4) for n262, as an example. - Other scenarios are not precluded - Phase noise (PN) model is up to UE implementation - Set Noc level for n262 in TS38.101-4 Table 4.5.3.2-1 based on REFSENS defined in RF session

This contribution discusses the applicability of the existing FR2 UE demodulation requirements to NR 47GHz band.

# 2	Discussion

RAN4#98-e discussed whether the existing FR2 UE demodulation performance requirements can be applicable for 47GHz band or not, and interested companies are encouraged to provide the simulation results by applying the phase noise. We have evaluated two scenarios to study the performance impacts due to the phase noise in higher carrier frequency as shown in Table 1.

Table 1	Evaluation scenarios for UE performance requirements in 47GHz band.

| Test case | Carrier frequency | CBW/SCS | MCS | Rank | Propagation condition | Antenna configuration |
| --- | --- | --- | --- | --- | --- | --- |
| Test 1 | 30GHz 45GHz | 100MHz / 120kHz | 64QAM CR=0.46 | 1 | TDLA30-300 | 2x2 ULA Low |
| Test 2 | 30GHz 45GHz | 50MHz / 120kHz | 256QAM CR=0.67 | 1 | TDLD30-75 | 2x2 ULA low |

Figure 1 compares the simulation results between (a) carrier frequency of 30GHz and (b) carrier frequency of 45GHz. In general, the impact of phase noise consists of common phase error (CPE) and inter-carrier interference (ICI). CPE causes the phase rotation and it affects especially to the higher order modulation such as 64QAM or 256QAM. On other hand, ICI is seen as Gaussian noise. It depends on the subcarrier spacing and phase noise bandwidth which error (CPE or ICI) is dominant. The figure shows the simulation results with the assumption UE applies CPE compensation (CPE comp) or ICI compensation (ICI comp).

| (a) Carrier frequency: 30GHz | (b) Carrier frequency: 45GHz |
| --- | --- |

Figure 1	Performance comparison between carrier frequency 30GHz and 45GHz.

If we focus on SNR to achieve 70% of maximum throughput (BLER=0.3 in the figure), the performance difference in Test 1 (64QAM) between 30GHz and 45GHz is less than 0.5dB. Even for test 2 (256QAM), we can observe the difference between 30GHz and 45GHz is not significant if UE apply the proper compensation techniques. In our understating, the impairment results provided by the companies should include the RF impairments caused by the phase noise, and we believe companies apply the proper compensation techniques to minimize the impact.

Observation: Existing FR2 UE demodulation requirements are applicable for NR 47GHz band.

Proposal: Extend the applicable FR2 operating bands in TS38.101-4 Clause 7.1.1.1 from 40000 MHz to 48200 MHz.

# References

- R4-2103875, “Way forward on UE/BS demodulation on NR 47GHz band”, Ericsson.