3GPP TSG-RAN WG4 Meeting #98bis-e	R4-2106862

Electronic Meeting, 12 April – 20 April 2021

Agenda Item:	8.6.3.2

Source:	Ericsson

Title:	Initial simulation result of PDSCH for CA in HST

Document for:	Information

# 1	Introduction

RAN4#98-e agreed with the way forward on PDSCH CA demodulation requirements for HST-SFN [1], where RAN4 agreed with the simulation assumption of HST-SFN/HST-DPS assuming the component carriers in CA. RAN#91-e also approved the revised WID [2], where HST-DPS is also the scope of PDSCH CA demodulation requirements.

This contribution provides our simulation results according to the simulation assumption.

# 2	Simulation results

## 2.1	HST-SFN joint transmission

| Test number | Duplex mode / SCS | Bandwidth (MHz) | MCS / Rank | Propagation condition | Antenna configuration |
| --- | --- | --- | --- | --- | --- |
| 1 | FDD SCS=15kHz | 5, 10, 15, 20, 25, 30, 40, 50 | MCS13 / Rank 2 | HST-SFN | 2x2 |
| 2 | FDD SCS=15kHz | 5, 10, 15, 20, 25, 30, 40, 50 | MCS13 / Rank 2 | HST-SFN | 2x4 |
| 3 | TDD SCS=30kHz | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | MCS13 / Rank 2 | HST-SFN | 2x2 |
| 4 | TDD SCS=30kHz | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | MCS13 / Rank 2 | HST-SFN | 2x4 |

Table 1 and Table 2 show our initial simulation results for HST-SFN joint transmission with FDD SCS=15kHz and TDD SCS=30kHz, respectively. Note we don’t consider additional margin of 1dB for CA requirements.

Table 1	HST-SFN simulation results of FDD SCS=15kHz for alignment.

|  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1 | 10.4 | 10.4 | 10.6 | 11.0 | 11.0 | 11.2 | 11.2 | 11.2 |
| Test 2 | 7.9 | 7.9 | 8.2 | 8.4 | 8.4 | 8.5 | 8.5 | 8.5 |

Table 2	HST-SFN simulation results of TDD SCS=30kHz for alignment.

|  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 3 | 11.1 | 11.5 | 11.8 | 11.6 | 11.8 | 12.0 | 12.1 | 12.1 | 11.8 | 11.8 | 11.8 | 11.7 |
| Test 4 | 9.1 | 9.6 | 9.9 | 9.7 | 9.9 | 10.2 | 10.2 | 10.3 | 10.2 | 10.2 | 10.3 | 10.2 |

## 2.2	HST-DPS

| Test number | Duplex mode / SCS | Bandwidth (MHz) | MCS | Propagation condition | Antenna configuration |
| --- | --- | --- | --- | --- | --- |
| 1 | FDD SCS=15kHz | 5, 10, 15, 20, 25, 30, 40, 50 | MCS17 / Rank 2 | HST-DPS | 2x2 |
| 2 | FDD SCS=15kHz | 5, 10, 15, 20, 25, 30, 40, 50 | MCS17 / Rank 2 | HST-DPS | 2x4 |
| 3 | TDD SCS=30kHz | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | MCS17 / Rank 2 | HST-DPS | 2x2 |
| 4 | TDD SCS=30kHz | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | MCS17 / Rank 2 | HST-DPS | 2x4 |

Table 3 and Table 4 show our initial simulation results for HST-DPS transmission with FDD SCS=15kHz and TDD SCS=30kHz, respectively. Note we don’t consider additional margin of 1dB for CA requirements.

Table 3	HST-DPS simulation results of FDD SCS=15kHz for alignment.

|  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1 | 9.7 | 10.1 | 10.1 | 9.7 | 10.1 | 10.2 | 10.1 | 10.2 |
| Test 2 | 7.2 | 7.3 | 7.3 | 7.2 | 7.3 | 7.4 | 7.3 | 7.4 |

Table 4	HST-DPS simulation results of TDD SCS=30kHz for alignment.

|  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 3 | 9.5 | 9.8 | 9.7 | 9.6 | 9.7 | 9.6 | 9.6 | 10.0 | 9.8 | 9.7 | 10.2 | 9.9 |
| Test 4 | 6.8 | 7.2 | 7.2 | 7.1 | 7.2 | 7.1 | 7.0 | 7.3 | 7.3 | 7.2 | 7.3 | 7.3 |

# 3	References

- R4-2103876, “WF on UE demodulation for FR1 HST”, CMCC.

- R4-210833, “Revised WID on enhanced NR support for high speed train scenario for frequency range 1 (FR1)”, CMCC.

# Appendix:	Simulation assumption

| Parameters | HST-SFN JT | HST-DPS |
| --- | --- | --- |
| DMRS type | Type 1 | Type 1 |
| Number of DMRS symbols | 1+1+1 | 1+1+1 |
| TDD pattern (for TDD) | 7DS2U for SCS=30kHz | 7DS2U for SCS=30kHz |
| MCS | MCS13 (64QAM table) | MCS17 (64QAM table) |
| TRS periodicity | 10ms, 2 slots pattern | 10ms, 2 slots pattern |
| PDSCH mapping | Type A, Start symbol: 2, Duration: 12 | Type A, Start symbol: 2, Duration: 12 |
| Ds | 700m | 700m |
| Dmin | 150m | 150m |
| Max Doppler shift | 870Hz for SCS=15kHz 1667Hz for SCS=30kHz | 870Hz for SCS=15kHz 1667Hz for SCS=30kHz |
| Rank | 2 | 2 |