3GPP TSG-RAN WG4 Meeting #98bis-e	R4-2106890

# Electronic meeting, April 12th-20th, 2021

Source:		Ericsson

Title:			Band n262 - BS conformance - MUs

Agenda item:		7.24.4.1

Document for:		Discussion

# Introduction

In RAN#88e, a WID to introduce the 47 GHz band was approved. The purpose of the WID is to specify NR band n262 covering the entire 47.2-48.2 GHz in order to accommodate operations in any part of this range.

In last RAN4#98-e meeting, BS MUs were discussed and an agreement ([2]) was made on the Tx MUs. But the Rx MUs reamined FFS.

This contribution is basically a re-submission of our tdoc submitted in last RAN4#98-e ([3]), re-affirming our position on Rx MUs for BS.

# Discussion

## MUs for Rx limits

In the  discussion thread on BS Rx MUs ([4]) in RAN4#98-e meeting, following values were discussed and proposed for agreements:

| Rx test | TS 38.141-2 24.25 ~ 29.5GHz | TS 38.141-2 37 ~ 40GHz | R4-2102049 | R4-2102445 | R4-2102160 |
| --- | --- | --- | --- | --- | --- |
| Reference sensitivity | 2.4 | 2.4 | 5.2 | 2.4 | 2.4 |
| ACS | 3.4 | 3.4 | 7.2 | 3.4 | 3.4 |
| IBB | 3.4 | 3.4 | 7.2 | 3.4 | 3.4 |
| Rx IM | 3.9 | 3.9 | 7.6 | 3.9 | 3.9 |
| In-channel selectivity | 3.4 | 3.4 | 7.2 | 3.4 | 3.4 |

The rationale for such high values (R4-2102409) was that there is no Vector Signal Generator to cover up this high frequency and a mixer should be then used for up conversion.

We understand the challenge to design such test equipment but we would like to highlight the following:

- If we the MU values might be up to 7dB, the limits checked with such equipment would not have any meaning.

- With careful design and calibration of the test environment, mixer uncertainty can mostly be removed by calibration.

- Even if the MU coming from a mixer might be large (e.g. ), its actual impact on the total MU remains limited: all contributors shall be RMS added in the linear scale.

For those reasons, we make the following proposal.

Proposal 1: Agree on following MU values for RX tests:

| Rx test | MU |
| --- | --- |
| Reference sensitivity | 2.4 |
| ACS | 3.4 |
| IBB | 3.4 |
| Rx IM | 3.9 |
| In-channel selectivity | 3.4 |

# Conclusion

In this contribution, we re-discussed the different MU proposals made in last RAN4#98-e meeting, and made the following proposals for the MU at 47.2-48.2 GHz.

Proposal 1: Agree on following MU values for RX tests:

| Rx test | MU |
| --- | --- |
| Reference sensitivity | 2.4 |
| ACS | 3.4 |
| IBB | 3.4 |
| Rx IM | 3.9 |
| In-channel selectivity | 3.4 |

# References

- RP-201232, “New WID on introduction of NR 47 GHz band”

- R4-2103874, TP to TR 38.847: BS conformance aspects, Nokia, Nokia Shanghai Bell

- R4-2102160, Band n262 - BS conformance - MUs , Ericsson

- R4-2103944, Email discussion summary for [98e][309] NR_47GHz_Band_BSRF, Nokia