3GPP TSG-RAN WG4 Meeting #98bis-e	R4-2106896

# Electronic meeting, April 12th – 20th, 2021

Source:		Ericsson

Title:			New band n85 – UE RF impacts

Agenda item:		7.30.1

Document for:		Discussion

# Introduction

In last RAN#90-e, a new WI has been approved to specify the new NR band n85, re-farmed from LTE band 85. This WI was revised in last RAN#91-e to add support for 15 MHz channel BW in n85.

This contribution is discussing the core UE RF impacts after this WI revision.

# Discussion

## Background

A Way Forward ([3]) was approved in last RAN4#98-e meeting listing all updates to be done in TS 38.104 and TS 38.141 when introducing band n85 in NR specifications.

The following section is discussing the additional ones when adding 15MHz channel BW in band n85.

## Revised WI impacts

The only additional updates to be considered are:

- To add 15MHz channel BW support in band n85 in table 5.3.5-1:

| NR band / SCS / BS channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
|  | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
| n85 | 30 |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |

- To update A-MPR table 6.2.3.1-1 for NS_06, as 15MHz channel BW is also supported:

| Network signalling label | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_06 | 6.5.2.3.4 | n12, n85 | 5, 10, 15 |  | N/A |
|  |  | n14, n85 | 5,10 |  |  |

## Rx requirements

### REFSENS

REFSENS requirements shall be added for 15MHz channel BW in table 7.3.2-1 and the corresponding RB allocation in table 7.3.2.-3:

The proposal here is to align REFSENS limits for 15MHz channel BW with those for band n12. Both bands are operating in the quasi-same frequency range, limits should then be the same.

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) | Duplex Mode |
| n85 | 15 | -97.0 | -93.8 | -84.0 |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -94.1 | -84.1 |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Operating band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Duplex Mode |
| n85 | 15 | 20 | 20 | 20 |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 10 | 10 |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# Conclusion

In this contribution, we analyzed the BS RF impacts with the revision the WI on introduction of band n85.

Proposal: Agree with the changes listed in this contribution (on top of the already agreed ones in [3]).

# References

- RP-202830, New WID on introduction of NR band n85, T-Mobile USA, Ericsson

- RP-210707, Introduction of NR band n85, T-Mobile USA, Ericsson

- R4-2103219, WF on the introduction of band n85, Ericsson