3GPP TSG-RAN WG4 Meeting # 98-bis-e	R4-2106915

Electronic Meeting, Apr. 12-20, 2021

Source:	ZTE Corporation

Title:	On RLM and RLF relaxation for UE power saving

Agenda item:	8.9.2

Document for:	Discussion

# 1	Introduction

In RAN4 98-e, work plan and general aspects for the new work item UE power saving enhancement were discussed. Technical discussions related to RLM and RLF were also triggered during the meeting and companies’ views were summarized in WF [1], the content copied below:

- This paper discusses the open issues listed above in the WF and provide our view.

2	Discussions

We give our views on the above issues one by one.

As to whether the UE or the network can determine for the UE to enter a mode of low mobility and then apply relaxed RLM/BLF requirements, the difference is not fundamental. In CONNECTED mode, the UE will do measurement as configured by the network and report the results to the network so the mobility status of the UE is known to both the network and the UE.

Observation 1: The mobility status of the UE is known to both the network and the UE in CONNECTED mode.

Our thinking is that the relaxation criteria shall be configured by the network to the UE. If the threshold (criteria) is not configured, it means the UE cannot go into relaxation mode.

The relaxation criteria shall be configured by the network to the UE. If the threshold (criteria) is not configured, it means the UE cannot go into relaxation mode.

Having network configuring UE to relax requirements would require extra signalling and increase the overhead, which can be seen as a drawback. Thus, if correct UE behaviors can be tested, we prefer to allow the UE to make decision locally. However, since the WI focuses on low mobility UEs, how to evaluate the UE speed and how to configure such speed in test cases remains unclear. To our understanding the UE speed can only be configured implicitly in the tests.

The UE can determine alone if the criteria (configured by the network) is met and enter the low mobility mode to use a relaxed requirements for RLM and RLF if there will be test cases defined to test the UE behaviors.

We confirm that the options suggested in the WF for the UE to retreat from low mobility mode and resume applying normal RLM / BLF requirements make sense and shall be specified.

The UE while performing relaxed RLM shall revert to the normal RLM operation (i.e. without relaxation) if the relaxation criterion is not met or N310 starts to count (1 out-of-sync indication).

The UE shall revert to the normal BFD operation upon detect 1 beam failure instance indication.

As for intra-band CA case, currently we think that the UE should relax only on serving cells where the relaxed criteria is fulfilled. This is also aligned with the motivation of the WI, which is to allow the UE save power when the conditions are met. When the criteria is not met, the legacy requirements shall apply.

For intra-band CA case, the UE should relax only on serving cells where the relaxed criteria is fulfilled.

As to the criteria, we think UE mobility should be a major factor to be aligned with the WI proposal [2].

Take UE mobility as the major factor into the criteria.

3	Conclusion

Observation 1: The mobility status of the UE is known to both the network and the UE in CONNECTED mode.

Proposal 1: The relaxation criteria shall be configured by the network to the UE. If the threshold (criteria) is not configured, it means the UE cannot go into relaxation mode.

- Proposal 2: The UE can determine alone if the criteria (configured by the network) is met and enter the low mobility mode to use a relaxed requirements for RLM and RLF if there will be test cases defined to test the UE behaviors.

Proposal 3: The UE while performing relaxed RLM shall revert to the normal RLM operation (i.e. without relaxation) if the relaxation criterion is not met or N310 starts to count (1 out-of-sync indication).

Proposal 4: The UE shall revert to the normal BFD operation upon detect 1 beam failure instance indication.

Proposal 5: For intra-band CA case, the UE should relax only on serving cells where the relaxed criteria is fulfilled.

Proposal 6: Take UE mobility as the major factor into the criteria.

References

- R4-2103670, WF on NR UE Power Saving Enhancements, MediaTek Inc.

- RP-200938, Revised WID UE Power Saving Enhancements for NR, MediaTek Inc.