3GPP TSG-RAN4 Meeting #98-bis-e	R4-2107002

# Electronic Meeting, April 12 –20, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draft | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to update UE Rx-Tx time difference measurement requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Core |  |  |  |  |  | Date: |  |  | 2021-03-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There are some remaining open issues in UE Rx-Tx measurement period: - Whether and how to account for resource muting in measurement period - Derivation of TPRS,i considering different resource periodicity - Impact of different resource offsets on measurement period - Definition of Lprs - How to capture measurement period equations for overlapping case - Measurement period equations for non-overlapping case |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the UE Rx-Tx measurement period requirements for above issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE Rx-Tx measurement period requirements are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.9.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

9.9.4.5 Measurement Period Requirements

When physical layer receives last of NR-Multi-RTT-ProvideAssistanceData message and NR-Multi-RTT-RequestLocationInformation message from LMF via LPP [34], UE shall be able to measure multiple (up to the UE capability specified in clause 9.9.4.3) UE Rx-Tx time difference measurements as defined in TS 38.215 [4] in configured positioning frequency layers within the measurement period TUERxTx,Total ms.

TUERxTx, Total=i=1LTUERxTx,i+L-1*maxTeffect,i.

where i is the index of PRS frequency layer,

TUERxTx,i is the measurement period for PRS frequency layer i when positioning frequency layer i is measured without other positioning frequency layer and is as further defined in this clause,

L is total number of positioning frequency layers, and

Teffect,i is the periodicity of the UE Rx-Tx time difference measurement in PRS frequency layer i

TUERxTx,i=CSSFi*NRxBeam,i*NPRS,islotN'LPRS,iN*Nsample-1*Teffect,i+Tlast

Where

CSSFi is scaling factor for NR positioning measurements as defined in clause 9.1.5.2,

NRxBeam,i is the scaling factor for Rx beam sweeping, and NRxBeam,i=1 if PRS layer i is in FR1 and NRxBeam,i=8 if PRS layer i is in FR2,

LPRS,i is the time duration specified in clause 5.1.6.5 of TS 38.214 [26, clause 5.1.6.5] calculcated over the time period of TPRS,i, and only includes the duration of PRS resources that are not muted and fall within MG

NPRS,islot is the maximum number of DL PRS resources of frequency layer i configured in a slot,

{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms processed every T ms for a given maximum bandwidth supported by UE as specified in clause 4.2.7.2 of TS 38.306 [14],

N’ is UE capability for number of DL PRS resources that it can process in a slot as specified in clause 4.2.7.2 of TS 38.306 [14],

Nsample is the number of UE Rx-Tx time difference measurement samples and Nsample= [4],

Teffect,i is periodicity of UE Rx-Tx time difference measurement in frequency layer i:

Teffect,i= TiTavailable_PRS,i*Tavailable_PRS,i

where Tavailable_PRS,i= LCMTPRS,i,MGRPi, the least common multiple between TPRS,i and MGRPi

TPRS,i=LCMj(Tresource,j*Nmuting,j),

Tresource,j is the configured periodicity of PRS resource j on the positioning frequency layer i, and

Nmuting,j=X*Y is the scaling factor if muting option 1 is applied on PRS resource j, provided that PRS resource j is fully or partly within the measurement gap, where X is the size of NR-MutingPattern-r16 for mutingOption1-r16, and Y is the value of dl-prs-MutingBitRepetitionFactor.

MGRPi is the repetition periodicity of the measurement gap applicable for measurement in frequency layer i.

Tlast is the measurement duration for the UE Rx-Tx time difference measurement last sample, including the sampling time and processing time, Tlast = k*Ti + LPRS,i , where k=2 if there are more than one measurement gap occasions in any time duration of Ti with PRS resource occasions, and k=1 otherwise

If the frequency layer i has more than one DL PRS resource sets with different PRS periodicities, the maximum PRS periodicity among DL PRS resource sets is used to derive the measurement period of that positioning frequency layer.

The time TUERxTx,i starts from the first MG instance aligned with DL PRS resources of positioning frequency layer i closest in time after both the NR-Multi-RTT-RequestLocationInformation message and NR-Multi-RTT-ProvideAssistanceData message from LMF via LPP [34] are delivered to the physical layer of UE.

The UE Rx-Tx time difference measurement period is restarted if HO occurs during the measurement period and after SRS reconfiguration on the target cell is complete.

Editors Note: The applicability of requirements if the time duration of a DL PRS resource exceeds the UE capability N.

<End of Change 1>