3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2107023

Electronic Meeting, 12 – 20 April, 2021

Title: 	Discussion on CSI-RSRP accuracy requirements

Source: 	Huawei, HiSilicon

Agenda item:	5.7.2.2.1

Document for:	Discussion

Introduction

CSI-RSRP accuracy requirements have been discussed in RAN4#98-e. The outcomes are captured in the WF [1]. Most open issues in defining the requirements were closed, and the only remaining open issue is the side condition of the upper bound of timing offset:

- Issue 2-1-1: The upper bound of timing offset for case 1 - For CSI-RSRP and CSI-RSRQ the upper bound of timing offset for case 1 is TBD - Option 1: 1 CP - Option 2: 0.9 CP - Companies are encouraged to bring additional simulation results for different values of timing offset and evaluate both positive and negative timing offsets. - Note: the timing offset value in the test cases can be specified in []

To determine the side condition, an updated simulation assumption was approved [2]. In this paper we will provide additional simulation results and our views on the timing offset condition.

Discussion

Simulation results

The simulation results in this section are based on assumptions in [2]. The 5%- and 95%-tile of CSI-RSRP error are shown in Table 1~3 for 15kHz, 30kHz and 120kHz, respectively.

Table 1: CSI-RSRP accuracy with 15kHz SCS

| Timing error | SNR | Channel |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | AWGN |  | ETU70 |  |
|  |  | 5% | 95% | 5% | 95% |
| 0 | -6 | -0.89 | 1.53 | -2.15 | 1.56 |
| -0.5*CP |  | -1.02 | 1.45 | -2.09 | 1.55 |
| 0.5*CP |  | -0.85 | 1.6 | -2.02 | 1.58 |
| -0.9*CP |  | -0.53 | 1.8 | -1.92 | 1.74 |
| 0.9*CP |  | -0.58 | 1.86 | -2.13 | 1.71 |
| -CP |  | -0.92 | 1.56 | -2.07 | 1.59 |
| CP |  | -0.94 | 1.89 | -2.01 | 1.9 |

Table 2: CSI-RSRP accuracy with 30kHz SCS

| Timing error | SNR | Channel |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | AWGN |  | EPA5 |  |
|  |  | 5% | 95% | 5% | 95% |
| 0 | -6 | -1.02 | 1.49 | -1.33 | 2.39 |
| -0.5*CP |  | -1.08 | 1.44 | -1.4 | 2.39 |
| 0.5*CP |  | -1.01 | 1.56 | -1.32 | 2.38 |
| -0.9*CP |  | -0.58 | 1.78 | -1.19 | 2.36 |
| 0.9*CP |  | -0.67 | 1.84 | -1.15 | 2.59 |
| -CP |  | -0.96 | 1.56 | -1.38 | 2.44 |
| CP |  | -0.97 | 1.88 | -1.38 | 2.9 |

Table 3: CSI-RSRP accuracy with 120kHz SCS

| Timing error | SNR | Channel |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | TDL-A30-3 |  | TDL-C-30-30 |  |
|  |  | 5% | 95% | 5% | 95% |
| 0 | -6 | -1.75 | 2.28 | -1.4 | 2.1 |
| -0.5*CP |  | -1.72 | 2.37 | -1.51 | 2.11 |
| 0.5*CP |  | -1.75 | 2.38 | -1.54 | 2.08 |
| -0.9*CP |  | -1.51 | 2.47 | -1.4 | 2.2 |
| 0.9*CP |  | -1.67 | 2.47 | -1.4 | 2.17 |
| -CP |  | -1.97 | 2.14 | -1.47 | 2.1 |
| CP |  | -1.75 | 2.77 | -1.43 | 2.58 |

Timing offset side condition

Based on the simulation results in section 2.1, it can be seen that the CSI-RSRP accuracy requirements, which was agreed to be same as existing SS-RSRP accuracy, can be met with timing offset at CP and -CP. Also, there is not much difference between 0.9*CP and 1*CP. Therefore, we suggest to define the upper bound of timing offset as 1*CP.

Proposal: For CSI-RSRP and CSI-RSRQ the upper bound of timing offset for case 1 is 1*CP.

Conclusions

In this paper we provided we will provide additional simulation results and our views on the timing offset condition for CSI-RSRP and CSI-RSRQ.

Proposal: For CSI-RSRP and CSI-RSRQ the upper bound of timing offset for case 1 is 1*CP.

Reference

- R4-2104089, WF on performance requirements of CSI-RS based L3 measurement, CATT/OPPO

- R4-2103725, Updated link-level simulation assumption for CSI-RS based L3 measurement, CATT