3GPP TSG-RAN WG4 Meeting#98bis-e	R4- 2107035

Online, 12th Apr – 20th Apr, 2021

Source: 	Qualcomm Incorporated

Title: 	Test Parameters for Application Layer Throughput Tests

Agenda item:	9.5.3

Document for:	Discussion

- Introduction

In RAN#90-e, RAN4 objectives were added to SID [1] to study the feasibility of defining requirements with link adaptation. In this paper, we provide our views on test parameters for simulation results.

- Test Parameters

Existing Rank Indication test cases in 38.101-4 have CQI, PMI and RI reporting enabled. As RAN4 is studying the feasibility of defining link adaptation test cases, it will be easier to reuse test parameters from rank indication tests as much as possible to minimize the work. Therefore, we propose the following.

Proposal 1: Evaluate link adaptation throughput requirements for following scenarios:

- FDD: 10MHz/15kHz

- FR1 TDD: 40MHz/30kHz

- FR2 TDD: 100MHz/120kHz

Proposal 2: Use existing rank indication test parameters as much as possible for studying the feasibility of defining NR link adaptation throughput requirements.

To minimize the RAN4 effort, we prefer to use the SNR points recommended by RAN5 in [2] as baseline. SNR points should be chosen such that it doesn’t fall in the rank transition regime. This will allow for better alignment across simulation results from different companies. Therefore, we propose the following.

Proposal 3: Test SNR points should be chosen such that it doesn’t fall in the rank transition regime.

Proposal 4: Use 20dB SNR for FR1 and 16dB SNR for FR2 as baseline for studying the feasibility of defining NR link adaptation throughput requirements.

As RAN4 is studying LA throughput requirements, it will be more practical to have less turnaround time in applying the CSI report. This will ensure that reported CSF metrics are still useful when they are getting applied. Aperiodic CSI reporting has lower processing time compared to periodic report. Therefore, we propose the following.

Proposal 5: Use aperiodic CSI reporting for studying the feasibility of defining NR link adaptation throughput requirements.

- Conclusions

This paper provides our views on test parameters for studying LA throughput requirements in RAN4. Following has been proposed.

Proposal 1: Evaluate link adaptation throughput requirements for following scenarios:

- FDD: 10MHz/15kHz

- FR1 TDD: 40MHz/30kHz

- FR2 TDD: 100MHz/120kHz

Proposal 2: Use existing rank indication test parameters as much as possible for studying the feasibility of defining NR link adaptation throughput requirements.

Proposal 3: Test SNR points should be chosen such that it doesn’t fall in the rank transition regime.

Proposal 4: Use 20dB SNR for FR1 and 16dB SNR for FR2 as baseline for studying the feasibility of defining NR link adaptation throughput requirements.

Proposal 5: Use aperiodic CSI reporting for studying the feasibility of defining NR link adaptation throughput requirements.

# References

[1] RP-202849, “Study on 5G NR UE Application Layer Data Throughput Performance”, 3GPP TSG RAN Meeting #90-e, Online, Dec 7 - Dec 11, 2020.

[2] R5-195422, “LS to RAN4 to update 38.101-4 with Link Adaptation requirements”, 3GPP TSG RAN WG5 Meeting #83, Reno, Nevada, United States, May 13 – May 17, 2019.