3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2107072

Electronic Meeting, 12 April – 20 April 2021

Source:	Ericsson, AT&T

Title:	TP for TR 37.717-41-11: Including band combinations DC_2A-29A-30A-66A_n66A

Agenda item:			7.6.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 37.717-31-11 to add  DC_2A-29A-30A-66A_n66A.

# 2. Text Proposal

---Start of changes---

### 5.1.X	DC_2-29-30-66_n66

#### 5.1.X.1	Configuration for EN-DC

Table 5.1.X.1-1: Band combinations EN-DC (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_2A-29A-30A-66A_n66A | DC_2A_n66A DC_30A_n66A DC_66A_n66A1 |
| NOTE 1: Only single switched UL is supported |  |

#### 5.1.X.2	∆TIB and ∆RIB values

Based on values for CA_2-29-30-66 in 36.101.

Table 5.1.X.2.-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-29-30-66_n66 | 2 | 0.5 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |

Table 5.1.X.2.-2: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_2-29-30-66_n66 | 2 | 0.4 |
|  | 29 | 0 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
|  | n66 | 0.4 |

#### 5.1.X.3	Reference sensitivity exceptions

Compared to its fallback modes, there are no additional MSD requirements for this band combination.

---End of changes---

# Reference

[1]		RP-202590, “Rel-17 WID on DC of 4 bands LTE inter-band CA (4DL1UL) and 1 NR band (1DL1U)” Nokia