3GPP TSG-RAN WG4 Meeting # 98-bis-e 	R4-2107079

# Electronic Meeting, Apr. 12-20, 2021

Source: 	vivo

Title: 	Discussion on RRM requirements for SRS antenna port switching

Agenda Item:	8.4.2.1

Document for:	Discussion

Introduction

In RAN4 98e, the WF R4-2103672 was agreed in [1]. In this paper our views on RRM requirements impacts for SRS antenna port switching are provided.

Discussion on the scope of SRS antenna switching requirements

In last meeting, the following was discussed.

- Issue 1-1-2: whether delay requirement would be defined in RRM for SRS antenna port switching

  - Agreements

    - RRM delay requirement for SRS antenna port switching is FFS

      - Option 1: Do not define SRS antenna port switching delay requirement in RRM.

      - Option 2: Define SRS antenna port switching delay requirement same as RF retuning time.

      - Option 3: Define SRS antenna port switching delay requirement. FFS for the value. At least RF retuning time shall be included.

On this issue, the current status of the spec is that

- In TS 38.214[2] clause 6.2.1.2, guard period is defined as “at least Y symbols”, “in which the UE does not transmit any other signal, in the case the SRS resources of a set are transmitted in the same slot. The guard period is in-between the SRS resources of the set”. Y=1 for 15/30/60 kHz SCS, and Y=2 for 120kHz SCS.

- In TS 38.101-1[3] clause 6.3.3, transient periods for SRS, and PUSCH-SRS time masks are defined. Actually the transient periods for the cases when antenna port is not switched, are extensively discussed in RF session in R16. UE capability was introduced to differentiate UE’s power adjustment performance, so that the impact to EVM and/or network scheduling is minimized. The final CR [4] was approved in last meeting. For the cases when antenna port is switched, there is no enhancement in R16 and hence the transient period is still 15us.

- In TS 38.101-2[5] clause 6.3.3, transient period for SRS antenna switching is not additionally specified for FR2. The transient period is neither discussed or enhanced in R16.

Based on above information, firstly it is clear that so far the concerned scenario for SRS antenna port switching, at least from the perspective of RAN4 spec impact, should be FR1. For FR2 there is no additional requirement for antenna switching. Based on RAN1 and RAN4 spec, especially in RF session discussion, the antenna switching time is considered as 15 us so far. Below a table is provided on the length of CP and OFDM symbol(incl. CP) for the various SCSs.

| SCS [kHz] | CP length [us] | OFDM symbol (incl. CP) [us] |
| --- | --- | --- |
| 15 | 4.7 ~ 5.2 | 71.3 ~ 71.8 |
| 30 | 2.3 ~ 2.9 | 35.7 ~ 36.2 |
| 60 (Normal) | 1.2 ~ 1.6 | 17.8 ~ 18.4 |
| 60 (Extended) | 4.2 | 20.8 |
| 120 | 0.6 ~ 1.1 | 8.9 ~ 9.4 |

Table 1  Length of CP and OFDM symbol(incl. CP) for the different SCSs

Table 1 can be regarded as one explanation why Y symbols gap is specified in 38.214. Comparing 15us to the various lengths above, especially for FR1, it is clear that 15us is a loose requirement. To accommodate such 15us switching time, a gap with 35~72 us is specified, considering 15kHz or 30kHz SCS for FR1 is most popular. Such gap would lead to scheduling restriction, which is possibly not needed for some higher capability UE. This is similar to what was discussed in RF session in R16 for the transient period.

On the other hand, so far the transient period enhancement in R16 is to specify a window for EVM relaxation. It means that if network is able to endure the EVM performance loss due to power adjustment, the scheduling restriction is not needed. However, SRS antenna port switching includes at least switching time, in which UL transmission is not possible. This is different to the transient period. Therefore, it is better to be specified as some delay requirements in RRM spec similarly to some other features, such as BWP switching delays.

Observation 1  So far the SRS antenna port switching feature only has impact on FR1 requirements.

Observation 2  RAN1 spec has only specified gaps between SRSs, while the transient period in RAN4 RF spec covers the case of potential separation between SRS and PUSCH/PUCCH.

Observation 3  Similar to what was discussed for transient periods in RF session in R16, 15us for SRS antenna switching delay can be a loose requirement for some higher capability UE.

Proposal 1  Specify SRS antenna port switching delay requirements in R17 for FR1.

Proposal 2  For SRS antenna port switching delay, RAN4 should consider to specify UE capability to differentiate the needed minimal separation between SRSs and/or between SRS-PUSCH/PUCCH, similar to the transient period capability defined in R16.

In last meeting, the following was discussed.

- Issue 1-1-3: Impact of SRS antenna port switching to other RRM requirements

  - Agreements

    - Further identify impact of SRS antenna port switching on RRM requirements, e.g.

      - Timing measurements and corresponding measurement requirements

      - Other RRM requirements

- Issue 1-1-4: RAN4 defines the requirement only for SRS antenna port switching in FR1 or in both FR1 and FR2

  - Agreements

    - Define the RRM requirements for SRS antenna port switching for FR1.

    - FFS for FR2 SRS antenna port switching requirements:

      - Further identify the applicability of the existing RF transient period for SRS antenna port switching.

In our understanding, such timing requirements for SRS antenna switching are mostly used for positioning. It is not clear why antenna switching needs to be considered for the positioning use cases. Therefore, we do not think timing measurements needs to be considered at least in R17.

Additionally, so far the RF transient period requirements do not differentiate the use case for SRS antenna switching in FR2. Moreover, in our view the use case for SRS antenna switching in FR2 is not clear so far, since T=R, 2T=2R are not typically considered as cases for antenna switching. Therefore, although the Y symbols gap is considered in RAN1 spec, it is recommended not to specify any requirements for FR2 in R17, unless if some clarification to the use case can be made.

Proposal 3  Do not consider impact to timing measurements in R17 SRS antenna port switching.

Proposal 4  Do not specify any requirements for FR2 in R17 SRS antenna port switching, unless if some clarification to the use cases can be made.

For SRS carrier switching, the collision case and the prioritization rule is already specified in TS 38.214 and TS 38.133. However, for SRS antenna port switching, no prioritization rule has been specified in RAN1 spec. In our understanding, the purpose of SRS antenna port switching is to provide DL CSI in DL-UL-reciprocal channel, and it should not be prioritized over some other procedures, e.g.

- SSB/CSI-RS for L1/L3 measurements

- PUSCH/PUCCH transmission with priority index 1 or DL pre-emption transmission

- PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/CRI/SSBRI and/or PRACH

- PUSCH transmission carrying aperiodic CSI (if periodic/semi-persistent SRS resources are configured)

Since the collision cases and prioritization rules are mostly capture in RAN1 spec, in our view LS to RAN1 can be triggered for this issue, to check RAN1’s understanding.

Proposal 5  Send LS to RAN1 to check the prioritization rule for SRS antenna switching, especially for the case in CA/DC operation.

Discussion on interruption requirement applicability

In last meeting, the following was discussed.

- Issue 1-2-1: Interruption requirement applicability

  - FFS:

    - Option 1 (MTK, Huawei, OPPO, NEC, Xiaomi): The interruption requirement should base on the band combination capability reporting by UE.

    - Option 1a: The interruption requirement should base on the band combination capability (indicated by txSwitchImpactToRx or txSwitchWithAnotherBand) reporting by UE.

    - Option 2 (Huawei (observation 1), Nokia (proposal 5), NEC, Apple, vivo, CATT, QC): For the interruption cause by the switching period, whether the victim CCs are same as those indicated by the IEs (txSwitchImpactToRx or txSwitchWithAnotherBand) needs further discussion.

    - Option 3 (Ericsson): Needs further checking and discussion.

- Issue 1-2-2: Victim cell type impacted by SRS antenna port switching

  - Agreement:

    - In the R17 FeRRM WI only RRM requirement of NR SRS antenna port switching would be discussed, including following cases (but all the victim CCs shall be determined by the conclusion in issue 1-2-1):

      - NR SRS antenna port switching impacting LTE CC

      - NR SRS antenna port switching impacting NR CC

- Issue 1-2-3: whether same interruption requirement applies to different SRS antenna port switching patterns

  - FFS:

    - Option 1 (vivo, OPPO, NEC, Apple, Xiaomi, Intel, CATT, QC): use same set of requirements for different SRS antenna switch patterns

    - Option 2 (LGE): Depending on UE capability supportedSRS-TxPortSwitch, the interruption requirements should be defined.

    - Option 3 (Ericsson, MTK, Nokia): further discussion is needed.

In R16 SRS carrier switching, per-FR gap capability was an important factor in defining interruption requirements, which infers that UE reuses the methodology of measurement gap in conducting interruptions due to SRS carrier switching. For R17, whether such methodology can be reused for SRS antenna port switching needs further discussion. At least for uplink there is no gap in the spec yet. In our view option 1a should be more reasonable for R17, i.e. the interruption requirements should be based on the band combination capability (indicated by txSwitchImpactToRx or txSwitchWithAnotherBand) reporting by UE. In other word, if UE indicates that in the corresponding band the Rx or Tx is impacted by antenna port switching, then only the corresponding band is allowed to be interrupted when UE is configured to switch SRS antenna port.

Proposal 6  The interruption requirements should be based on the band combination capability (indicated by txSwitchImpactToRx or txSwitchWithAnotherBand) reporting by UE.

Proposal 7  If UE indicates that in the corresponding band the Rx or Tx is impacted by antenna port switching, then only the corresponding band is allowed to be interrupted when UE is configured to switch SRS antenna port.

Another issue is that whether different interruption is needed for different SRS antenna port switching pattern. In our understanding, the pattern here means 1T4R or 2T4R or 1T2R. In our understanding the similar methodology for transient periods as 38.101-1 can be re-used. When defining antenna port interruption requirements, only the period when UE switches antenna port is considered, while referring to different switching pattern is not necessary.

Proposal 8  Do not refer antenna switching patterns in the spec when defining SRS antenna switching interruption requirements.

Discussion on interruption requirement design

In last meeting, the following was discussed.

- Issue 1-3-1: The interruption requirement is defined based on slot level or symbol level

  - FFS:

    - Option 1 (Apple, QC, MTK, Huawei, OPPO, Xiaomi, vivo, CATT, Intel): based on slot level

    - Option 2 (LGE, NEC, Huawei, Ericsson): based on symbol level

    - Option 3 (Nokia, CMCC): FFS

- Issue 1-3-2: The components within interruption time of SRS antenna port switching in FR1

  - FFS:

    - Option 1 (Xiaomi, vivo, Huawei, OPPO, Intel, CATT): includes antenna switching time and SRS transmission time

    - Option 1a (QC, NEC, LG, MTK): includes antenna switching time and SRS transmission time; and transient periods before and after SRS transmission slot have to be taken into consideration.

    - Option 1b (Ericsson, Xiaomi, NEC, LG): includes antenna switching time and SRS transmission time; and other components can be further discussed.

    - Option 2 (Ericsson): FFS on followings

      - FFS: time to switch to transmit,

      - FFS: time to switch back,

      - FFS: guard symbol(s),

      - FFS: SRS transmission.

    - Option 3 (Apple, Nokia): only antenna switching time

      - If the transient period (15us) can be captured within the guard period in FR1, the UL interruption at SRS antenna switching shall be defined based on the minimum guard period specified in RAN1

- Issue 1-3-5: Would the interruption requirement based on different SCS?

  - FFS:

    - Option 1 (Apple): Interruption requirement is based on the victim CC SCS.

    - Option 2 (QC, MTK, vivo, OPPO, LG, CATT): Interruption requirement is based on the aggressor CC and victim CC SCS.

    - Option 3 (Ericsson, Apple, Xiaomi, Nokia, Intel, CATT, Huawei):

      - The interruption requirement depends at least on the SCS of the victim cell.

      - FFS: the interruption requirement depends on the SCS of aggressor cell.

- Issue 1-3-6: Would the interruption requirement differentiate between sync and async cases?

  - FFS:

    - Option 1 (Apple, QC, MTK, OPPO, Xiaomi): No; based on the async case for the minimum requirement.

    - Option 2 (LGE, CATT, Ericsson, Nokia): Yes.

    - Option 3 (Huawei): depends on whether to have slot level interruption or symbol level interruption. Need FFS.

Regarding whether interruption requirement is defined based on slot level or symbol level, in our understanding, the TA and synchronization assumption needs to be considered firstly. The requirement should include the asynchronous case based on current WID description. Therefore, it would be very difficult to define requirement based on symbol level. Even if sync case is considered, the TA for UL may cause random mis-alignment of the frame boundary. Therefore, in our view it is more reasonable to be defined based on slot level. In this case, it is simpler if requirement for sync and async case are not differentiated.

Proposal 9  The interruption requirement is preferred to be defined based on slot level.

Proposal 10  The interruption requirement is preferred to be defined without differentiating sync and async case, at least in R17.

Since it is the interruption requirement discussed here, which is for other CCs, in our view if slot level interruption is considered, then there is no need to preclude the symbol for SRS transmission. At least for the case of intra-band UL CA it is necessary to interrupt the corresponding UL/DL transmission, if it is not prioritized according to some prioritization rules. In this case, it is simpler if interruption requirement is based on the aggressor CC and victim CC SCS.

Proposal 11  For interruption requirements, the interruption time is preferred to include antenna switching time and SRS transmission time.

Proposal 12  For interruption requirements, the interruption time is preferred to be based on the aggressor CC and victim CC SCS.

However, we see quite many network vendors and one operator have shown concern in defining interruption requirements in slot level. In our view, it would be very difficult to exchange UE synchronization information between different BS, especially for interband DC cases. Therefore, for these 4 issues, RAN4 should firstly study how network can obtain the interrupted symbol information, when SRS antenna port switching is performed in another band.

Proposal 13  RAN4 should firstly study whether and how network can obtain the interrupted symbol information, when SRS antenna port switching is performed in another band.

Conclusions

Based on above analysis, we have following observations and proposals.

Observation 1  So far the SRS antenna port switching feature only has impact on FR1 requirements.

Observation 2  RAN1 spec has only specified gaps between SRSs, while the transient period in RAN4 RF spec covers the case of potential separation between SRS and PUSCH/PUCCH.

Observation 3  Similar to what was discussed for transient periods in RF session in R16, 15us for SRS antenna switching delay can be a loose requirement for some higher capability UE.

Proposal 1  Specify SRS antenna port switching delay requirements in R17 for FR1.

Proposal 2  For SRS antenna port switching delay, RAN4 should consider to specify UE capability to differentiate the needed minimal separation between SRSs and/or between SRS-PUSCH/PUCCH, similar to the transient period capability defined in R16.

Proposal 3  Do not consider impact to timing measurements in R17 SRS antenna port switching.

Proposal 4  Do not specify any requirements for FR2 in R17 SRS antenna port switching, unless if some clarification to the use cases can be made.

Proposal 5  Send LS to RAN1 to check the prioritization rule for SRS antenna switching, especially for the case in CA/DC operation.

Proposal 6  The interruption requirements should be based on the band combination capability (indicated by txSwitchImpactToRx or txSwitchWithAnotherBand) reporting by UE.

Proposal 7  If UE indicates that in the corresponding band the Rx or Tx is impacted by antenna port switching, then only the corresponding band is allowed to be interrupted when UE is configured to switch SRS antenna port.

Proposal 8  Do not refer antenna switching patterns in the spec when defining SRS antenna switching interruption requirements.

Proposal 9  The interruption requirement is preferred to be defined based on slot level.

Proposal 10  The interruption requirement is preferred to be defined without differentiating sync and async case, at least in R17.

Proposal 11  For interruption requirements, the interruption time is preferred to include antenna switching time and SRS transmission time.

Proposal 12  For interruption requirements, the interruption time is preferred to be based on the aggressor CC and victim CC SCS.

Proposal 13  RAN4 should firstly study whether and how network can obtain the interrupted symbol information, when SRS antenna port switching is performed in another band.

- References

[1]  R4-2103672  WF on further RRM enhancement for NR and MR-DC - SRS antenna port switching, Apple.  RAN4 #98e

[2]  TS 38.214  v16.4.0

[3]  TS 38.101-1  v16.6.0

[4]  R4-2103152  CR on introduction of shorter Transient Period Capability, Qualcomm Incorporated , Huawei, HiSilicon, Skyworks, Ericsson, Nokia, Anritsu, CMCC.  RAN4 #98e

[5]  TS 38.101-2  v16.6.0

Annex

Title:	[DRAFT]LS on prioritization rule on SRS antenna port switching in CA/DC

Response to:

Release:	Rel-17

Work Item:	NR_RRM_enh2-Core

Source:	RAN WG4

To:	RAN WG1

Cc:

Contact Person:

Name:	Yanliang SUN

E-mail Address:	 yanliang.sun@vivo.com

Attachments:

1. Overall Description:

In RAN4 #98-bis-e meeting, RAN4 was discussing the RRM requirements for SRS antenna port switching and RAN4 has identified questions about prioritization rules on SRS antenna port switching in CA/DC scenarios. RAN4 sincere requests RAN1 to provide answers of following questions for RAN4 future work.

For SRS carrier switching, the collision cases and the prioritization rules are already specified in TS 38.214. However, for SRS antenna port switching, no prioritization rule has been specified in RAN1 spec for the CA/DC scenarios. RAN4 respectfully ask RAN1 that for CA/DC scenarios, whether SRS transmission for antenna port switching in one of the active serving cell can be prioritized over the following transmissions/receptions on any other active serving cells

•	SSB/CSI-RS for L1/L3 measurements

•	PUSCH/PUCCH transmission with priority index 1 or DL pre-emption transmission

•	PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/CRI/SSBRI and/or PRACH

•	PUSCH transmission carrying aperiodic CSI (if periodic/semi-persistent SRS resources are configured)

2. To RAN WG1 group.

ACTION: 	RAN4 respectfully asks RAN1 to provide feedback on these issues.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN4 Meeting #99e	19-27, May, 2021	Online

TSG-RAN4 Meeting #99-e	23 - 27 Aug 2021	Online