3GPP TSG-RAN WG4 Meeting # 98-bis-e 	R4-2107085

# Electronic Meeting, Apr. 12-20, 2021

Source: 	vivo

Title: 	Updated evaluation assumptions for R17 RLM/BFD relaxation

Agenda Item:	8.9.2

Document for:	Approval

Introduction

This contribution provides updated evaluation assumptions for R17 RLM/BFD relaxation

System level simulation assumptions for mobility impact analysis from RLM/BFD relaxation

The intention of this evaluation is to justify the mobility impact of RLM/BFD relaxation under different scenarios. For FR1 and FR2, the evaluation assumption is provided in Table I.

Table I Evaluation assumptions for mobility impact of RLM/BFD relaxation in FR1 and FR2

| Parameters | Dense urban (UMi) in FR1 | Dense urban (UMi) in FR2 |
| --- | --- | --- |
| UE speed | 3km/h; 30km/h Companies are encouraged to further consider UE rotation in FR2. |  |
| Relaxation factor (X) | 2, [3], 4, [8] Note: For DRX cycle <=320 ms, “1.5” factor is considered in the R15/R16 baseline. | 2, [3], 4, [8] Note: For DRX cycle <=320 ms, “1.5” factor is considered in the R15/R16 baseline. |
| DRX cycle length | 20ms, 40ms |  |
| Layout | Single layer: Macro layer: Hex. Grid |  |
| Moving across Boarder | Option 1: Wrap around as defined in 36.839 [1] Option 2: Bouncing back as defined in 36.839 [1] |  |
| Inter-BS distance | 200m | 200m |
| Carrier frequency and SCS | 4GHz and 30kHz SCS for SSB | 30GHz and 120kHz SCS for SSB |
| Simulation bandwidth | 20MHz (for CORESET0 BW) | 80MHz (for CORESET0 BW) |
| N factor (# of RX beams for FR2) | N.A. | 8 for SSB, 1 for CSI-RS |
| SSB multiplex with Data | No |  |
| LoS probability | According to Table 7.4.2-1 in TR 38.901 [2] |  |
| Pathloss and shadow fading std. | Refer to UMi model in TR38.901 [2] |  |
| Shadow fading correlation distance | 10 for LOS and 13 for NLOS |  |
| BS Tx power (PA power) | 44dBm | 40dBm |
| UE receiver noise figure | 9dB | 13dB |
| UE distribution | 100% outdoor |  |
| BS antenna height | 10m |  |
| UE antenna height | 1.5m |  |
| BS antenna element gain + connector loss | Follow 38.802 [3] Table A.2.1-4 |  |
| UE antenna model | Follow 38.802 [3] Table A.2.1-4 | Follow 38.802 [3] Table A.2.1-4 (M,N,P,Mg,Ng) = Option 1: (1,2,2,1,2) Option 2: (2,2,2,1,2) Θmg,ng=90°; Ω0,1=Ω0,0+180°; (dg,H, dg,V)=(0,0) Note: only one of panels should be selected |
| UE beam sweeping | No | Yes. # of beams: N = 8 |
| BS antenna down-tilt | 15 degree |  |
| BS array configuration | (M,N,P,Mg,Ng) = Option 1: (8,8,2,1,1) Option 2: (2,4,2,1,1) (dH,dV) = Option 1: (0.5, 0.8)λ Option 2: (0.5, 0.9)λ +45°, -45° polarization | (M,N,P,Mg,Ng) = Option 1: (4,8,2,1,1) Option 2: (4,8,2,2,2) (dH,dV) = (0.5, 0.5)λ, (dg,H,dg,V) = (4.0, 2.0)λ +45°, -45° polarization |
| BS beam set | For direction of analog beam steering (in LCS): Azimuth angle φi = [-45, -15, 15, 45] degree Zenith angle θj = [90,115] degree Note: Other options are not precluded. | For direction of analog beam steering (in LCS): Azimuth angle φi = [-60, -42.86, -25.71, -8.57, 8.57, 25.71, 42.86, 60] degree Zenith angle θj = [90, 100, 110, 120] degree Note: Other options are not precluded. |
| T310 | 1000ms Note: Other options, e.g. 2000ms, are not precluded. | 1000ms Note: Other options, e.g. 2000ms, are not precluded. |
| N310 | 1 Note: Other options are not precluded. | 1 Note: Other options are not precluded. |
| N311 | 1 Note: Other options are not precluded. | 1 Note: Other options are not precluded. |
| Qout/Qin | SINR for hypothetical PDCCH BLER in Table 8.1.1-1 of TS 38.133 Companies are encouraged to provide specific values used in evaluations | SINR for hypothetical PDCCH BLER in Table 8.1.1-1 of TS 38.133 Companies are encouraged to provide specific values used in evaluations |
| beamFailureInstanceMaxCount | n1 Note: Other options are not precluded. | n1 Note: Other options are not precluded. |
| beamFailureDetectionTimer | pbfd4 Note: Other options are not precluded. | pbfd4 Note: Other options are not precluded. |
| SNR of set q0 | -6dB | -6dB |

The UE performs signal and interference measurements based on RLM-RS (i.e. SSB or CSI-RS) to derive a downlink link quality metric, and SINR is considered here for simplicity.

The performance metric of this evaluation can be

- increased latency in RLF triggering (for RLM), and the baseline latency is the RLF triggering latency in R15.

- increased latency in beam failure detection and the initiation of beam recovery procedure (for BFD), and the baseline latency is the beam failure triggering latency in R15.

- the CDF curves of Delta SINR comparison between R15 an R17.

- ΔSINR = MAX (ABS [(estimated SINR at Rel-17– estimated SINR at Rel-15) CDF=X ] ), ABS [(estimated SINR at Rel-17 – estimated SINR at REl-15) CDF=Y ]) [dB]

  - (X,Y) =(5%, 95%) or (1%, 99%)

  - Note: Other options to (X,Y) are not precluded. Companies are encouraged to bring CDF curves of ΔSINR in the next meeting.

As one reference, the definition of Delta SINR in R15 is

•	Delta SINR = (estimated SINR – ideal SINR) 	[dB].

Note that estimated SINR can be obtained based on

•	Ideal L1 measurement with L1 filtering, i.e. L1 one-shot estimation error is NOT considered, or

•	Realistic L1 measurement with L1 filtering, i.e. L1 one-shot estimation error is considered.

Evaluation assumptions for power saving benefits from RLM/BFD relaxation

This evaluation is going to provide power consumption comparison between relaxed RLM/BFD and R15/R16 baseline. The evaluation is based on power model in annex of TR 38.840 [4] and some further evaluation assumptions are provided in Table II as follows.

Table II Evaluation assumptions for power saving gain of RLM/BFD relaxation

| Parameters | FR1 | FR2 |
| --- | --- | --- |
| longDRXCycle | 40ms | 40ms |
| OnDurationTimer | 4ms | 4ms |
| Drx_InactiveTimer | 4ms | 4ms |
| Slot length | 0.5ms | 0.125ms |
| Relaxation factor (X) | 2, [3], 4, [8] Note: For DRX cycle <=320 ms, “1.5” factor is considered in the R15/R16 baseline. | 2, [3], 4, [8] Note: For DRX cycle <=320 ms, “1.5” factor is considered in the R15/R16 baseline. |
| PDCCH-WUS configuration | Option 1: Yes WUS offset: 4 slot No wake-up if WUS is missed Option 2: No | Option 1: Yes WUS offset: 4 slot No wake-up if WUS is missed Option 2: No |
| Traffic Model | Option 1: FTP Model 3 0.1Mbytes packet size Option 1a: 200ms mean inter-arrival time Option 1b: 50ms mean inter-arrival time (Note: Companies are encouraged to evaluate both options) Option 2: VoIP traffic models in TR 38.840 | Option 1: FTP Model 3 0.1Mbytes packet size Option 1a: 200ms mean inter-arrival time Option 1b: 50ms mean inter-arrival time (Note: Companies are encouraged to evaluate both options) Option 2: VoIP traffic models in TR 38.840 |
| RLM-RS periodicity | SSB: 20 ms CSI-RS: TBD | SSB: 20 ms CSI-RS: TBD |
| RRM-RS periodicity | SSB(SMTC periodicity): 20ms Note: Other options are not precluded. | SSB(SMTC periodicity): 20ms Note: Other options are not precluded. |
| SMTC window length | Option 1: 5ms Note: Other options are not precluded. | Option 1: 5ms Note: Other options are not precluded. |
| UL transmission | Interested companies are encouraged to consider this and claim the detailed assumptions when delivering results. |  |
| BW of RLM/BFD-RSs | Interested companies are encouraged to consider this and claim the detailed assumptions when delivering results. |  |
| RS for L1 measurements | Interested companies are encouraged to consider this and claim the detailed assumptions when delivering results. |  |
| PDCCH monitoring within DRX onduration | UE monitors PDCCH on a per-slot basis during DRX on-duration. Note: Other options are not precluded. Companies are encouraged to bring detailed assumptions if above option is not considered. |  |
| UE behaviour assumptions for RRM measurement when RLM measurement interval is extended K times | Consider the following options for UE behaviour as baseline: Option 1: When RLM measurement interval is extended K times, RRM measurement interval is also extended K times. Option 2: When RLM measurement interval is extended K times, RRM measurement interval is not extended from the Rel-15 assumption. Note 1: It is assumed that UE can still meet R15 RRM measurement period and accuracy requirements in the above UE behaviour assumptions. How UE can meet these requirements will be further discussed in future meetings. Note 2: Other options are not precluded. Companies are encouraged to bring detailed assumptions if above options are not considered. |  |

The performance metric of this evaluation can be:

- Per-slot average power level, including both R15 an R17

- Per-slot average power level comparison between R15 an R17

- ΔPower = (average power level at Rel-15) – (average power level at Rel-17)

- Gain =ΔPower / (average power level at Rel-15)

Link level evaluation assumptions for one-shot SINR estimation error

This evaluation is going to provide the one-shot SINR estimation error, especially in high SINR regions, so that the assumed margin between relaxation thresholds and Qout can be identified.

The baseline assumption for this simulation is in [6].

Only SS-SINR are evaluated. Details can be found in the tables below

Table III: General parameters

| Simulation parameters | Comments/values |  |
| --- | --- | --- |
| Carrier frequency for Cell 1 and Cell 2 | 4 GHz | 30 GHz |
| System bandwidth | 5 MHz; 10 MHz | 50 MHz; 100 MHz |
| Prior knowledge of Cell 1 / Cell 2 by the UE | No / Yes | No / Yes |
| DRX | No | No |
| BS transmit antennas for SS blocks | 1 tx or single layer transmissions | 1 tx or single layer transmissions |
| UE receive antennas | 2 rx (uncorrelated with equal gain, no rx beamforming); | Case A: 2 rx (uncorrelated with equal gain, no rx beamforming); Case B: 2 rx (with rx beamforming Note1) |
| Data and control channel subcarrier spacing | The same as SS block subcarrier spacing | The same as SS block subcarrier spacing |
| Measurement period (in number of measurement samples) | 5 | 5 |
| SS burst set configuration in Cell 1 and Cell 2: |  |  |
| - Subcarrier spacing | 15 kHz; 30 kHz | 120 kHz; 240 kHz |
| - Number of SS blocks per SS burst set | 1 | 1 |
| - SS burst periodicity | 20 ms | 20 ms |
| - Number of transmit antenna ports | 1 (the same port for NR-SSS, NR-PSS, NR-PBCH) | 1 (the same port for NR-SSS, NR-PSS, NR-PBCH) |
| - Actual SS block transmissions | always transmitted | always transmitted |
| NOTE1: For Case B, companies are encouraged to state their assumptions on beamforming and rx antenna configuration together with the provided simulation results |  |  |

Table IV: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| NR-PSS and NR-SSS sequences | - | To be indicated by companies | To be indicated by companies |
| PBCH and DMRS power offset with respect to NR-PSS and NR-SSS | dB | 0 | 0 |
| Data and control PSD relative to NR-PSS and NR-SSS | dB | 0 | 0 |
| RB Utilization | % | 100 | 100 |
| Data Modulation | - | QPSK | QPSK |
| Slot length | - | 14 symbols | 14 symbols |
| CP Length | - | Normal | Normal |
| Frequency Offset relative to UE frequency reference | Hz | 0 | 0 |
| 1)Relative Delay of 1st Path (synchronous) | µs | 0 | CP/2 |
| 2) Relative Delay of 1st Path (asynchronous): Fixed delay | Ms | 0 | 3 ms |
| Es/IoT (SNR) | dB | Companies claims the detailed assumption for SNR region |  |
| Propagation conditions | - | AWGN |  |

- References

[1]  TR 36.839	v11.1.0

[2]  TR 38.901	v15.1.0

[3]  TR 38.802	v14.2.0

[4]  TR 38.840	v16.0.0

[5]  TR 37.910	v16.1.0

[6]  R4-1806551  Link-level simulation assumptions for SSB-based measurement accuracy, Ericsson.  RAN4 #87.