3GPP TSG-RAN WG4 Meeting # 98-bis-e 	R4-217088

# Electronic Meeting, Apr. 12-20, 2021

Agenda Item:	5.1.2.3

Source:	MediaTek Inc.

Title:	Discussion on Scell activation requirement in NR-U

Document for:		Discussion

- Introduction

The remaining issues of SCell activation in the WF [1] are addressed in this paper.

- Applicability of SCell activation requirements when sCellDeactivationTimer is NOT configured

In the last meeting, the following options for applicability of SCell activation requirements when sCellDeactivationTimer is NOT configured are discussed [1].

Applicability of SCell activation requirements when sCellDeactivationTimer is NOT configured

- Option 1 (E///, QC, Apple, Huawei/HiSilicon):

  - The SCell activation requirements for NR-U do not apply when the sCellDeactivationTimer is not configured, when the SCell activation delay exceeds some pre-defined time (e.g., equivalent or comparable to the longest possible value of sCellDeactivationTimer).

- Option 2 (Nokia, ZTE, MTK):

  - SCell activation delay requirements are applicable in Scenario A (CA with NR PCell and NR SCell) with any LBT type and in Scenario B and C (E-UTRAN-NR-U DC/SA NR-U) with LBT type 2C. Requirements are also applicable in all scenarios, if the UE does not experience any UL LBT failures during SCell activation/deactivation.

  - SCell activation delay requirements are applicable when sCellDeactivationTimer is not configured also in Scenarios B and C (EN-DC and SA) LBT types other than 2C.

- Option 3 (possible compromise solution):

  - SCell activation delay requirements are applicable in Scenario A (CA with NR PCell and NR SCell) with any LBT type and in Scenario B and C (E-UTRAN-NR-U DC/SA NR-U) with LBT type 2C. Requirements are also applicable in all scenarios, if the UE does not experience any UL LBT failures during SCell activation/deactivation.

  - For all other scenarios the SCell activation requirements for NR-U do not apply when the sCellDeactivationTimer is not configured, when the SCell activation delay exceeds some pre-defined time (e.g., equivalent or comparable to the longest possible value of sCellDeactivationTimer).

Option 3 can be a compromise and it can still solve the issue of never-end Scell activation. However, if it is conditioning on the delay exceeds the longest possible value of sCellDeactivationTimer, then it can just apply a predefined value and there is no need to configure the sCellDeactivationTimer. Thus, a bit revision on Option 3 is provided.

Proposal 1: Revised from the Option 3.

- SCell activation delay requirements are applicable in Scenario A (CA with NR PCell and NR SCell) with any LBT type and in Scenario B and C (E-UTRAN-NR-U DC/SA NR-U) with LBT type 2C. Requirements are also applicable in all scenarios, if the UE does not experience any UL LBT failures during SCell activation/deactivation. (from Option 2)

- For all other scenarios the SCell activation requirements for NR-U do not apply when the SCell activation delay exceeds some pre-defined time (e.g., equivalent or comparable to the longest possible value of sCellDeactivationTimer). (from Option 1)

- Summary

In this paper, the Scell activation requirements in NR-U have been discussed. It proposes

Proposal 1: Revised from the Option 3.

- SCell activation delay requirements are applicable in Scenario A (CA with NR PCell and NR SCell) with any LBT type and in Scenario B and C (E-UTRAN-NR-U DC/SA NR-U) with LBT type 2C. Requirements are also applicable in all scenarios, if the UE does not experience any UL LBT failures during SCell activation/deactivation. (from Option 2)

- For all other scenarios the SCell activation requirements for NR-U do not apply when the SCell activation delay exceeds some pre-defined time (e.g., equivalent or comparable to the longest possible value of sCellDeactivationTimer). (from Option 1)

References

[1] R4-2103025, WF on NR-U RRM Core Requirements, Ericsson.