3GPP TSG RAN WG4 Meeting #98e-bis	R4-2107113

Online, 12th – 20th April 2021

Source:		Rohde & Schwarz

Title:	Discussion on FR1 UL MIMO EVM measurements

Agenda Item:		5.8.1.2

Document for:	Approval

# Introduction

In the last meetings the principle of a MIMO receiver for UL EVM measurements was proposed in several documents [1], [2], [3]. In this paper we provide further details on how the UL MIMO EVM measurements should be performed for FR1.

# Discussion

## Background

Calculation of EVM for single layer transmission without receive diversity is stable and described in Annex E: Global In-Channel TX-Test of TS 38.521-1 [4]. In this document a method of EVM calculation for UL MIMO with multi-layer transmission is described. For simplicity the document describes calculation of EVM using a maximum of 2 layers and 2 receive ports in TE for the multi-layer case. Same procedure is applicable also for transmission with 4 layers received on 4 TE ports.

## EVM calculation method for UL MIMO

The EVM calculation method for 2-layer transmission is shown in Figure 2.2-1.

Figure 2.2-1: EVM calculation block diagram for 2-Layer UL MIMO

The TE receives signals from 2 different ports which are connected to the two uplink antenna connectors on the UE.

Compared to single layer transmission an additional step for MIMO equalization is introduced to separate the 2 layers.

Once the layers are separated, each layer is processed using already defined single layer procedure. Basic measurement unit for EVM calculation is 1 slot comprising up to 14 OFDM symbols including reference and data symbols.

### MIMO Equalization

Each MIMO system needs an appropriate design of reference signals to ensure proper estimation of MIMO channel coefficients. Same is valid for NR where either reference signals use different subcarriers (CDM groups) per layer, or if same subcarriers are used orthogonal codes wf and wt are applied to reference signals of different layers. Since this essential property doesn’t apply for data symbols the channel estimation for MIMO equalization is based only on reference signals (DMRS) without including any data symbols. One or all DMRS symbols of one slot can be considered, but in order to obtain comparable EVM results independent of number of DMRS symbols per slot, only first DMRS symbol is used.

Estimation of effective 2x2 channel matrix is well known procedure if reference signals use different subcarriers, e.g. in case of DMRS antenna ports 0 and 2. In case that same subcarriers are used, e.g. DMRS antenna ports 0 and 1, a channel decomposition is necessary taking advantage of the orthogonal codes wf and wt and assuming identical channel coefficients for adjacent subcarriers of same CDM group.

Effective channel including the precoding matrix P is:

H=HP= h0,0h0,1h1,0h1,1

with

hn,ν=ynrν*|rν|2

where y denotes the received symbol on port index n and r the reference signal for layer index ν.

Since reference signals of a specific layer are transmitted only on subcarriers of one CDM group channel interpolation is needed in order to obtain channel coefficients for all subcarriers. Channel interpolation is done using the channel coefficients of active CDM group in all other CDM groups.

The channel coefficients used to calculate the equalizer coefficients are obtained after channel smoothing in frequency domain by computing the moving average of interpolated channel coefficients. The moving average window size is 7. For subcarriers at or near the edge of allocation the window size is reduced accordingly.

The ZF equalizer coefficients are calculated as pseudo inverse of effective channel matrix, in general:

GZF=H+=(HHH)-1HH

### Layer processing

Each layer is then processed using already defined single layer procedure in Annex E of TS 38.521-1 [4]. Since MIMO (or maximum ratio combining in case of single layer) channel estimation is calculated only on first DMRS symbol an averaging including all 14 symbols of one slot, i.e. data and reference signals, is suitable in order to minimize EVM. This averaging is achieved by the LS equalization method described for single layer in [3] section E.3.3 Post FFT equalization.

MS(f,t) and NS(f,t) are processed with a least square (LS) estimator, to derive one equalizer coefficient per time slot and per allocated subcarrier. EC(f) is defined for each layer as:

ECν(f)=t=013NSν(f,t)*NSν(f,t)t=013MSν(f,t)*NSν(f,t)

With * denoting complex conjugation. EC(f) are used to equalize layer data symbols.

EVM equalizer spectral flatness is derived from equalizer coefficients for each layer as follows:

cν=ECν(f) gν,02+gν,12

## Summary

In this paper we present a method to measure the EVM for UL MIMO transmissions in FR1 on a per layer basis. The approach is well aligned with the proposal for Tx diversity [5] and for FR2 [6] presented in this meeting.

Compared to the approach proposed in the last meeting in [2], the approach in this paper has several advantages, e.g. it will work even if only a single symbol is transmitted, it is fully independent of the data transmitted and has a reduced complexity when being implemented.

Therefor we propose to adapt the above described approach for FR1 UL MIMO EVM measurements.

Proposal: RAN4 agrees on the presented approach for FR1 UL MIMO EVM measurements.

# Proposals

The following proposals are made:

Proposal: RAN4 agrees on the presented approach for FR1 UL MIMO EVM measurements.

# References

- R4-2016569, ‘EVM Measurement for 2-Layer Uplink MIMO’, Lenovo, Motorola Mobility, RAN4#97-e, Nov 2020

- R4-2102661, ‘On FR1 2L UL EVM Requirement’, Qualcomm et al, RAN4#98-e, Jan-Feb 2021

- R4-2102383, ‘Discussion on FR1 EVM measurements’, Rohde & Schwarz, RAN4#98-e, Jan-Feb 2021

- 3GPP TS 38.521-1 v16.6.0

- R4-2107112, ‘Discussion on FR1 Tx Diversity EVM measurements’, Rohde & Schwarz, RAN4#98e-bis, April 2021

- R4-2107111, ‘Text proposal to TR38.884: FR2 UL EVM measurements’, Rohde & Schwarz, RAN4#98e-bis, April 2021