3GPP TSG-RAN WG4 Meeting #98-Bis-e                 	R4-2107125

Electronic Meeting, 12nd – 20th Apr., 2021

Agenda item:		12.1

Source:	MediaTek Inc.

Title:	Discussion on temporary RS

Document for:		Discussion

1	Introduction

In RAN4 #98e meeting, an LS was sent to reply the RAN1 questions on temporary RS for efficient SCell activation in NR CA [1]. Companies had reached consensus on activation time improvement for the cases of known cell. However, for the cases of unknown cell, further discussions are needed:

Reply to Q1: So far there is no conclusion on whether/how much benefit can be achieved for the temporary RS based SCell activation in other scenarios (e.g. SCell to be activated is unknown and belongs to FR1, SCell to be activated is unknown and belongs to FR2 if there is no active serving cell on that FR2 band). RAN4 will continue the discussion and provide feedback to RAN1 if there is conclusion.

Based on RAN1 agreement in #102e meeting, TRS is selected as temporary RS to expedite the activation procedure of AGC setting and time/frequency tracking because the target to be considered is the known cell. If more functionalities are confirmed to be supported by temporary RS, other RS candidates are not precluded.

Working Assumption At least for the case of known cell, temporary RS is supported to expedite the activation process during the SCell activation procedure for efficient SCell activation for both FR1 and FR2: - The temporary RS should provide at least the functionalities of AGC settling and time/frequency tracking during SCell activation procedure. - FFS potential functionalities of CSI measurement/acquisition and cell search Agreements: TRS is selected as temporary RS for Scell activation - If more functionalities are confirmed to be supported by temporary RS, other RS candidates, e.g. aperiodic CSI-RS, P/SP-CSI RS, SRS and RS based on SSS/PSS, are not precluded. - The TRS should be triggered by DCI or MAC-CE. FFS which exact triggering command.

As specified in TS38.133, for the cases of unknown cell, especially in FR 2, the cell search time might be the dominant term of the overall SCell activation time Tactivation_time. In this paper, we analysis how much improvement in activation time can be obtained, if SSS/PSS based temporary RS can also be provided to expedite the cell search procedure.

2	SCell activation delay requirement in FR1 and FR2

The requirement of SCell activation delay for the case of unknown cell in FR1 is shown below:

If the SCell is unknown and belongs to FR1, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, Tactivation_time is: - TFirstSSB_MAX + TSMTC_MAX + 2*Trs + 5ms.

In Table 1, we take 15kHz as an example and assume that SMTC periodicity is 20ms, timing distance between 2 burst of TRS is 5ms. The improved activation delay requirement between w options are compared. Here, the option 1 is when only TRS is provided, and option 2 is when both TRS and SSS/PSS based RS are provided. We also assume the timing gap between last SSB and first TRS that UE used to perform measurement is 5ms, and the periodicity of SSS/PSS based RS is also 5ms.

Table 1: SCell activation delay for unknown cell in FR1

|  | Only TRS is provided |  | Both TRS and SSS/PSS based RS are provided |  | Rel-15 requirement Tactivation_time (ms) |
| --- | --- | --- | --- | --- | --- |
|  | Tactivation_time (ms) | improved percentage | Tactivation_time (ms) | improved percentage |  |
| Unknown cell in FR1 | 75 | 11.76% | 25 | 70.59% | 85 |

Observation 1: For the case of unknown cell in FR1, approximately 11.76% SCell activation delay can be reduced if only TRS is provided as the temporary RS, and 70.59% SCell activation delay can be reduced if both TRS and SSS/PSS based RS are provided as the temporary RS

The requirement of SCell activation delay for the case of unknown cell in FR2 are shown below:

If the PCell/PSCell and the target SCell are configured as FR1-FR2 CA or if the PCell/PSCell and the target SCell are in a FR2 band pair with independent beam management, and the target SCell is unknown to UE and semi-persistent CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is: - 6ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs + TL1-RSRP, measure + TL1-RSRP, report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP). If the PCell/PSCell and the target SCell are configured as FR1-FR2 CA or if the PCell/PSCell and the target SCell are in a FR2 band pair with independent beam management, and the target SCell is unknown to UE and periodic CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is: - 3ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs + TL1-RSRP, measure + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)}.

InTable 2, we assume that SMTC periodicity is 20ms and timing distance between 2 burst of TRS is 5ms. The improved activation delay requirement between 2 options are compared.  Here, we also assume the timing gap between last SSB and first TRS that UE use to perform measurement is 5ms, the periodicity of SSS/PSS based RS is 5ms, and THARQ, TL1-RSRP, report,Tuncertainty_MAC, Tuncertainty_RRC, Tuncertainty_SP can be ignored.

Table 2: SCell activation delay for unknown cell in FR2

|  | Only TRS is provided |  | Both TRS and SSS/PSS based RS are provided |  | Rel-15 requirement Tactivation_time (ms) |
| --- | --- | --- | --- | --- | --- |
|  | Tactivation_time (ms) | improved percentage | Tactivation_time (ms) | improved percentage |  |
| SCell is unknown to UE and semi-persistent CSI-RS is used | 518 | 1.89% | 143 | 72.92% | 528 |
| SCell is unknown to UE and periodic CSI-RS is used | 518 | 1.89% | 143 | 72.92% | 528 |

Observation 2: For the case of unknown cell in FR2, approximately 1.89% SCell activation delay can be reduced if only TRS is provided as the temporary RS, and 72.92% SCell activation delay can be reduced if both TRS and SSS/PSS based RS are provided as the temporary RS

Considering that the improvement will be very limited if only TRS is in use as the temporary RS, we suggest RAN4 to inform RAN1 the evaluation results for unknown cell cases and suggest RAN1 to further study the possibility of introducing other RS type to expedite the activation process when cell search procedure is needed.

Proposal 1: RAN4 to inform RAN1 the evaluation results for unknown cell cases and suggest RAN1 to further study the possibility of introducing other RS type to expedite the activation process when cell search procedure is needed.

7	Summary

In this contribution, we have the following observations:

Observation 1: For the case of unknown cell in FR1, approximately 11.76% SCell activation delay can be reduced if only TRS is provided as the temporary RS, and 70.59% SCell activation delay can be reduced if both TRS and SSS/PSS based RS are provided as the temporary RS

Observation 2: For the case of unknown cell in FR2, approximately 1.89% SCell activation delay can be reduced if only TRS is provided as the temporary RS, and 72.92% SCell activation delay can be reduced if both TRS and SSS/PSS based RS are provided as the temporary RS

And we propose

Proposal 1: RAN4 to inform RAN1 the evaluation results for unknown cell cases and suggest RAN1 to further study the possibility of introducing other RS type to expedite the activation process when cell search procedure is needed.

Reference

- R4-2104067, “Reply LS on temporary RS for efficient SCell activation in NR CA,” Huawei, Jan., 2021.

- R4-2104083, “WF on temporary RS for efficient SCell activation in NR CA,” Huawei, Jan., 2021.