3GPP TSG RAN WG4 Meeting #98bis-e	R4-2107134

Electronic Meeting, April 12-20, 2021

Agenda Item:	5.3.3.1

Source: 	Ericsson

Title:	Analysis of conditions for IAB-MT RRM test cases

Document for:	Discussion

- Introduction

The following IAB-MT core requirements are definef for certain side conditions: SSB Ês/Iot and minimum SSB_RP.

- IAB MT Re-establishment delay requirement (section 12.1.1.1.2.1)

- RRC connection release with redirection to NR (section 12.1.1.3.2.1)

Currently the side conditions in the above core requirements are referred to the UE side conditions in the annex B of TS 38.133. However, side conditions specific to IAB-MT are needed in TS 38.174. This is analyzed in this contribution.

# Side conditions for IAB-MT

There are two main side conditions for which the requirements apply: SSB Ês/Iot and minimum SSB_RP

## SSB Ês/Iot

The SSB Ês/Iot side conditions for RRC re-establisment and RRC connection release with redirection requirements for IAB-MT are shown in table 1. They are the same as used for the same UE procedures. The side conditions are applicable for both local area and wide area IAB-MT classes and also for all IAB types (1-H, 1-O and 2-O).

Table 1: Applicable SSB Ês/Iot for RRC re-establisment and RRC release requirements for IAB-MT

| Section | Requirements | SSB Ês/Iot (dB) |
| --- | --- | --- |
| 12.1.1.1.2.1 | RRC connection re-establishment to NR intra-frequency cell | -6 |
| 12.1.1.1.2.1 | RRC connection re-establishment to NR inter-frequency cell | -4 |
| 12.1.1.3.2.1 | RRC connection release with redirection to NR | -4 |

## Minimum SSB_RP

The minimum SSB_RP is derived from the reference sensitivity (REFSENS). This also applied for IAB-MT. However, the principe for specifying the REFSENS for IAB-MT is fundamentally different than that used for the UE REFSENS. The IAB-MT REFSENS has following characteristics which are different than those related to the UE REFSENS:

- IAB-MT REFSENS depends on IAB type: different for 1-H, 1-O and 2-O

- IAB-MT REFSENS depends on IAB-MT class: different for LA and WA classes

- IAB-MT REFSENS does not depend on the frequency band

- IAB-MT REFSENS for IAB types 1-O and 2-O (OTA REFSENS) include a declarable parameters (ΔOTAREFSENS) which is the difference between conducted reference sensitivity and OTA REFSENS and include the antenna gain for different directions.

However, REFSENS for both UE and IAB-MT depend on channel bandwidth.

Based on the above analysis the SSB_RP side conditions based on the UE RRM in 38.133 cannot be reused for IAB-MT. We therefore need to define SSB_RP side conditions for IAB-MT based on the IAB-MT REFSENS and associated principle.

The IAB-MT REFSENS is derived to achieve throughput ≥ 95% of the maximum throughput of the reference measurement channel assuming certain SNR over certain channel bandwidth. However, the minimum SSB_RP is expressed on per SCS basis and is defined for certain SSB Ês/Iot. The IAB-MT minimum SSB_RP can therefore be derived from the IAB-MT REFSENS as described below:

### Minimum SSB_RP for IAB type 1-H

For IAB type 1-H, the minimum SSB_RP can be expressed by the following expression:

- Minimum SSB_RP = PREFSENS -10*Log10(NPRB *12) – SNRP_REFSENS + SSB Ês/Iot

Where:

- PREFSENS is the received sensitivity power level for certain channel bandwidth for WA IAB-MT class and LA IAB-MT class defined in Table 7.2.2.2-1, TS 38.174 and Table 7.2.2.2-2, TS 38.174 respectively.

- NPRB is the number of PRBs in a channel bandwith for the corresponding PREFSENS. NPRB for different channel BW is defined in section 5.3.2, TS 38.174.

- SNRP_REFSENS  is the SNR for which the PREFSENS requirements are derived. SNRP_REFSENS =- 1 dB according to section 8.2.2 (IAB-MT Reference sensitivity), TR 38.809.

- SSB Ês/Iot is -4 dB or -6 dB depending on the RRM requirement.

According to Tables 7.2.2.2-1 and 7.2.2.2-2 in TS 38.174, the same REFSENS value is applicabe for multiple bandwidths e.g. -102 dBm for 10 and 15 MHz for WA IAB-MT class. Therefore, it is more suitable to define the minimum SSB_RP as function of PREFSENS and NPRB. Furthermore, minimum SSB_RP for 1-H IAB type is valid only for SCS of 30 kHz. Based on the above methodology the minimum SSB_RP for IAB type 1-H is shown in table 2.

Table 2: Minimum SSB_RP for IAB type 1-H

| Section | Requirements | Minimum SSB_RP [dBm] |
| --- | --- | --- |
| 12.1.1.1.2.1 | RRC connection re-establishment to NR intra-frequency cell | PREFSENS -10*Log10(NPRB *12) - 5 |
| 12.1.1.1.2.1 | RRC connection re-establishment to NR inter-frequency cell | PREFSENS -10*Log10(NPRB *12) - 3 |
| 12.1.1.3.2.1 | RRC connection release with redirection to NR | PREFSENS -10*Log10(NPRB *12) - 3 |

### Minimum SSB_RP for IAB type 1-O

The methodology to derive minimum SSB_RP for IAB type 1-O is similar to that used for IAB type 1-H except that in 1-O the REFSENS is expressed by Equivalent Isotropic Sensitivity (EISREFSENS)

the minimum SSB_RP can be expressed by the following expression:

- Minimum SSB_RP = EISREFSENS -10*Log10(NPRB *12) – SNREIS_REFSENS + SSB Ês/Iot

Where:

- EISREFSENS is the EIS sensitivity value for certain channel bamdwidth for WA IAB-MT class and LA IAB-MT class defined in Table 10.3.3.2-1, TS 38.174 and Table 10.3.3.2-2, TS 38.174 respectively.

- NPRB is the number of PRBs in a channel bandwith for the corresponding EISREFSENS. NPRB for different channel BW is defined in section 5.3.2, TS 38.174.

- SNREIS_REFSENS  is the SNR for which the EISREFSENS requirements are derived. SNREIS_REFSENS =- 1 dB according to section 10.3.2 (IAB-MT OTA Reference sensitivity), TR 38.809.

- SSB Ês/Iot is -4 dB or -6 dB depending on the RRM requirement.

According to Tables 10.3.3.2-1 and 10.3.3.2-2, TS 38.174, the same REFSENS value is applicabe for multiple bandwidths e.g. (-102- ΔOTAREFSENS) dBm for 10 and 15 MHz for WA IAB-MT class. Therefore, it is more suitable to define the minimum SSB_RP as function of EISREFSENS and NPRB. Furthermore, minimum SSB_RP for IAB type 1-O is valid only for SCS of 30 kHz. Based on the above methodology the minimum SSB_RP for IAB type 1-O is shown in table 3.

Table 3: Minimum SSB_RP for IAB type 1-O

| Section | Requirements | Minimum SSB_RP [dBm] |
| --- | --- | --- |
| 12.1.1.1.2.1 | RRC connection re-establishment to NR intra-frequency cell | EISREFSENS -10*Log10(NPRB *12) - 5 |
| 12.1.1.1.2.1 | RRC connection re-establishment to NR inter-frequency cell | EISREFSENS -10*Log10(NPRB *12) - 3 |
| 12.1.1.3.2.1 | RRC connection release with redirection to NR | EISREFSENS -10*Log10(NPRB *12) - 3 |

### Minimum SSB_RP for IAB type 2-O

The methodology to derive minimum SSB_RP for IAB type 2-O is similar to that used for IAB type 1-O and can be broadly expressed by the same expression as below:

the minimum SSB_RP can be expressed by the following expression:

- Minimum SSB_RP = EISREFSENS -10*Log10(NPRB *12) – SNREIS_REFSENS + SSB Ês/Iot

Where:

- EISREFSENS is the EIS sensitivity value for certain channel bamdwidth for WA IAB-MT class and LA IAB-MT class defined in Table 10.3.3.3-1.

- NPRB is the number of PRBs in a channel bandwith for the corresponding EISREFSENS. NPRB for different channel BW is defined in section 5.3.2, TS 38.174.

- SNREIS_REFSENS  is the SNR for which the EISREFSENS requirements are derived. SNREIS_REFSENS =- 1 dB according to section 10.3.2 (IAB-MT OTA Reference sensitivity), TR 38.809.

- SSB Ês/Iot is -4 dB or -6 dB depending on the RRM requirement.

The minimum SSB_RP for IAB type 2-O is valid only for SCS of 120 kHz and associated channel BWs (50, 100, 200 and 400 MHz). Based on the above methodology the minimum SSB_RP for IAB type 2-O is shown in table 4.

Table 4: Minimum SSB_RP for IAB type 2-O

| Section | Requirements | Minimum SSB_RP [dBm] |
| --- | --- | --- |
| 12.1.1.1.2.1 | RRC connection re-establishment to NR intra-frequency cell | EISREFSENS -10*Log10(NPRB *12) - 5 |
| 12.1.1.1.2.1 | RRC connection re-establishment to NR inter-frequency cell | EISREFSENS -10*Log10(NPRB *12) - 3 |
| 12.1.1.3.2.1 | RRC connection release with redirection to NR | EISREFSENS -10*Log10(NPRB *12) - 3 |

- Summary

In this paper we have analyzed the side conditions for IAB-MT RRM requirements. The main observation and proposals are as follows:

- Observation 1: Minimum SSB_RP is derived from the reference sensitivity (REFSENS).

- Observation 2: Current minimum SSB_RP for IAB-MT RRM requirement refers to the UE minimum SSB_RP in TS 38.133.

- Observation 3: IAB-MT REFSENS differs with UE REFSENS in the following aspects:

- IAB-MT REFSENS depends on IAB type: different for 1-H, 1-O and 2-O

- IAB-MT REFSENS depends on IAB-MT class: different for LA and WA classes

- IAB-MT REFSENS does not depend on the frequency band

- IAB-MT REFSENS for IAB types 1-O and 2-O (OTA REFSENS) include a declarable parameters (ΔOTAREFSENS) which is the difference between conducted reference sensitivity and OTA REFSENS and include the antenna gain for different directions.

- Proposal 1: Minimum SSB_RP for IAB-MT RRM requirement is derived from the reference sensitivity (REFSENS) defined for LA and WA IAB-MT classes and for all IAB types (1-H, 1-O and 2-O).

CR to 38.174 on defining the side conditions for IAB-MT RRM requirements is provided in [1].

- References

- R4-2107135, Side conditions for IAB-MT RRM test cases in TS 38.174, Ericsson