3GPP TSG RAN WG4 Meeting #98bis-e	R4-2107156

Electronic Meeting, April 12-20, 2021

Agenda Item:	7.25.5

Source: 	Ericsson

Title:	Impact of new FR1 channel BWs on RRM requirements

Document for:	Discussion

- Introduction

Under Rel-17 WI on, “Introduction of channel bandwidths 35MHz and 45MHz for NR”, [NR_FR1_35MHz_45MHz_BW], RAN4 is defining requirements to introduce 35 MHz and 45 MHz channel BWs for several bands.

In this paper we analyze impact of new channel bandwidths on RRM requirements.

# Impact on RRM requirement

Currently the following channel bandwidths are defined for FR according to TS 38.101-1. The bandwidths supported by the UE depends on the frequency band:

Table 5.3.2-1: Maximum transmission bandwidth configuration NRB [TS 38.101-1]

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | 133 | 160 | 216 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | 11 | 24 | 38 | 51 | 65 | 78 | 106 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | 11 | 18 | 24 | 31 | 38 | 51 | 65 | 79 | 93 | 107 | 121 | 135 |

The UE and BS RF requirements are being specified for the new channel bandwidths namely 35 MHz and 45 MHz by the main session. While the specification work is done under Rel-17 WI. However, as per latest approved WID the following was approved at the last RAN plenary (RAN#91e): UE support of 35 MHz and 45 MHz channel bandwidth from Rel-15 onwards in a release independent manner

The UE RF requirements are not yet completed. However considerable progress has been made based on the approved WFs on various UE RF issues needed to complete the corresponding requirements [2-4].

Several RRM requirements depend on SSB bandwidth (in MHz) or CSI-RS bandwidth. But the RRM core requirements are agnostic to the channel bandwidth.

In FR1 the RRM performance requirements comprise of side conditions for SSB, CSI-RS, PRS, SRS (e.g. minimum SSB_RP, minimum CSI-RS_RP etc) are normalized to their SCS. The RRM test configurations and test cases in FR1 are currently defined for 10 MHz channel bandwidth for FDD and TDD bands and for 40 MHz channel bandwidth for TDD bands. The test requirements in the RRM tests do not depend on the channel bandwidth rather than RRM specific configurations e.g. SSB, SMTC, SRS configurations etc. Therefore, the current tests give good coverage for the bands in terms of RRM testing. We therefore do not foresee any impact of 35 MHz and 45 MHz channel bandwidths on any RRM core or performance requirements.

- Observation 1: In FR1 the RRM core requirements are agnostic to the channel bandwidth rather mainly depends on measured signals e.g. SSB, CSI-RS, PRS, SRS etc.

- Observation 2: In FR1 the RRM performance requirements including RRM test configurations and test cases are defined for 10 MHz channel bandwidth for FDD and TDD bands and for 40 MHz channel bandwidth for TDD bands.

- Proposal 1: Introduction of 35 MHz and 45 MHz channel bandwidths does not have any impact on RRM core requirements.

- Proposal 2: Introduction of 35 MHz and 45 MHz channel bandwidths does not have any impact on RRM performance requirements including RRM test cases.

- Summary

The impact of 35 MHz and 45 MHz channel bandwidths on RRM core and performance requirements has been analyzed. Following are the main observations and proposals:

- Observation 1: In FR1 the RRM core requirements are agnostic to the channel bandwidth rather mainly depends on measured signals e.g. SSB, CSI-RS, PRS, SRS etc.

- Observation 2: In FR1 the RRM performance requirements including RRM test configurations and test cases are defined for 10 MHz channel bandwidth for FDD and TDD bands and for 40 MHz channel bandwidth for TDD bands.

- Proposal 1: Introduction of 35 MHz and 45 MHz channel bandwidths does not have any impact on RRM core requirements.

- Proposal 2: Introduction of 35 MHz and 45 MHz channel bandwidths does not have any impact on RRM performance requirements including RRM test cases.

- References

- RP‑210722, Introduction of channel bandwidths 35MHz and 45MHz for NR, Huawei

- R4-2103186,	Way forward on spectrum utilization for 35MHz and 45MHz, Huawei

- R4-2103187,	Way forward on REFSENS for 35MHz and 45MHz, Qualcomm

- R4-2103188,	Way forward on A-MPR for 35MHz and 45MHz, Apple