3GPP TSG-RAN WG4 Meeting #98-bis-e                     	R4-2107182

# Electronic Meeting, April. 12 - April. 20, 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	On PRS-RSRP measurement period definition

Agenda item:	5.5.1.2

Document for:	Discussion

Introduction

At RAN4 #98-e, the discussion on PRS-RSRP measurement period definition was continued. A CR [1] has been agreed for framework on PRS-RSRP measurement requirement. Further remaining issues are captured in WF [2]. This contribution treats some of the open issues.

Discussion

RSRP measurement period with RSTD or UE Rx-Tx

- Measurement period when configured with RSTD or UE Rx-Tx - FFS Scenario #1: UE being configured to measure PRS-RSRP for DL-TDOA (or Multi-RTT) - UE behavior: - Option 1 - UE continues PRS-RSRP over the entire RSTD/UE Rx-Tx measurement period, when - PRS-RSRP is configured together with RSTD/UE Rx-Tx, and - the required PRS-RSRP measurement period is shorter than that for RSTD/UE Rx-Tx (without PRS-RSRP) - Option 2 - UE does not continue PRS-RSRP measurement - Applicable PRS-RSRP measurement period requirements - Option 1: - The minimum measurement period is extended to that for RSTD/UE Rx-Tx - Option 2: - The minimum measurement period is linked with the accuracy and thus does not need to be extended, but the UE shall continue measuring so the PRS-RSRP matches RSTD/UE Rx-Tx - Option 3 - Current requirements in clause 9.9.3 also apply for the case when PRS-RSRP is measured for DL-TDOA or Multi-RTT, the UE stops PRS-RSRP measurement - Scenario #2: UE being configured to measure PRS-RSRP for different positioning methods - If PRS-RSRP measurement is separately configured for each positioning method, the requirements shall be met independently for each positioning method.

We need to investigate RSRP value usage in localization algorithm. LMF can request various positioning measurements for specific localization algorithm, the RSRP values are very fundamental values to set up positioning measurements and to evaluate measurement reliability.

Regarding UE behavior, the LPP spec TS 37.355 defines a UE capability report on DL TDOA measurement with DL-PRS RSRP measurements “supportofDL-PRS-RSRP-Meas” field.

Therefore, if a UE reports this capability, the UE continues to measure PRS-RSRP over the entire RSTD/UE Rx-Tx measurement period.

Proposal 1: Regarding UE behavior, a UE continues to measure PRS-RSRP over the entire RSTD/UE Rx-Tx measurement period, when PRS-RSRP is configured together with RSTD/UE Rx-Tx.

Regarding the RSRP requirements when configured with RSTD or UE Rx-Tx, current requirements in clause 9.9.3 also apply for this case.

Proposal 2: RSRP requirements in clause 9.9.3 also apply for the case when configuring PRS-RSRP with RSTD or UE Rx-Tx.

Conclusion

This contribution has provided our view on some open issues with regard to defining PRS-RSRP measurement period.

Proposal 1: Regarding UE behavior, a UE continues to measure PRS-RSRP over the entire RSTD/UE Rx-Tx measurement period, when PRS-RSRP is configured together with RSTD/UE Rx-Tx.

Proposal 2: RSRP requirements in clause 9.9.3 also apply for the case when configuring PRS-RSRP with RSTD or UE Rx-Tx.

References

- R4-2104074, "CR 38.133 PRS-RSRP measurement requirements", source: Ericsson

- R4-2104076, "WF on UE PRS measurement requirements", source: Huawei, HiSilicon