3GPP TSG-RAN WG4 Meeting #98bis-e														R4-2107192

Electronic meeting, April 12th – 20th February 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	On UE Rx RF aspects for a NR band in the range 52.6GHz – 71GHz

Agenda item:	8.12.4.2

Document for:	Approval

# Introduction

The latest WID on extending current NR operation to 71GHz [1] was approved at RAN#91. Before that, 3GPP RAN studied requirements for NR beyond 52.6GHz up to 114.25GHz, potential use cases and deployment scenarios, and NR system design requirements and considerations on top of regulatory requirements as captured in [2].

In this contribution UE RF related aspects are discussed.

# Discussion

#### UE Rx requirements for licensed operation

NR operation in 52.6 – 71 GHz range can be seen as an extension of current FR2 NR operation, thus Rx requirements for NR operation in this frequency range can be reused as baseline for licensed operation. However, currently there are no part of the 52.6 – 71 GHz frequency range which have available regulations defining licensed operation. As a result, it might be premature to define UE Rx requirements for a licensed operation.

Observation 1: For licensed operation in the 52.6 – 71 GHz range NR FR2 Rx requirements can be reused as baseline.

Proposal 1: Postpone further discussion of UE Rx requirements for licensed operation until available spectrum becomes clear.

#### UE Rx requirements for unlicensed operation

When considering unlicensed deployments, it is suggested to align, where possible, to the ETSI EN 303 753 harmonized standard [3] in a similar manner as done for the unlicensed band in the 5 GHz and 6 GHz range.

Proposal 2: Align UE Rx requirements to the ETSI EN 303 753 harmonized standard where possible for unlicensed operation in the 57 – 71 GHz range.

However, e.g. reference sensitivity level, is currently not defined by the ETSI EN 303 753 harmonized standard [3]. Therefore, as a starting point it is suggested to use current FR2 NR operation as a baseline.

On top of this baseline it would be beneficial to discuss what are representative UE antenna array sizes used in this frequency range, as this has dependency with the achievable sensitivity level. This discussion is common with Tx requirements, as it antenna array size will naturally impact also achievable EIRP.

Observation 2: UE antenna array sizes for NR operation up to 71 GHz should be discussed.

For some receiver requirements, like adjacent channel selectivity, it would be possible to consider extracting the requirements from co-existence study documented in 38.803 [4]. Note that the required ACIR values for channel bandwidth wider than 200MHz (which was used in the NR study item) should be smaller considering the relatively impact of the ACI on the higher noise floor with the wider receive bandwidth, so there should be no issue to use the ACLR and ACS values from the NR study item for channel bandwidth wider than 200MHz.

Observation 3: It is possible to extract some requirements also from the co-existence study in 38.803.

Proposal 3: Where no Rx requirements is given by the ETSI EN 303 753 harmonized standard use current FR2 NR requirements as a baseline for unlicensed operation in the 57 – 71 GHz range.

However, as it is unlicensed deployments in shared spectrum some relaxations are expected as compared to licensed operation. These relaxations should carefully be chosen to ensure efficient NR-U operation but also consider already deployed 802.11 ad/ay in the shared spectrum.

Proposal 4: RAN4 to further discuss relaxation, if needed, of Rx requirements as compared to current FR2 NR requirements for unlicensed operation in the 57 – 71 GHz range.

# Conclusion

In this contribution, we discuss UE Tx RF aspects for a NR band in the range 52.6GHz – 71GHz. We have made following observations and proposals:

Observation 1: For licensed operation in the 52.6 – 71 GHz range NR FR2 Rx requirements can be reused as baseline.

Observation 2: UE antenna array sizes for NR operation up to 71 GHz should be discussed.

Observation 3: It is possible to extract some requirements, like ACS, also from the co-existence study in 38.803.

Proposal 1: Postpone further discussion of UE Rx requirements for licensed operation until available spectrum becomes clear.

Proposal 2: Align UE Rx requirements to the ETSI EN 303 753 harmonized standard where possible for unlicensed operation in the 57 – 71 GHz range.

Proposal 3: Where no Rx requirements is given by the ETSI EN 303 753 harmonized standard use current FR2 NR requirements as a baseline for unlicensed operation in the 57 – 71 GHz range.

Proposal 4: RAN4 to further discuss relaxation, if needed, of Rx requirements as compared to current FR2 NR requirements for unlicensed operation in the 57 – 71 GHz range.

# References

- RP‑210862, Revised WID on Extending current NR operation to 71GHz

- TR 38.807, “Study on requirements for NR beyond 52.6 GHz”, 3GPP

- draft EN 303 753 v0.0.3, ETSI BRAN

- TR 38.803, “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects”, 3GPP