3GPP TSG-RAN WG4 #98bis-e	R4-2107210

e-Meeting, Apr 12 – 20, 2020

Agenda item:	TBD

Source: 	Qualcomm Incorporated

Title: 	60GHz UE switching times

Document for:	Approval

# Discussion

Based on our analysis we have concluded that 7.015 µsec is needed for switching between UE TX and RX. For beam switching 200 nsec is needed, and there is a minimum time between consecutive beam switches of 4.5 µsec. Below we have a proposal for the response LS for UE. Any gNodeB answers would have to be added to this draft of the response LS.

3GPP TSG RAN WG4 Meeting #98-bis-e						R4-TBD

e-Meeting, April 12 –20, 2021

Title:	Reply LS on beam switching gap for 60 GHz band

Response to:	R1-2102202

Release:	Rel-17

Work Item:	NR_ext_to_71GHz

Source:	RAN4

To:	RAN1

Cc:

Contact Person:

#### Name:	TBD

E-mail Address:	TBD

Attachments:

1. Overall Description:

RAN1 has asked RAN4 about times required for gNBs and UEs operating in 52.6 GHz to 71 GHz to perform the following operations (in R1-2102202):

- Switching Tx beams

- Switching Rx beams

- Switching from DL to UL

- Switching from UL to DL

RAN4 has concluded discussion on the LSin  R1-2102022. Our conclusions are as follows:

Switching UE from downlink to uplink or from uplink to downlink:

For frequencies between 52.6 ~ 71 GHz, UE switching time DL-to-UL or UL-to-DL requires up to 13792 Tc (=7.015 µsec). This time includes any necessary beam switches.

Switching UE beams (applicable for both TX and RX beams):

For frequencies between 52.6 ~ 71 GHz UE, the time to switch beams is 200 nsec and the minimum duration between the start of any two consecutive beam switches is 4.5  µsec.

[TBD  on gNB]

2. Actions:

To RAN1: 	RAN4 would like to kindly ask RAN1 to take this into consideration in the RAN1 design.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG4 Meeting #99-e 	19 – 27 May 2021			Online

TSG-RAN WG4 Meeting #100-e	TBD August 2021			Online