3GPP TSG-RAN WG4 Meeting # 98-bis-e 	R4-2107219

# Electronic Meeting, Apr. 12-20, 2021

Agenda item:	5.2.1.2

Source:	Qualcomm Inc.

Title:	On NR V2X Single Link Demod Requirement

Document for:	Discussion

# Introduction

In RAN4#98e meeting, partial alignment results are collected. In this contribution, we provided views on the potential issues.

# Discussion

## PSCCH and PSBCH test comparison

From the simulation results summary in the previous meeting, we observe larger deviation in PSBCH simulation results. We conduct further analysis on our results to verify its validity by comparing the results from PSBCH and PSCCH.

The code rate of PSCCH test is: (26+24)/(10*2*18) = 0.14

The code rate of PSBCH test is (32+24)/(11*8*18) = 0.035

Since both code rates are low, we can ignore the different coding scheme and length. Based on code rate comparison, under the same channel condition, we expect PSBCH is roughly 6dB better than PSCCH. Our simulation results show that PSCCH in TDL_A30 with 260km/h speed can achieve 1% BLER at 2.84dB. Here are our PSBCH simulation results:

Under the same channel condition, PSBCH achieve 1% BLER at -4dB, which is 6.84dB lower than PSCCH 1% BLER. The difference aligns to code rate difference.

Note that both PSBCH and PSCCH have DMRS on every symbol. Therefore, Doppler spread doesn’t degrade the performance. Since the code rate is low, diversity on time domain improves the performance, and high Doppler can introduce higher diversity on time domain. Therefore, as we shown in the PSBCH results, 260km/h speed has better performance than 30km/h speed.

Proposal: Use the following guidelines to align PSBCH alignment results:

- Under the same propagation condition, code rate difference contributes to most of the performance difference between PSCCH and PSBCH.

- Higher speed (Doppler spread) yields better performance for PSBCH.

# Conclusions

Proposal: Use the following guidelines to align PSBCH alignment results:

- Under the same propagation condition, code rate difference contributes to most of the performance difference between PSCCH and PSBCH.

- Higher speed (Doppler spread) yields better performance for PSBCH.