3GPP TSG-RAN WG4 Meeting #98bis-e	R4- 2107258

Electronic Meeting, 12 – 20 April 2021

Source: 	Huawei, HiSilicon

Title:	MPR Simulation Assumptions for 16QAM NB-IoT Uplink

Agenda Item:	10.9.2.2

Document for:	Approval

# 1	Introduction

The objective is to specify the following enhancements to NB-IoT and/or LTE-MTC for BL/CE UEs. When an objective applies to NB-IoT, it is indicated with [NB-IoT]. When an objective applies to LTE-MTC, it is indicated with [LTE-MTC].

- Specify 16-QAM for unicast in UL and DL, including necessary changes to DL power allocation for NPDSCH and DL TBS. This is to be specified without a new NB-IoT UE category. For DL, increase in maximum TBS of e.g. 2x the Rel-16 maximum, and soft buffer size will be specified by modifying at least existing Category NB2. For UL, the maximum TBS is not increased. [NB-IoT] [RAN1, RAN4]

  - Extend the NB-IoT channel quality reporting based on the framework of Rel-14—16, to support 16-QAM in DL. [NB-IoT] [RAN2, RAN1, RAN4]

- Support additional PDSCH scheduling delay for introduction of 14-HARQ processes in DL, for HD-FDD Cat M1 UEs. [LTE-MTC] [RAN1]

- Specify signaling for neighbor cell measurements and corresponding measurement triggering before RLF, to reduce the time taken to RRC reestablishment to another cell, without defining specific gaps. [NB-IoT] [RAN2, RAN4].

- Introduce support for NB-IoT carrier selection based on the coverage level, and associated carrier specific configuration (e.g. maximum repetitions UL/DL, DRX configurations, etc.). [NB-IoT] [RAN2, RAN3]

- For UEs supporting PUSCH sub-PRB resource allocation, study and if found feasible, specify support power reduction for PRACH, PUCCH, and full-PRB PUSCH, with a maximum reduction of e.g. 3 dB below sub-PRB PUSCH power. [LTE-MTC] [RAN4]

In RAN#86, the WI Rel-17 enhancements for NB-IoT and LTE-MTC was approved [1]. The WI objectives are reproduced below.

It can be seen that one of RAN4’s tasks is to specify the support of 16QAM in the UL for NB-IoT.

# 2	Discussion

It is well known that the SC-FDMA waveforms using 16QAM typically exhibit larger PAPR than those using QPSK. Therefore extra MPR may be needed for 16QAM. A similar simulation campaign as in Rel-13 is expected.

Proposal 1: Study and specify MPR for 16QAM for NB-IoT UL.

MPR=0 for single tone transmissions (worst case among 3.75 pi/2 BPSK, pi/4 QPSK, 15 kHz pi/2 BPSK, and pi/4 QPSK)

Image rejection = 25 dB

LO leakage = 25 dB

CIM3 = 60 dB

Power classes 3 and [5 in case approved]

Phase noise: If phase noise model is used in simulations it shall be reported in contribution

Pulse shaping filtering: If used companies shall report the parameters of filtering

In RAN4#78bis, the MPR simulation assumptions were agreed for QPSK [2]. The key points are reproduced below.

We believe that these assumptions are still valid for 16QAM MPR simulations.

Proposal 2: The 16QAM MPR study should reuse the simulation assumptions for QPSK. Companies are encouraged to provide simulation results in the following meetings.

# 3	Conclusion

MPR study is identified as one of the tasks in order to support 16QAM in NB-IoT UL. And the following proposals are presented.

Proposal 1: Study and specify MPR for 16QAM for NB-IoT UL.

Proposal 2: The 16QAM MPR study should reuse the simulation assumptions for QPSK. Companies are encouraged to provide simulation results in the following meetings.

# References

[1] RP-193264 New WID on Rel-17 enhancements for NB-IoT and LTE-MTC, Huawei, HiSilicon, RAN#86, Dec 2019

[2] R4-162850 WF on MPR simulation assumption, Nokia, Intel, RAN4#78bis, April 2016