3GPP TSG-RAN WG4 Meeting#98-bis             	R4-2107325

E-meeting, 12th – 20th April, 2021

Title:	n2 25,30,40MHz REFSENS

Source:	Skyworks Solutions Inc.

Agenda item:	7.24.2.1 Reference sensitivity [NR_bands_R17_BWs -Core]

Document for:	Approval

# 1	Introduction

This document presents power amplifier (PA) noise measurements to finalize 25,30,40MHz REFSENS requirements for bands n2 based on WF agreement [1].

## 2	Discussion

### 2.1	Background

REFSENS levels for 25,30 and 40MHz have already been agreed for n25. This document brings PA noise emission levels in n2 receiver band which, as observed for the band n25 studies [2] may lead to REFSENS levels lower than the n25 baseline agreements. In which case, an alignment on previously agreed maximum sensitivity degradation is adopted to derive n2 REFSENS levels.

### 2.2	Power Amplifier Calibration and REFSENS Assumptions

For Power Amplifier (PA) transmitter (Tx) noise emission in receiver (Rx) band, we assume:

- PA calibration point is 20 MHz, 15 kHz, QPSK, DFT-S-OFDM, 100 RB at lower channel edge with 1 dB MPR;

- Post PA losses: 4dB;

- Power Class 3 (PC3) operation;

- Local Oscillator (LO) leakage: -28dBc;

- IQ Image rejection: -28dB; and

- C-IM3: -60dBc, C-IM5: -70dBc.

For REFSENS estimations, we assume:

- 50 dB Tx to RX and Tx to Antenna duplexer rejection in Rx band;

- 10 dB Primary antenna to diversity antenna isolation;

- 4 dB Post PA loss;

- 4 dB Antenna to LNA front-end losses;

- 3 dB Antenna diversity gain;

- -1 dB SNR; and

- Previously agreed rules to compute correlated and uncorrelated MRC contributions.

### 2.3	n2 MSD and REFSENS

We measure Tx noise in Rx band with Lcrb=50, 48 and 40 RBs located as close as possible to the downlink operating band for 25,30 and 40MHz channel bandwidth (CBW) respectively. Noise levels are -49.1 dBm/23.955MHz, -46.6 dBm/28.815MHz and -39.5dBm/38.895MHz. The corresponding MSD levels are 0.5dB,0.8dB and 2.1dB for 25,30,40MHz respectively as shown in Table 1.

Table 1 n2 25,30,40MHz Tx noise level in Rx band and MSD analysis

For 25MHz, MSD can be neglected. This is in line with previously agreed REFSENS for n25.

We note however that for 30MHz and 40MHz CBW, the MSD levels are lower than those previously agreed for n25. We assume that the REFSENS levels for n2 30 and 40MHz cannot lower than legacy levels for n25. The MSD for n25 is 6.5dB and 7.5dB for 30 and 40MHz respectively. We propose to adopt n25 6.0 dB MSD for 30MHz and 7.5dB for 40MHz.

Proposal 1: Adopt the following n2 25,30 and 40MHz antenna port Reference Sensitivity QPSK and Uplink Configuration tables.

Table 2 Two Antenna Port 25,30,40MHz Reference Sensitivity QPSK PREFSENS

| Operating Band | SCS kHz | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | Duplex Mode |
| --- | --- | --- | --- | --- | --- |
| n21 | 15 | -90.7 | -83.9 | -81.1 | FDD |
|  | 30 | -90.8 | -84.0 | -81.2 |  |
|  | 60 | -91.0 | -84.1 | -81.3 |  |

Table 3 Uplink Configuration for 25,30,40MHz Reference Sensitivity

| Operating Band | SCS kHz | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | Duplex Mode |
| --- | --- | --- | --- | --- | --- |
| n21 | 15 | 501 | 481 | 401 | FDD |
|  | 30 | 241 | 241 | 201 |  |
|  | 60 | 101 | 101 | 101 |  |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1) |  |  |  |  |  |

3	Conclusion

In this contribution, we make proposals n2 REFSENS requirements.

Proposal 1: Adopt the following n2 25,30 and 40MHz antenna port Reference Sensitivity QPSK and Uplink Configuration tables.

Table 2 Two Antenna Port 25,30,40MHz Reference Sensitivity QPSK PREFSENS

| Operating Band | SCS kHz | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | Duplex Mode |
| --- | --- | --- | --- | --- | --- |
| n21 | 15 | -90.7 | -83.9 | -81.1 | FDD |
|  | 30 | -90.8 | -84.0 | -81.2 |  |
|  | 60 | -91.0 | -84.1 | -81.3 |  |

Table 3 Uplink Configuration for 25,30,40MHz Reference Sensitivity

| Operating Band | SCS kHz | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | Duplex Mode |
| --- | --- | --- | --- | --- | --- |
| n21 | 15 | 501 | 481 | 401 | FDD |
|  | 30 | 241 | 241 | 201 |  |
|  | 60 | 101 | 101 | 101 |  |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1) |  |  |  |  |  |

# References

[1] R4-2103184, Way forward on the introduction of 25, 30 and 40MHz in band n2, 3GPP TSG-RAN WG4 #98-e, Online, Electronic Meeting, 25 Jan. – 5 Feb. 2021, AT&T.

[2] R4-2103184, WF on REFSENS for 35MHz and 45MHz, 3GPP TSG-RAN WG4 #98-e, Online, Electronic Meeting, 25 Jan. – 5 Feb. 2021, Qualcomm, Skyworks.