3GPP TSG-RAN WG4 Meeting # 98-bis-e 	R4-2107332

Electronic Meeting, Apr. 12-20, 2021

Source:	T-Mobile USA

Title:	TP for TR 37.826 for PC2 DC_66A_n41A

Agenda item:	7.21.2

Document for:	Approval

- Introduction

This contribution is a text proposal for TR 37.826 to include DC_66A_n41A according to the request in [1].

The MSD due to cross band isolation for DC_66a_n41A in PC3 was 3.5 dB. 5.4 dB was calculated for PC2 by calculating the interferer power for PC3, and adding 3 dB, then calculating the new MSD. Spreadsheet attached.

- Reference

[1] RP-210327, Revised WID on High power UE (power class 2) for EN-DC with 1 LTE band + 1 NR TDD band

- Text Proposal

<Beginning of changes>

#### 6.6.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B DC_66A_n77A are same as the Case A defined in table 6.6.2.1.1-1.

## 6.x	DC_66a_n41A

### 6.x.1	Transmitter Characteristics

#### 6.x.1.1	Maximum Output Power

Table 6.x.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_66a_n41A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. |  |  |

#### 6.x.1.2	Co-existence study

The co-existence study for PC2 DC_66a_n41A can be reused from the PC3 DC_66a_n41A captured in TR37.716-11-11, where:

-    MSD due to cross band isolation into B66 Rx.

Therefore, additional MSD for cross band isolation needs to be defined for PC2 DC_66a_n41A.

### 6.x.2	Receiver Characteristics

#### 6.x.2.1	MSD for cross band isolation

The MSD due to cross band isolation for DC_66a_n41A in PC3 was 3.5 dB. 5.4 dB was calculated for PC2 by calculating the interferer power for PC3, and adding 3 dB, then calculating the new MSD.

Table 6.x.2.1 Reference sensitivity exceptions (MSD) due to cross band isolation for PC2 EN-DC in NR FR1

|  | E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n41 | 66 | 5.4 | 5.4 | 5.4 | 5.4 |  |  |  |  |  |  |  |  |

<End of changes>