3GPP TSG-RAN WG4 Meeting # 98-bis-e 	R4-2107333

Electronic Meeting, Apr. 12-20, 2021

Source:	T-Mobile USA

Title:	TP for TR38.841 for PC2 CA_n25A-n41A

Agenda item:	7.20.2

Document for:	Approval

- Introduction

This contribution is a text proposal to introduce PC2 2DL/2UL CA_n25A-n41A according to the request in [1].

The cross band isolation MSD was calculated by calculating the interferer power for 0.6 dB MSD for PC3, adding 3 dB to the interferer power and calculating MSD of 1.1 dB. The spreadsheet is attached.

- Reference

[1] RP-210476, Revised basket WID: High power UE for NR inter-band Carrier Aggregation with 2 bands downlink and x bands uplink (x =1,2)

- Text Proposal

<Beginning of changes>

### 5.7.4		∆TIB and ∆RIB values

For the ∆TIB,c and ∆RIB,c values, same PC3 CA_n66A-n77A requirements are applied for PC2 CA_n66A-n77A.

## 5.x	CA_n25A-n41A

### 5.x.1	Configurations

Table 5.x.1-1: NR CA configurations and bandwidth combinations sets for supporting power class 2

| NR CA configuration | Uplink CA configuration | NR Band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n25A-n41A | CA_n25A-n41A | n25 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n41 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |

### 5.x.2	Maximum output power

Table 5.x.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n41A-n71A | CA power class | Carrier n25 power class | Carrier n41 power class |
| --- | --- | --- | --- | --- |
| CA_n25A-n41A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.x.3	REFSENS requirements

According to the PC3 CA_n25A-n41A study, there is n25 MSD due to cross band isolation from the n41 UL. For PC3 the MSD is 3.5 dB.

#### 5.x.3.1	Power class 2 case a

For power class 2 case a MSD due to cross band isolation is the same as for PC3 because the n41 power is limited to 23 dBm.

#### 5.x.3.2	Power class 2 case b

For power class 2 case b there is 3 dB more power in n41, so the MSD was calculated by calculating the interferer power for 0.6 dB MSD for PC3, adding 3 dB to the interferer power and calculating MSD of 1.1 dB.

Table 5.x.3.2-1 UE Power Class 2 MSD due to cross band isolation

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n411 | n25 | 1.1 | 1.1 | 1.1 | 1.1 |  |  |  |  |  |  |  |  |  |

### 5.x.4	Void

<Editor’s note: If no change by comparing to the values for power class 3 CA, this section will be void.>

<End of changes>

# Annex <A> (informative): Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-11 | RAN4 #97-e | R4-2015187 |  |  |  | TR skeleton | 0.0.1 |
| 2020-12 | RAN4 #97-e | R4-2015188 |  |  |  | Implemented TPs from RAN4 #97-e: R4-2016855, TP for TR38.xxx_ PC2 CA_n3A-n41A , ZTE Corporation, CMCC R4-2015054, TP for TR38.xxx_ PC2 CA_n28A-n41A, ZTE Corporation, CMCC R4-2015055, TP for TR38.xxx_ PC2 CA_n28A-n79A, ZTE Corporation, CMCC R4-2015056, TP for TR38.xxx_ PC2 CA_n40A-n41A, ZTE Corporation, CMCC | 0.1.0 |
| 2021-02 | RAN4 #98-e | R4-2101125 |  |  |  | Implemented TPs from RAN4 #98-e: R4-2103169, TP for TR38.xxx for PC2 CA_n2A-n77A, Verizon, Ericsson, MediaTek, LGE R4-2103170, TP for TR38.xxx for PC2 CA_n5A-n77A, Verizon, Ericsson, MediaTek, LGE R4-2103171, TP for TR38.xxx for PC2 CA_n66A-n77A, Verizon, MediaTek, LGE, Ericsson R4-2102220, TP for TR38.xxx_Clarification on PC2 CA_n28A-n41A, CA_n28-n79A and CA_n40A-41A, ZTE Corporation, CMCC | 0.2.0 |