3GPP TSG-RAN WG4 Meeting # 98-bis-e 	R4-2107334

Electronic Meeting, Apr. 12-20, 2021

Source:	T-Mobile USA

Title:	TP for TR38.841 for PC2 CA_n41A-n66A

Agenda item:	7.20.2

Document for:	Approval

- Introduction

This contribution is a text proposal to introduce PC2 2DL/2UL CA_n41A-n66A according to the request in [1].

The cross band isolation MSD was calculated by calculating the interferer power for 3.5 dB MSD for PC3, adding 3 dB to the interferer power and calculating MSD of 5.4 dB. The spreadsheet is attached.

- Reference

[1] RP-210476, Revised basket WID: High power UE for NR inter-band Carrier Aggregation with 2 bands downlink and x bands uplink (x =1,2)

- Text Proposal

<Beginning of changes>

### 5.7.4		∆TIB and ∆RIB values

For the ∆TIB,c and ∆RIB,c values, same PC3 CA_n66A-n77A requirements are applied for PC2 CA_n66A-n77A.

## 5.x	CA_n41A-n66A

### 5.x.1	Configurations

Table 5.x.1-1: NR CA configurations and bandwidth combinations sets for supporting power class 2

| NR CA configuration | Uplink CA configuration | NR Band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n41A-n66A | CA_n41A-n66A | n41 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  |  |

### 5.x.2	Maximum output power

Table 5.x.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n41A-n71A | CA power class | Carrier n41 power class | Carrier n66 power class |
| --- | --- | --- | --- | --- |
| CA_n41A-n71A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |

### 5.x.3	REFSENS requirements

According to the PC3 CA_n41A_n66A study, there is a  an IMD4 that falls into n41. However, since n41 UL is TDD this is irrelevant.

There is also n66 MSD due to cross band isolation from the n41 UL. For PC3 the MSD is 3.5 dB.

#### 5.x.3.1	Power class 2 case a

For power class 2 case a MSD due to cross band isolation is the same as for PC3 because the n41 power is limited to 23 dBm so no change to MSD.

#### 5.x.3.2	Power class 2 case c

For power class 2 case b there is 3 dB more power in n41, so the MSD was calculated by calculating the interferer power for 3.5 dB MSD for PC3, adding 3 dB to the interferer power and calculating MSD of 5.4 dB.

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n411 | n66 | 5.4 | 5.4 | 5.4 | 5.4 |  |  | 5.4 |  |  |  |  |  |  |

### 5.x.4	Void

<Editor’s note: If no change by comparing to the values for power class 3 CA, this section will be void.>

<End of changes>