3GPP TSG-RAN4 Meeting #98-bis-e		R4-2104444

Electronic Meeting, 12 April – 20 April, 2021

3GPP TSG RAN WG1#104-e		  	R1-2102011

e-Meeting, January 25th – February 5th, 2021

Title:	Reply LS on measuring CSI-RS during SCell activation

Response to:	LS R1-2100008 (R4-2017381) from TSG RAN WG4

Release:	16

Work Item:	NR_unlic-Core

Source:	TSG RAN WG1

To:	TSG RAN WG4

Contact person:	Alexander Golitschek

aelbwart @ lenovo.com

# 1	Overall description

RAN1 would like to thank RAN4 for their LS in R1-2100008 (R4-2017381) on measuring CSI-RS during SCell activation.

RAN1 discussed the questions about the UE behavior with respect to CSI reports during the SCell activation procedure in case none or some of RRC parameters CO-DurationPerCell-r16, SlotFormatIndicator, and CSI-RS-ValidationWith-DCI-r16 are configured with or without corresponding DCIs for the SCell being activated:

Question by RAN4 (1)   When none of the RRC parameters CO-DurationPerCell-r16, SlotFormatIndicator, and CSI-RS-ValidationWith-DCI-r16 is configured for a UE on the being-activated SCell,

a.      What is the expected UE behavior for this P/SP CSI-RS measurement and report on the being-activated SCell?

Reply by RAN1: As in Rel-15, the UE is expected to receive the P/SP CSI-RS.

Question by RAN4 (2)   When RRC parameters CSI-RS-ValidationWith-DCI-r16 is configured, but SlotFormatIndicator and CO-DurationPerCell-r16 are not configured for the being-activated SCell,

a.      What is the expected UE behavior for this P/SP CSI-RS measurement and report on the being-activated SCell? Does UE need to decode a DCI format from other active serving cell (indicating an aperiodic CSI-RS reception or scheduling a PDSCH reception in the set of symbols of the slot) for this being-activated SCell to validate this P/SP CSI-RS?

Question by RAN4 (3)   When RRC parameters CO-DurationPerCell-r16 is configured but SlotFormatIndicator is not configured for the being-activated SCell,

a.      What is the expected UE behavior for this P/SP CSI-RS measurement and report on the being-activated SCell? Does UE need to decode a DCI format 2_0 (indicating remaining channel occupancy duration) from other active serving cell for this being-activated SCell to validate the CSI-RS?

Question by RAN4 (4)   When RRC parameters CO-DurationPerCell-r16 is not configured but SlotFormatIndicator is configured for the being-activated SCell,

a.      What is the expected UE behavior for this P/SP CSI-RS measurement and report on the being-activated SCell? Does UE need to detect a DCI format 2_0 (indicating the starting point of CO duration and the slot format) from other active serving cell for this being-activated SCell to validate the CSI-RS?

Reply by RAN1: RAN1 has discussed these cases, but has not achieved consensus on the expected UE behaviour. RAN1 will inform RAN4 if consensus is achieved in the future.

# 2	Actions

To TSG RAN WG4

ACTION: 	RAN1 respectfully asks RAN4 to consider the above reply.

# 3	Dates of next TSG RAN WG1 meetings

TSG RAN WG1 Meeting #104bis-e	12 - 20 April 2021 		Online

TSG RAN WG1 Meeting #105-e	19 - 27 May 2021 		Online