3GPP TSG-RAN WG4 Meeting # 99-e 													R4-2107650

Electronic Meeting, 19th – 27th May 2021

Agenda item:			8.31, 8.32

Source:	Moderator (Qualcomm Incorporated)

Title:	Email discussion summary for [99-e][124][ HPUE_PC1_5_n77_n78_n79]

Document for:	Information

# Introduction

This document summarizes the email discussion on PC 1.5 in Bands n77, n78, and n79 for FWA and smartphone.  The topics are

- MPR for smartphone

- MPR for FWA

- SAR and MPR

- UE RF requirements for Band n79

# Topic #1: MPR for smartphone

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2109441 | Apple | Title: Considerations on n77 and n78 Proposal 1: The considerations and proposal from our older contributions on PC1.5 MPR should be considered for the new discussion on mobile handset MPR. |
| R4-2110985 | Qualcomm Incorporated | Title: A reconsideration of PC1.5 MPR for smartphones Modulation; MPR (dB). Edge RB allocations; Outer RB allocations; Inner RB allocations. DFT-s-OFDM; Pi/2 BPSK; ≤ 6.5; ≤ 0.5; ≤ 0. QPSK; ≤ 6.5; ≤ 1; ≤ 0. 16 QAM; ≤ 6.5; ≤ 2; ≤ 1. 64 QAM; ≤ 6.5; ≤ 3.0; ≤ 3. 256 QAM; ≤ 7.5; ≤ 5.5; ≤ 5.5. CP-OFDM; QPSK; ≤ 6.5; ≤ 3; ≤ 1.5. 16 QAM; ≤ 6.5; ≤ 3; ≤ 2. 64 QAM; ≤ 6.5; ≤ 4; ≤ 4. 256 QAM; ≤ 9.5; ≤ 7.5; ≤ 7.5. |
| R4-2111009 | Skyworks Solutions, Inc. | Title: Evaluation of Reverse IMD versus antenna isolation and its impact to MPR Proposal on TxDiv and single stream UL MIMO evaluation: • The emissions should be evaluated as the mathematical (RMS) sum of the two PA coupled spectrums and not with a physical (RF combiner) sum. • Even so the PA coupled spectrum is subject to nulls due to phase combinations or delay which can impair measurement by up to 3dB: o Phase and/or delay may need to be adjusted to avoid such nulling/summing effects especially for narrow allocations o PA output power needs to be summed mathematically (RMS sum) o Ideally for UL MIMO all 4 states should be evaluated o Ideally for CDD based TxDiv companies should agree on which delay to use o Reasonable margin should be allowed in the MPR as these issues will not reveal at conformance (the antenna coupling disappears with connected measurements) but will exist in the field. o When submitting MPR values companies should provide an assessment of the issues above in their measurement or simulation framework. Proposal on MPR evaluation: • MPR assessment must account for RIMD and its different behavior for different PA architectures • MPR improvements may not be all related to RIMD and antenna isolations thus variability in WOLA design and TRX impairment must be accounted for |
| R4-2108974 | CMCC | Title: Discussion on the UE RF requirements of PC1.5 n79 Proposal 1: We propose to re-evaluate PC1.5 MPR in inner, outer and edge case, and the MPR re-evaluate need to consider Smartphone and FWA UEs. |

## Open issues summary

Apple reinforces the measurements that were previously provided in R4-2009943, Qualcomm evaluates MPR based on previously provided data as well as new data, and Skyworks describes the method for collecting data.  CMCC requests that PC 1.5 MPR is re-evaluated which was already agreed last meeting, but wants to ensure that the re-evaluation includes all waveforms inner, outer, and edge for both smartphone and FWA.

### Sub-topic 1-1

Is the previously reported data still regarded as valid for smartphone given the previous assumptions (i.e., antenna isolation 10 dB, etc)?  Was the data collected correctly given the discussion in Skyworks R4-2111009?  Note that this is not calling into question the assumptions, but the way the data was collected.

Open issues and candidate options before e-meeting:

Issue 1-1: Validity of previously reported data

- Proposals

  - Option 1: Previous data is valid

  - Option 2: Previous data was not collected properly.  It should be discarded and updated.  Identify specifically what was incorrect about how the data was collected.

- Recommended WF

  - TBA

### Sub-topic 1-2

If the answer to sub-topic 1-2 is “Option 1: Previous data is still valid”, is the analysis of that data provided by Qualcomm in R4-2110985 correct?  Does it take into account all the data (not necessarily the proposals but just the data) that was made available?  If not, how should the analysis be modified and/or what data was omitted?

Open issues and candidate options before e-meeting:

Issue 1-2: Analysis of previously reported data

- Proposals

  - Option 1: The analysis in R4-2110985 is correct for the data available

  - Option 2: The analysis needs to be revised.  Please be specific.

- Recommended WF

  - TBA

### Sub-topic 1-3

A proposal for PC1.5 smartphone MPR was provided by Qualcomm in R4-2110985.  From both perspectives of UE feasibility and network suitability, is the proposal acceptable?

Open issues and candidate options before e-meeting:

Issue 1-3: Proposal for smartphone MPR

- Proposals

  - Option 1: The proposal in R4-2110985

  - Option 2: Other proposal.

- Recommended WF

  - TBA

## Companies views’ collection for 1st round

### Open issues

One of the two formats, i.e. either example 1 or 2 can be used by moderators.

Example 1

| Company | Comments |
| --- | --- |
| Qualcomm | Sub topic 1-1: In reviewing the data previously presented, very few details were provided on the test setup so it was not always possible to fully comprehend how the data was collected. However, we assume that the data was collected by each company according to best engineering practices and expect that it is valid for the assumptions taken. Sub topic 1-2: …. Others: |
| CMCC | Sub topic 1-1: Option 2 Sub-topic 1-3: Option 1: The proposal in R4-2110895 |
| Skyworks | Sub topic 1-1: As explained over email, our previous released measured data for PC1.5 was done with varying phase and delays to remove the issues described in our paper. As for the other contributions we don’t want to suggest they were wrong but at the time the agreed MPR had sufficient margin in our view that the errors we describe would not affect the result. Conversely since we are now trying to optimize to the last 0.5dB, these issues can heavily affect the results and the remaining margin. We thus think it is important that every contributors has a critical look at his data especially if different phases have not been analyzed or if the way the PA coupling is modelled/reproduced in the setup prevents from seeing the effect we describe in our paper. Especially we have not seen any explanation on which waveforms have been used for the different evaluations and their parameters (phse for single stream UL MIMO, or delay for CDD TxDiv) Sub topic 1-2: in the data taken into account there is a mix of cases: for example R4-2011782 only uses 2 stream MIMO thus the issues I describe in my paper is not present, no edge allocation For TxDiv, Qualcomm has also shown that PC2 based on two PC# PA need relaxation for 256QAM EVM (even with 31dB ACLR calibration). This should apply here aslo. Sub topic 1-3: at this point we would not agree to the proposed value without a meticulous measurement campaign which cover all types of 2Tx cases (TxDiv/1 stream UL MIMO/2 stream Ul MIMO) |
| Apple | Issue 1-1: Option 1. In our earlier paper R4-2009943 we provided a detailed list and description for the measurements. Issue 1-2: The contribution from Qualcomm does a good job at summarizing the earlier contributions. However, we want to emphasize that the proposed MPR is quite aggressive and should be revised. The proposal seems to be based on measurements on 20MHz CBW only. During the evaluation from last year, we found that larger CBW at 100MHz see limitations, especially with ET. The results for 20MHz were typically better. As MPR is generally applicable to all CBW the proposal should be based on measurements made on CBW for 100MHz. Furthermore, we would like to propose that ET should be used. ET is an essential tool to decrease power consumption in the UL. Unfortunately, it has higher rIMD compared to APT and therefore will require slightly increased power backoff. However, in general it should be stated whether measurements were conducted with ET or APT. Issue 1-3: Option 2. Please see our comment on Issue 1-2. |
| Huawei | Sub topic 1-1: Checking the validity of past measurement data is not an easy task. Without thorough examination and close collaboration from the proponents, it’s difficult to make any conclusions. It’s only fair to say proponents have made their best effort and their contributions are appreciated. Sub topic 1-2: The analysis claim “In all cases the SEM was easily met with good margin.” But the yellow marked numbers in Figure 1 show that the margin is about 1 dB for “Bias B”. For “Bias A”, the SEM margin is improved but the ACLR margin is 0.8 dB. In our view, this does not count as “good margin”. More importantly, only in Skyworks’ contribution the PA calibration point was stated, i.e. drive the PC2 PA into compression until 1 dB MPR is needed for the reference waveform. Based on some reasonable analysis of the available data, it’s suspected that some measurements were not based on such calibration, resulting in large margins. Sub topic 1-3: Option 2. Option 1 is based on measurements on limited PA samples or PA types. It does not consider the effect of different PA design technologies or the variation of semiconductor process. For the time being, we don’t see strong evidence to change the existing PC1.5 MPR. |
| Qualcomm | In response to some of the comments provided on R4-2110985, both 20 MHz and 100 MHz for both APT and ET were used. This is in the datasets that companies previously provided and serves as the basis for the proposal in R4-2110985. There is no reason to believe that companies did not provide data according to best engineering practices but I leave it to the companies who provided data previously to comment if they had overlooked something. The results reported in R4-2011782 do show good margin to SEM because the reported results are without any MPR at all, not even the PC2 MPR. Therefore, when PC2 MPR (or higher) is applied, then there is good margin for all waveforms. And again, I trust that companies who provided data followed all the usual PA calibration techniques since this is done for any of these types of measurements. But I invite companies who supplied the data to respond directly if my assumptions are wrong. |
| LGE | Issue 1-1: Validity of previously reported data Prefer option 1: Option 1: Previous data is valid Issue 1-2: Analysis of previously reported data Prefer to need further analyse the outer and inner RB allocation based on LGE & SKW results (R4-2106639) since dominant factor is not only EVM to derive MPR requirements. RAN4 should be consider the emission requirements (ACLR, general SEM, and general SE) and cubic metric. Issue 1-3: Proposal for smartphone MPR LGE prefer to reuse the MPR requirements for PC1.5 n41 UL-MIMO. |
| Vivo | Sub topic 1-1: Prefer to option 1 |
| T-Mobile USA | Issue 1-1: Option 1: Previous data is valid. Issue 1-2: Option 1 Issue 1-3: Option 1 |
| Verizon | Issue 1-1: Option 1 Issue 1-2: Option 1: Yes, previous data is still valid! Issue 1-3: Agree with R4-2110985 |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
|  | Company A |
|  | Company B |
|  | Company A |
|  | Company B |

## Summary for 1st round

## Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1-1: Validity of previously reported data | While details of test setup and data collection method was often not provided, it is agreed by most companies that the previously provided data is valid for the assumptions taken. Tentative agreements: Option 1: Previous data is valid Candidate options: Recommendations for 2nd round: No further discussion |
| Sub-topic #1-2: Analysis of previously reported data | There were no comments suggesting that the analysis in R4-2110985 is incorrect after clarification by Qualcomm. However, a few companies commented that the underlying datasets were incomplete to fully derive MPR. Tentative agreements: The analysis in R4-2110985 is correct for the data available Candidate options: Recommendations for 2nd round: Discuss what data is missing to fully derive MPR and explain how MPR was previously agreed if this data was missing or point to a reference where that data was made available or how it was accommodated. |
| Sub-topic #1-3: Proposal for smartphone MPR | In support of the proposal: Verizon, T-Mobile USA, CMCC, Qualcomm Not supporting: LGE, Huawei, Apple, Skyworks The arguments of those not supporting are that the data analyzed is not comprehensive enough. Tentative agreements: Candidate options: Recommendations for 2nd round: Try to address the concerns - The waveforms do not cover all 2Tx cases. - Larger channel bandwidths are not considered. ET PA is not considered. - Different PA types and design technologies not adequately represented, process variation not accounted for. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

| Company | Comments |
| --- | --- |

# Topic #2: MPR for FWA

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2109441 | Apple | Title: Considerations on n77 and n78 Proposal 2: The baseline architecture assumption for FWA devices should fundamentally differ from typical handsets (i.e. antenna isolation and antenna gain, PCB isolation, MPE handling etc.) to allow decent improvements in UL and DL. Proposal 3: Introduce a new device type signalling for FWA devices. This signalling shall inform the network that it is connected to an FWA device type with a different set of capabilities compared to typical handsets. |
| R4-2111009 | Skyworks Solutions, Inc. | Title: Evaluation of Reverse IMD versus antenna isolation and its impact to MPR Proposal on TxDiv and single stream UL MIMO evaluation: • The emissions should be evaluated as the mathematical (RMS) sum of the two PA coupled spectrums and not with a physical (RF combiner) sum. • Even so the PA coupled spectrum is subject to nulls due to phase combinations or delay which can impair measurement by up to 3dB: o Phase and/or delay may need to be adjusted to avoid such nulling/summing effects especially for narrow allocations o PA output power needs to be summed mathematically (RMS sum) o Ideally for UL MIMO all 4 states should be evaluated o Ideally for CDD based TxDiv companies should agree on which delay to use o Reasonable margin should be allowed in the MPR as these issues will not reveal at conformance (the antenna coupling disappears with connected measurements) but will exist in the field. o When submitting MPR values companies should provide an assessment of the issues above in their measurement or simulation framework. Proposal on MPR evaluation: • MPR assessment must account for RIMD and its different behavior for different PA architectures • MPR improvements may not be all related to RIMD and antenna isolations thus variability in WOLA design and TRX impairment must be accounted for |
| R4-2111297 | Huawei,HiSilicon | Title: Discussion on MPR requirements for PC1.5 FWA Proposal 1: Relatively high antenna isolation/PCB isolation values may be used for evaluation, but they are not the baseline for product implementation. Proposal 2: When determining the PC1.5 MPR for FWA, RAN4 needs to decide how to use the evaluation results based on both optimistic and conservative RF assumptions, which are derived for different bands and form factors. Proposal 3: If RAN4 decides that only one set of MPR requirements is to be defined for PC1.5 FWA regardless of frequency bands or form factors, it should be based on the conservative RF assumptions. |
| R4-2110832 | OPPO | Title: R17 PC1.5 FWA Proposal 1: It is proposed to not specify FR1 FWA UE type in the specification, instead use more generic conditions like antenna isolation and PCB isolation as the applicability of large form factor FWA requirements. Any UE type (e.g. FWA/CPE/MIFI) that meet these conditions can apply the requirements. Proposal 2: It is proposed to consider 10dB antenna isolation for the small form factor FWA, and for low band with more than 2 antennas FWA. |
| R4-2108974 | CMCC | Title: Discussion on the UE RF requirements of PC1.5 n79 Proposal 1: We propose to re-evaluate PC1.5 MPR in inner, outer and edge case, and the MPR re-evaluate need to consider Smartphone and FWA UEs. |

## Open issues summary

The papers for this topic wanted to discuss assumptions for FWA devices, in particular, those FWA devices that are small in size.  However, a way forward was already agreed at the last meeting in R4-2105492 with the following assumptions for FWA

  - 15/20 dB antenna isolation

  - PCB isolation 70dB or higher

  - Post-PA front-end loss assumed to be 4 dB per Tx Chain

Therefore, the moderator does not believe it is productive to rediscuss these aspects at this time especially with the recognition that the evaluation is focused on high bands assuming FWA has a large form factor.  Also as a part of this way forward and as proposed in papers presented to this meeting that a capability signaling could be considered.  While the exact form of this capability signaling needs further discussion, it is certainly possible that “small FWA” could have a different capability from the baseline assumption, perhaps the same capability as a smartphone as suggested in R4-2109441.

No specific data and quantitative proposals were provided for MPR evaluation at this meeting.

### Sub-topic 2-1

Skyworks provides a paper detailing assumptions and methods for data collection to determine dual chain MPR.  On the other hand, dual chain MPR has already been evaluated before.  Is there anything that needs to be changed based on updated learning that should be applied for this effort?

Open issues and candidate options before e-meeting:

Issue 2-1: Method of data collection

- Proposals

  - Option 1: The method of data collection as companies have followed before is still appropriate.

  - Option 2: The method of data collection should be enhanced.  Please be specific.

- Recommended WF

  - TBA

### Sub-topic 2-2

According to the agreed WF from last meeting as well as contributions in this meeting, the notion of signaling a device type or capability should be discussed.  What should be signaled?  How would the network use this information?  How much signaling is too much, either from signaling overhead, specification complexity, UE fragmentation, or ability of network to use the information?

Open issues and candidate options before e-meeting:

Issue 2-2: Capability signaling

- Proposals

  - Option 1: Signal the device type, i.e., Type A, Type B, Type C.  A set of performance requirements would be associated with each device type.

  - Option 2: Signal the performance items separately, i.e., MPR1, MPR2, MPR3, Refsens1, Refsens2, A-MPR1

  - Option 3: Prefer not to have any signaling.  Prefer not to have different requirements for FWA.

  - Option 4: Other ideas, or still needs more study.  Please offer ideas for future discussion.

- Recommended WF

  - TBA

## Companies views’ collection for 1st round

### Open issues

One of the two formats, i.e. either example 1 or 2 can be used by moderators.

Example 1

| Company | Comments |
| --- | --- |
| Qualcomm | Sub topic 2-1: Option 1, we tend to think that the method of data collection followed by companies is appropriate although we would certainly appreciate if more details were shared. Sub topic 2-2: Option 4. It may be premature to agree on the exact signaling format or approach at this meeting. To make progress, however, we suggest to list the requirements that are expected to differ between signaling alternatives and how they might be used by the network. Once we have identified those, then it may be easier to visualize the best approach for signaling. …. Others: |
| Nokia | Sub topic 2-2: Option 4. We basically have a similar view with Qualcomm. But at least we don’t believe that option 2 is a good idea. There would be some correlation between MPR and refsense to some extent. A UE with big formfactors would have better isolations with better RF components so that both MPR and refsense would be better. Also from UE design perspective, it does not make sense to develop UEs with better UL but with poorer DL performance or vice versa. |
| CMCC | ### Sub-topic 2-2：option 4 |
| Samsung | Sub-topic 2-2: So far, we prefer Option 4 given that nothing is identified and agreed for the device type having a large form factor. We can think about Option 1 when all of the related requirements or plan for FWA (or other device type than smartphone) are ready. |
| OPPO | Issue 2-2: Capability signaling Option 1, similar as the handling of FR2 UE types. |
| Skyworks | Issue 2-1: since there is no data presented for FWA I just encourage every company to clarify their setup, type of waveform and 2Tx techniques evaluated so that we have a clean and comparable set of data. Also measured/simulated evaluation should be clarified as how the coupling is reproduced in the setup or modelled in the simulation can completely change the interaction between the two PAs. Issue 2-2: Since we already have a number of cases where combinations or performance or even antenna architecture is linked to larger form factors than a smartphone. Having a signalling related to form factor in FR1 would allow to properly map those cases. (latest is a 3 LB antenna architecture proposal for LB-LB-LB combination which is also only feasible with larger from factor) |
| Apple | Sub-topic 2-2: Option1. Defining clear device type signaling for FWA allows to create specific sets with architecture assumptions including antenna & PCB isolation, antenna gain and the associated requirements (e.g. MPR, MPE). Those sets should be easier to handle for the network compared to a large buffet of individual capabilities (for MPR, REFSENS, MPE) from which the UE can pick from, especially as this approach would lead to a large amount of different possible combinations. |
| Huawei | Issue 2-1: The proposals from Skyworks are good for the future work of the group. This is not to say that other measurements are wrong, but it’d be good to double check and ensure the problems identified by Skyworks are avoided. As we have pointed out, it’s crucial to align the PA calibration point at the beginning of any measurements. Issue 2-2: It sounds premature to discuss signalling design before we’re even clear about how to define the MPR requirements for PC1.5. For example, will there be separate MPRs for smartphones and FWAs? Will the MPR be dependent of frequency bands and/or form factors? The signalling design is the means while the requirements are the end. Discussing the means before the end is clear, or mixing the two together might not be very effective or efficient. It’d be better for the group to take a holistic approach on defining the PC1.5 MPR requirements, not just for n77/78 but also in the context that it may apply to more and more bands as well as different form factors. |
| LGE | Sub topic 2-1: Option 1 Sub-topic 2-2: Option 1. As exceptionally, RAN4 can allow multiple capability signaling. Without capability signaling for FWA device, RAN4 is quite difficult to define just one set MPR requirements since the antenna isolation and PCB isolation is quite different with the device size and frequency. |
| AT&T | Issue 2-2: Option 4. We support the comments raised by Qualcomm and Nokia on this topic. |
| Vivo | Option 4 |
| T-Mobile USA | Issue 2-1: Option 1 Issue 2-2:Option 4 |
| Verizon | Issue 2-1: Option 1 Issue 2-2:Option 4 |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
|  | Company A |
|  | Company B |
|  | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #2-1 Method of data collection | Tentative agreements: Option 1: The method of data collection as companies have followed before is still appropriate - Companies are encouraged to provide more details on test setups and method of data collection and to be mindful of the pitfalls when 2Tx streams interact with one another Candidate options: Recommendations for 2nd round: No further discussion |
| Sub-topic #2-2 Capability signaling | Tentative agreements: Outcome of GTW on May 21 Tentative agreement: use Option 1 as starting point and also take MPR evaluation results into account for further discussion whether the capability signaling is needed. Tentative agreement: Further discuss defining different device types signaling - Understand what different requirements are needed, take the MPR evaluation into account - Compare whether the MPR requirements will be different before discussing the signaling - Further discuss whether the different device type is needed considering the following options - Option 1 : Signal the device type, i.e., Type A, Type B, Type C. A set of performance requirements would be associated with each device type. - Option 3: Prefer not to have any signalling. Prefer not to have different requirements for FWA. - Discussion are limited to PC1.5 Candidate options: Recommendations for 2nd round: Further discussion to try to narrow down and refine the tentative agreements above |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

| Company | Comments |
| --- | --- |

# Topic #3: SAR and MPE

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2109843 | Samsung | Title: MPE handling for high power FWA UE in FR1 Table 4: A brief outline of duty cycle solutions Parameter; maxUplinkDutyCycle-PC2-FR1; maxUplinkDutyCycle-FR2; [maxUplinkDutyCycle-FWA-FR1]. Applicability; FR1 HPUE (Smartphone); FR2 (All PCs); FR1 HPUE (FWA). Criterion; SAR (W/kg); MPE (W/m2); MPE (W/m2). Element; 60%, 70%, 80%, 90%, 100%; 15%, 20%, 25%, 30%, 40%, 50%, 60%, 70%, 80%, 90%, 100%; [20%, …, 100%]. > Max UL (%); PC fall-back; P-MPR; P-MPR. Default; 50% (PC2), 25% (PC1.5); None (scaling down); None (declaring distance). Proposal: A new signalling can be introduced for the high power FWA UE in FR1 to mitigate the impact of the MPE requirement as outlined in Table 4 above. |

## Open issues summary

Samsung presents a paper discussion options for handling of MPE for FWA in FR1.  The previous idea of Gtx based specification is found to be challenging due to the uncertainty and variation in Gtx as well as the possible declared value of distance, R, by the manufacturer for compliance.  The paper then suggests that some kind of duty cycle approach either similar to the FR1 or to the FR2 approach is considered for FR1 FWA.

CMCC also provides CR’s to correct the existing SAR specification for PC1.5 smartphone.

### Sub-topic 3-1

Open issues and candidate options before e-meeting:

Issue 3-1: FWA MPE

- Proposals

  - Option 1: Adopt the FR1 maxUplinkDutyCycle-PC2-FR1

  - Option 2: Adopt the FR2 maxUplinkDutyCycle-FR2

  - Option 3: Adopt the hybrid maxUplinkDutyCycle-FWA-FR1

- Recommended WF

  - TBA

## Companies views’ collection for 1st round

### Open issues

Example 1

| Company | Comments |
| --- | --- |
| Qualcomm | Sub topic 3-1: Option 1. To the extent that the same approach can be used for smartphone and FWA, it makes the specifications and the network management simpler. Sub topic 1-2: …. Others: |
| CMCC | - Option 1: Adopt the FR1 maxUplinkDutyCycle-PC2-FR1 |
| Samsung | We prefer option 3 as mentioned in our paper. But, we are also fine to have further discussion whether to introduce new signaling based on the pros and cons of each option such as simplification vs. UL restriction. All the proposed options are able to handle the MPE regulation according to our further investigation. |
| OPPO | In principle duty cycle can be used as the solution, but the values could be different for this specific FWA UE type. So the values need to be evaluated whether the existing capability can be reused, if not then new capability is needed, i.e. Option 3. |
| Skyworks | It should be feasible to reuse the SAR dutyCycle framework for MPE, then FWA would have to signal higher values than default. |
| Huawei | The existing FR1 duty cycle scheme should still work with some potential adaptation such as longer evaluation period for MPE compliance. We’re also open to further discuss other options if there’re clear benefits. |
| LGE | Option 1 or option 3 can be applied for FWA UE. If RAN4 agree to make new signling for FWA UE, then option 3 is feasible. |
| Vivo | Sub topic 3-1: Option 1. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2108940 CR on PC1.5 HPUE SAR issue into Rel-16 TS 38.101-1 (CMCC) | Qualcomm: There is one missed correction of maxUplinkDutyCycle/2 to be replaced by 0.5*maxUplinkDutyCycle-PC2-FR1. |
|  | CMCC：A new tdoc number is needed to update the CR revision number on coversheet and update the CR category to F. |
| R4-2108941 CR on PC1.5 HPUE SAR issue into Rel-17 TS 38.101-1 (CMCC) | Qualcomm: Why is this a R17 Cat F CR? Should this be a Cat A? |
|  | CMCC：A new tdoc number is needed to update the CR revision number on coversheet and update the CR category to A. |
|  | Company A |
|  | Company B |
|  | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#3-1: FWA MPE | Tentative agreements: Outcome of GTW on May 21 Down-select to Option 1 and Option 3 - Option 1: Adopt the FR1 maxUplinkDutyCycle-PC2-FR1 (SAR-based duty cycle) - Option 3: Adopt the hybrid maxUplinkDutyCycle-FWA-FR1 (new signaling) Candidate options: Recommendations for 2nd round: Further discussion on whether the maxUplinkDutyCycle-PC2-FR1 IE is sufficient, or whether new signaling IE is needed. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2108940 CR on PC1.5 HPUE SAR issue into Rel-16 TS 38.101-1 (CMCC) | To be revised |
| R4-2108941 CR on PC1.5 HPUE SAR issue into Rel-17 TS 38.101-1 (CMCC) | To be revised |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

| Company | Comments |
| --- | --- |

# Topic #4: UE RF for Band n79

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2108974 | CMCC | Title: Discussion on the UE RF requirements of PC1.5 n79 Proposal 2: No A-MPR issue for PC1.5 n79, and No changes to section 6.2.3 (A-MPR) in 38.101-1 for PC1.5 n79 are needed. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 4-1

Open issues and candidate options before e-meeting:

Issue 4-1: A-MPR for PC1.5 n79

- Proposals

  - Option 1: No coexistence issues requiring A-MPR for PC1.5 Band n79.  No changes to 6.2.3 needed..

  - Option 2: Some change is needed or still needs to be studied.  Please be specific.

- Recommended WF

  - TBA

## Companies views’ collection for 1st round

### Open issues

Example 1

| Company | Comments |
| --- | --- |
| Qualcomm | Sub topic 4-1: Option 1. Sub topic 1-2: …. Others: |
| CMCC | Option 1: No coexistence issues requiring A-MPR for PC1.5 Band n79. No changes to 6.2.3 needed. |
| Skyworks | Option 1, we do not see specific emission requirement/issue for PC1.5 and single CC operation. |
| Apple | Option1, we did not observe any coexistence issues for this band and power class. |
| Huawei | Option 1 |
| LGE | Option 1 |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2108942 CR on PC1.5 UE RF requirements of n79 in Rel-17 TS 38.101-1 (CMCC) | Company A |
|  | Company B |
|  | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#4-1 | Tentative agreements: No coexistence issues requiring A-MPR for PC1.5 Band n79. No changes to 6.2.3 needed. Candidate options: Recommendations for 2nd round: Discuss whether the CR should be agreed for only the Band n79 changes or whether the CR can be technically endorsed, but wait for formal submission until all the aspects of PC1.5 are completed (MPR, UE type signaling, MPE signaling) and submit complete CR package at that time |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2108942 CR on PC1.5 UE RF requirements of n79 in Rel-17 TS 38.101-1 (CMCC) | Technically endorse? |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

| Company | Comments |
| --- | --- |

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on MPR for PC1.5 smartphones and FWA | [Qualcomm Incorporated] |  |
| WF on device type signaling for PC1.5 | Apple |  |
| WF on duty cycle signaling for RF exposure mitigation for PC1.5 FWA | Samsung |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2108940 | CR on PC1.5 HPUE SAR issue into Rel-16 TS 38.101-1 | CMCC | Revised |  |
| R4-2108941 | CR on PC1.5 HPUE SAR issue into Rel-17 TS 38.101-1 | CMCC | Revised |  |
| R4-2108942 | CR on PC1.5 UE RF requirements of n79 in Rel-17 TS 38.101-1 | CMCC | TBD (technically endorse?) | Should we agree the band-specific CR now? Or wait for PC1.5 general requirements to be completed and submit all at once? |
| R4-2108974 | Discussion on the UE RF requirements of PC1.5 n79 | CMCC | Noted |  |
| R4-2109441 | Considerations on n77 and n78 | Apple | Noted |  |
| R4-2109843 | MPE handling for high power FWA UE in FR1 | Samsung | Noted |  |
| R4-2110832 | R17 PC1.5 FWA | OPPO | Noted |  |
| R4-2110985 | A reconsideration of PC1.5 MPR for smartphones | Qualcomm Incorporated | Noted |  |
| R4-2111009 | Evaluation of Reverse IMD versus antenna isolation and its impact to MPR | Skyworks Solutions Inc. | Noted |  |
| R4-2111297 | Discussion on MPR requirements for PC1.5 FWA | Huawei,HiSilicon | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents