3GPP TSG-RAN WG4 Meeting # 99-e	R4-2107732

Electronic Meeting, May. 19-27, 2021

Title:	TP to TR 38.717-04-02: Addition of CA_n25-n66-n71-n77

Source:	Nokia, T-Mobile USA

Agenda item:	8.14.2

Document for:	Approval

# 1	Introduction

This is a TP to TR 38.717-04-01 to add CA_n25A-n66A-n71A-n77A with 2UL

************************************* Start of TP*****************************************

## 5.2.x	CA_n25-n66-n71-n77

### 5.2. x.1	Operating bands for CA

Table 5.2.x-1: Inter-band CA operating bands of CA_n25-n66-n71-n77

| NR CA Band | NR Band (Table 5.2-1 in TS38.101-1[2]) |
| --- | --- |
| CA_n25-n66-n71-n77 | n25, n66, n71, n77 |

CA_n25A-n66A 2UL defined in 38.101-1 Table 5.5A.3.1-1

CA_n25A-n71A 2UL defined in 38.101-1 Table 5.5A.3.1-1

CA_n25A-n77A 2UL defined in 38.101-1 Table 5.5A.3.1-1

CA_n66A-n71A 2UL defined in 38.101-1 Table 5.5A.3.1-1

CA_n66A-n77A 2UL defined in 38.101-1 Table 5.5A.3.1-1

CA_n71A-n77A 2UL defined in 38.101-1 Table 5.5A.3.1-1

#### 5.2.x.2	Channel bandwidths per operating band for CA

Table 5.2.x.2-1: Supported bandwidths per CA_n25-n41-n71-n77

| NR CA Configuration | UL Config | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n25A-n66A-n71A-n77A | CA_n25A-n66A CA_n25A-n71A CA_n25A-n77A CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

#### 5.2.x.3	UE Co-existence study

The coexistence studies have been captured into the constituent fallback modes in TR 38.717-03-02 and in TS 38.101-1, there is no additional harmonic and intermodulation impact for the additional band receiver.

#### 5.2.x.5	REFSENS requirements

There are no additional MSD requirements for this band combination.

************************************* End of TP*****************************************