3GPP RAN WG4 Meeting #99-e		R4-2107999

Electronic meeting, May 19 – 27, 2021	(revision of R4-2107893)

Agenda item:	10.4.4

Source:	Apple

Title:	TP to TR38.860 on band plan and duplex filter considerations for 600 MHz

WI/SI:	FS_NR_600MHz_ext

Release:	Rel-17

Document for:	Approval

# 1	Introduction

During the RAN #90e meeting a new study item was approved [1] to study the the possibilities to use the 600MHz frequency range according to the LS from APT [2]:

The purpose of this study item is to: Study a harmonised frequency variant approach within the frequency range of 612-652/663-703 MHz. The liaison statement from AWG to RAN4 has given two options B1 and B2 respectively. For each option it will be desirable to study the technical feasibility of the duplex filters needed, centre band gap, insertion loss. For option B2 the duplex distance is 46 MHz as is the case with NR band n71. The bottom duplexer is the same as that of n71, with an additional upper duplexer that should have as large possible overlap as possible with the lower duplexer in n71 but at the same time being able to handle the duplex gap of 6 MHz. The size of this upper duplexer needs to be studied. The co-existence requirement with adjacent broadcast service below 617 MHz can be fulfilled with the same condition as in band n 71. It is assumed that there are no services in 657- 663 MHz. For option B1 the duplex distance is 51 MHz , which may be considered in case of an additional broadcasting channel can be vacated such that the guard band to the adjacent broadcast service is still maintained similarly to band n 71. In addition, the protection of radio astronomy is required in certain countries in Region 3 ( WRC 15). Both options B1 and B2 addressed here are just starting point for the feasibility study to enable the utilization on extended 600MHz band.. The AWG work plan forwarded to the 3GPP shows this work to be completed by September 2021. Specifically, this study item includes the following objectives: - Regulatory study of the frequency range around 600MHz in Region 3 - Co-existence study for the frequency range of 612-652/663-703 MHz such as with DTV (if needed) - Study potential frequency arrangements and conclude the possible implications (such as insertion loss, transmitter and receiver characteristics for both BS and UE, system limitations such as channel bandwidths, etc.) of different duplex filter implementations. - Consider options B1 and B2 from AWG LS, but other options are not precluded. - Answer the request from AWG regarding the technical feasibility of option B1 and B2, respectively. Further options are not precluded and may be included in LS to AWG. NOTE: Since regulatory study of frequency range around 600MHz is for Region 3, the SI outcome will not impact any requirements defined for US 600MHz band.

An internal TR will be used to capture the study’s outcomes.  This contribution provides the duplex filter implementation issues related to the band plan B2a proposed in [3] as a text proposal to the new TR [4].

# 2	Discussion

This contribution provides the duplex filter implementation issues related to the band plan B2a proposed in [3] as a text proposal to the new TR [4].

Proposal 1:	It is proposed to approve the text proposal provided in this contribution.

# 3	References

- RP-202924, “New SID on extended 600 MHz NR band,” Spark NZ, CBN, 3GPP RAN #90e, December 2020

- RP-202143, “LS on Frequency arrangements for IMT in the band 470-703MHz”, Asia-Pacific Telecommunity, 3GPP RAN #90e, December 2020

- R4-2102589, “Band Plan for 600MHz SI,” Apple, 3GPP RAN4 #98e, February 2021

- R4-2103265, “Blank TR for extended 600MHz NR band,” Spark NZ Ltd, 3GPP RAN4 #98e, February 2021

# Text Proposal

<< start of change >>

## 6.4	Duplex filter implementation issues

#### 6.4.3	Option B2a

#### 6.4.3.1	General

A device that supports the extension of the 600MHz band may choose to also support the existing 600MHz band, n71, with the same or different hardware.  [Specifying a new dual duplexer solution re-using band 71/n71, which is already specified allowing at least a dual duplexer architecture, may be an issue, as a dual duplexer band is not very flexible when spectrum allocations change.] A new dual duplexer band or any other band that extends beyond the current 600 MHz band definitions would require new UE designs. To enable existing UE’s already on the market, the APT band plan would need to include the existing band 71/n71 band definition and its requirements.  [Any deisred extension such as the additional 2x5 MHz spectrum proposed in the SI to this would require new designs.]

One option illustrated below is to use band 71/n71 as is, covering the frequency range 663-698MHz for UL and 617-652MHz for DL in the APT region, and specify a new band covering at least the additional 2x 5MHz spectrum proposed in the SI, but it could also support up to 2x 30MHz to maximize the overlap with band 71/n71 to enable CA/DC within the new band. The new band plan could look like shown in Figure 4.3-1 below.

Figure 6.4.3.1-1: Band plan proposal for Option B2a

To maximize the overlap to enable CA/DC, option 5 above can specify one new single duplexer FDD band nyyy additionally to band n71, which is already specified. The operating band table can be as shown in Table 4.3-1 below:

Table 6.4.3.1-1: Operating band table for Option B2a

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n71 | 663 MHz – 698 MHz | 617 MHz – 652 MHz | FDD |
| nyyy | 673 MHz – 703 MHz | 627 MHz – 657 MHz | FDD |
| NOTE x: UE that complies with the NR Band nyyy minimum requirements in this specification shall also comply with the NR Band n71 minimum requirements. |  |  |  |

#### 6.4.3.2	Option B2a with Single Duplexer

Band 71 together with Option B2a span 663-703 MHz for TX and 617-657 MHz for RX. Since the duplex gap is just 6MHz, there are challenges to implement it with a single duplexer using current filter technology.

#### 6.4.3.3	Option B2a with Dual Duplexer

Option B2a can be implemented with a dual duplexer configuration. One duplexer could cover the frequency range of n71 with 35MHz and 663-698 MHz for TX and 617-652 MHz for RX and the second the additional APT frequencies with a bandwidth of 35MHz or less, for example for a 30MHz duplexer 673-703 MHz for TX and 627-657 MHz for RX.  Option B2a has no impact on the filter performance of the Band 71 duplexer.

Due to the small duplex gap there will be an issue to protect the upper part of the RX band with -50dBm/MHz from the own TX, in case this is specified as a single band with Option B2. If the band is split into n71 and the additional frequency range as Option B2a, this issue still exists since the n71 Tx filter is not able to provide rejection to the upper 652 – 657 MHz portion of the downlink in the new band with only 6 MHz separation without, e.g., a relaxation of the protection requirement.

<< end of change >>