3GPP TSG-RAN WG4 Meeting # 99-e	R4-2108164

# Electronic Meeting, May. 19-27, 2021

Agenda item:			11.9.4

Source:	Moderator (Huawei)

Title:	Email discussion summary for [99-e][240] NB_IOTenh4_LTE_eMTC6_RRM

Document for:	Information

Introduction

Companies are encouraged to identify the impact on RRM requirements of Rel-17 NB_IOTenh4_LTE_eMTC6 and have initial discussion on identified issues.

1st round:

Companies provide comments on the scope of RRM impact of Rel-17 NB_IOTenh4_LTE_eMTC6 and have initial discussion on identified issues.

2nd round:

Keep discussing the scope of RRM impact of Rel-17 NB_IOTenh4_LTE_eMTC6 and identified issues if needed.

Topic #1: RRM impactd of NB_IOTenh4_LTE_eMTC6

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2110275 | Nokia, Nokia Shanghai Bell | Observation 1: The support of 16 QAM unicast in DL will require extensions to channel quality reporting specified in TS 36.133 for NB-IoT presumably using existing subclauses. This work needs to be coordinated with RAN1 and RAN2. Observation 2: The support of neighbour cell measurements prior to RLF for NB-IoT will require measurement support specified in TS 36.133 presumably using a separate clause. This work needs to be coordinated with RAN2. Observation 3: Support of power reduction for PRACH, PUCCH, and full-PRB PUSCH for LTE-MTC will not impact RRM core requirements. Proposal 1: RAN4 scope of the RRM work for Additional enhancements for NB-IoT and LTE-MTC is on defining channel quality reporting for 16-QAM unicast in DL based on RAN1 and RAN2 agreements and measurement requirements for support of neighbour cell measurements in RRC Connected state prior to RLF based on RAN2 agreements. Therefore, work should proceed in tight coordination with RAN1 and RAN2. |
| R4-2110346 | Huawei, HiSilicon | Proposal 1: For Rel-17 additional enhancements for NB-IoT and LTE-MTC WI, discuss the following RRM requirements: - Neighbour cell measurement in CONNECTED state of NB-IoT before RLF Observation 1: Up to RAN2 to decide the triggering and the stopping mechanism. If any issue is identified in RAN2, RAN2 will further check with RAN4 if necessary. Observation 2: From the perspective of enabling the neighbour cell measurement before RLF, UE should start the measurement earlier than OOS for the following reasons: - OOS indication implies that the channel condition is already extremely low for a certain period of time. - Large number of repetitions are needed for transmission and reception which leaves few opportunities for measurement on other frequency layers. - Time could be used for neighbour cell measurement before RLF is insufficient. Observation 3: The minimum length of a measurement occasion and maximum interval shall be considered when defining the RRM requirements for neighbour cell measurement on carrier frequency which is different from the serving cell. Proposal 2: When the target frequency carrier is different from the serving carrier, the maximum interval between two occasions shall be less than 5 seconds and the minimum length of as measurement occasion shall be at least 400 ms for normal coverage. Observation 4: Neighbour cell measurement in connected state is not “routine measurement”, which could only be triggered when the channel condition is deteriorating and the RLF and RRC re-establishment is going to happen, and UE shall utilize the limited the measurement occasions wisely to find a suitable cell before RLF. Observation 5: The neighbour cell measurement in enhanced coverage shall be deprioritized for the following reasons: - Time needed for detection/measurement of a cell in enhanced coverage is extremely longer than what is needed for a cell in normal coverage. - UE may fails to find a suitable cell as the measurement occasions are limited before RLF and the cell detection of a cell in enhanced coverage will occupy almost all occasions. - UE needs to start neighbour cell measurement long before RLF when the channel condition may haven’t change at all. Proposal 3: The neighbour cell measurement of a cell in enhanced coverage shall be deprioritized from RAN4’s perspective. Proposal 4: For neighbour cell measurement in connected state, UE shall be able to monitoring at least the carrier which is same as the serving carrier and at least two carriers which are different from the serving carrier. Then detection/measurement delay shall be scaled by the number of carriers. |
| R4-2111250 | Ericsson | Proposal #1: RAN4 shall wait for RAN2 conclusions on the exact conditions for starting and stopping CONNECTED mode neighbour cell measurements. Proposal #2: Release 17 intra-frequency and inter-frequency neighbor cell measurement requirements are introduced in section 8.14 of TS 36.133. Proposal #3: Consider the example in Figure 1 for defining the release 17 neighbor cell measurement requirements. Observation 1: No impact on NRSRP/NRSRQ measurement accuracy sections. |

Open issues summary

Sub-topic 1-1  RRM Work scope

Issue 1-1-1: RRM requirements for neighbour cell measurements in RRC Connected state before RLF for NB-IoT

- Proposals

  - Option 1: Define RRM requirements for neighbour cell measurements in RRC Connected state before RLF (Nokia P1, Huawei P1, Ericsson)

- Recommended WF

  - Can we agree on option 1?

Issue 1-1-2: RRM requirements for channel quality reporting for 16-QAM for NB-IoT

- Proposals

  - Option 1: Define channel quality reporting for 16-QAM unicast in DL based on RAN1 and RAN2 agreements. (Nokia P1)

- Recommended WF

  - Moderator: According to the approved work plan R4- 2107255, the channel quality reporting to support 16-QAM will be discussed in Demod  session starting from RAN4#101.

Companies views’ collection for 1st round for Sub-topic 1-1

| Company | Comments |
| --- | --- |
| XXX | Issue 1-1-1 Issue 1-1-2 |
| Ericsson | Issue 1-1-1 Option 1 can be agreed. Issue 1-1-2 Fine with recommended WF from moderator. |
| Huawei | Issue 1-1-1 Support option 1. Issue 1-1-1 Support the recommended WF. |
| Qualcomm | Issue 1-1-1 Support option 1 Issue 1-1-2 Agree with the recommended WF |

Sub-topic 1-2 	Issues related to neighbour cell measurements in RRC Connected state before RLF for NB-IoT

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 1-2-1: Ways to organize the specification to capture the RRM requirements for neighbour cell measurements

- Proposals

  - Option 1: Capture the neighbour cell measurements requirements in a separate clause (Nokia O2)

  - Option 2: Capture the neighbour cell measurement requirements in clause 8.14 which will include measurements requirements for intra-frequency measurements on serving cell (legacy) and intra/inter frequency neighbour cell measurements (Ericsson P2/P3)

- Recommended WF

  - Need more discussion

Issue 1-2-2: Starting and stopping triggering of CONNECTED mode neighbour cell measurement

- Proposals

  - Option 1: RAN4 shall wait for RAN2 conclusions on the exact conditions for starting and stopping CONNECTED mode neighbour cell measurements. (Ericsson P1)

- Recommended WF

  - Can we agree on option 1?

Issue 1-2-3: Conditions for CONNECTED mode neighbour cell measurement when the target frequency carrier is different from the serving carrier

- Proposals

  - Option 1: When the target frequency carrier is different from the serving carrier, the maximum interval between two occasions shall be less than 5 seconds and the minimum length of measurement occasion shall be at least 400 ms for normal coverage. (Huawei P2)

- Recommended WF

  - Need more discussion

Issue 1-2-4: Neighbour cell measurement of a cell in enhanced coverage

- Proposals

  - Option 1: The neighbour cell measurement of a cell in enhanced coverage shall be deprioritized from RAN4’s perspective. (Huawei P3)

- Recommended WF

  - Need more discussion

Issue 1-2-5: Multiple carriers operation

- Proposals

  - Option 1: UE shall be able to monitoring at least the carrier which is same as the serving carrier and at least two carriers which are different from the serving carrier. Then detection/measurement delay shall be scaled by the number of carriers. (Huawei P4)

- Recommended WF

  - Need more discussion

Issue 1-2-6: Measurement accuracy

- Proposals

  - Option 1: No impact on NRSRP/NRSRQ measurement accuracy sections. (Ericsson O1)

- Recommended WF

  - Need more discussion

Companies views’ collection for 1st round for Sub-topic 1-2

| Company | Comments |
| --- | --- |
| XXX | Issue 1-2-1 Issue 1-2-2 Issue 1-2-3 Issue 1-2-4 Issue 1-2-5 Issue 1-2-6 |
| Ericsson | Issue 1-2-1 Option 1 and 2 are related and both can be agreed. Issue 1-2-2 Option 1 is agreeable. Issue 1-2-3 It may depend on how the measurements are carried out. For example, the inter-frequency carriers can be masued during any of: - Vacant slots not scheduled for data transmission, i.e. when not required to do data transmission/reception - When not required to do NPDCCH monitoring - during the DRX inactive period i.e. when the UE is configured with DRX. Perhaps the measurement interval and the measurement interval may be affected depending on when the measurements are carried out. For the measurement period, we prefer to reuse the current delay requirements. Since this is the first meeting, companies can do further analysis on this issue. Issue 1-2-4 In our understanding the neighbour cell measurements are supported in both normal and enhanced coverage in RAN2. RAN4 should therefore develop the requirements for both scenarios. Issue 1-2-5 We would like to understand the motivation for at least two carriers and also hear more views on the number of carriers, especially from the operators. Issue 1-2-6 We agree that the current accuracy requirements can be reused. |
| Huawei | Issue 1-2-1 No strong views, but it is too early to discuss the spec structure. Prefer to focus the work scope and the technical issues. Issue 1-2-2 Option 1 is agreeable. Issue 1-2-3 Support option 1 but also fine to let companies to have further check. Some further explanation: The detection time agreed in the last meeting which is also captured in the LS to RAN2 is inherited from RRC re-establishment (e.g. 1400 for NC). In RRC re-establishment, UE could use the whole 1400ms for cell detection. But for the neighbour cell measurement before RLF, 1400ms subjects to the actual measurement occasions. For example, if UE is fully scheduled within 1400 ms, UE cannot do the measurement at all. The option 1 is proposed as the side conditions for the 1400ms and 14800 ms. Issue 1-2-4 From RAN4’s perspective, we will not define requirements for some corner or uncommon cases even the procedure is supported by RAN1/RAN2 just as usual. What’s special is that the neighbour cell measurement before RLF is not some “routine measurement” but “emergency measurement”. The measurement will only be triggered when the channel is getting band and the RLF is about to happen. So the available time for searching a cell before RLF is limited. As agreed in the last meeting, the cell detection time for a cell in NC is 10 time more than that in EC. We don't think it is a wise choice for UE to waste some much time to find a cell in bad condition. More important, it means the measurement should be started very long before RLF, when the channel condition may haven’t changed yet. Issue 1-2-5 The two carriers is reused from the measurement capability in IDLE mode cell selection which we believe is technically reasonable. We are also fine to hear more opinions from companies. Issue 1-2-6 We think it is a little bit early to discuss the accuracy requirements. Prefer to discuss it in the perf part. |
| Qualcomm | Issue 1-2-1 New requirements for intra-frequency neighbor cell measurements could be introduced in separate subclauses under 8.14.2/8.14.3 or 8.14.2/8.14.3 could be renamed to indicate measurement requirements only for serving cell and parallel clauses could be added for the new neighbor cell measurement requirements. For inter-frequency neighbor cell measurements, parallel clauses (8.14.x) could be added for the new requirements. Issue 1-2-2 Support option 1. Issue 1-2-3 There would be some advantages to measuring neighbor cells at least once every 5 sec. We will consider the proposal and analyze further. Issue 1-2-4 We think it would be better to have more information about the RAN2 design before reaching this conclusion. Issue 1-2-5 The measured carriers should include the serving carrier. The number of inter-frequency layers to be monitored is TBD. Issue 1-2-6 We agree that this would be the preferred outcome from a workload perspective but it is still early to reach this conclusion. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1-1 | Issue 1-1-1: RRM requirements for neighbour cell measurements in RRC Connected state before RLF for NB-IoT Tentative agreements: Define RRM requirements for neighbour cell measurements in RRC Connected state before RLF Candidate options: NA Recommendations for 2nd round: NA Issue 1-1-2: RRM requirements for neighbour cell measurements in RRC Connected state before RLF for NB-IoT Tentative agreements: Channel quality reporting to support 16-QAM in NB-IoT will be discussed in Demod session. Candidate options: NA Recommendations for 2nd round: NA |
| Sub-topic #1-2 | Issue 1-2-1: Ways to organize the specification to capture the RRM requirements for neighbour cell measurements One company comments that both option 1 and option 2 are supported. One company comments it is too early to discuss the spec structure. One company comments that for inter-frequency neighbour cell measurement, parallel clauses could be added and provides two alternatives for intra-frequency neighbour cell measurements. Tentative agreements: NO Candidate options: The following options are summarized based on 1st round comments - Option 1: 8.14.2/3 include both legacy intra-frequency measurement and intra-frequency neighbour cell measurements. 8.14.x for inter-frequency neighbour cell measurements - Option 2: 8.14.2/3 are renamed for legacy intra-frequency measurement only and 8.14.x for intra-frequency neighbour cell measurement and 8.14.y for inter-frequency neighbour cell measurement Recommendations for 2nd round: Companies are encourage to discuss the above options in the 2nd round. Issue 1-2-2: Starting and stopping triggering of CONNECTED mode neighbour cell measurement Tentative agreements: RAN4 shall wait for RAN2 conclusions on the exact conditions for starting and stopping CONNECTED mode neighbour cell measurements Candidate options: NA Recommendations for 2nd round: NA Issue 1-2-3: Conditions for CONNECTED mode neighbour cell measurement when the target frequency carrier is different from the serving carrier One company supports option 1, and two companies comment that the issue needs further analysis. Tentative agreements: No Candidate options: same as 1st round - Option 1: When the target frequency carrier is different from the serving carrier, the maximum interval between two occasions shall be less than 5 seconds and the minimum length of measurement occasion shall be at least 400 ms for normal coverage. Recommendations for 2nd round: Continue discussion in the 2nd round. Issue 1-2-4: Neighbour cell measurement of a cell in enhanced coverage One company supports option 1, and one company supports to define requirements for both cases, and one company suggests to wait more information from RAN2. Tentative agreements: NO Candidate options: same as 1st round - The neighbour cell measurement of a cell in enhanced coverage shall be deprioritized from RAN4’s perspective. Recommendations for 2nd round: Continue discussion in the 2nd round. Issue 1-2-5: Multiple carriers operation One company supports option 1, and one company want to hear more views on the number, and one company comments that the serving carrier shall be included and the number of inter-frequency needs FFS. Tentative agreements: NO Candidate options: same as 1st round - Option 1: UE shall be able to monitoring at least the carrier which is same as the serving carrier and at least two carriers which are different from the serving carrier. Then detection/measurement delay shall be scaled by the number of carriers. Recommendations for 2nd round: Continue discussion in the 2nd round. Issue 1-2-6: Measurement accuracy One company supports option 1, and two companies comment that it is too early to have this discussion. Tentative agreements: NO Candidate options: same as 1st round - Option 1: No impact on NRSRP/NRSRQ measurement accuracy sections Recommendations for 2nd round: Companies can continue discussion in the 2nd round, but it is recommended to discuss this issue in future meetings. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on RRM requirements of Rel-17 Additional enhancements for NB-IoT and LTE-MTC | Huawei, HiSilicon |  |
| LS on … | ZZZ | To: RAN_X; Cc: RAN_Y |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents